PARALLEL WIRELESS · RCT — STUDY GUIDE

Module 10

Lab & Measurement Practice

Mentor's note: Where it all meets the bench: instruments, the FHGW platform bring-up, IQ playback, and the pitfalls that catch every new engineer.

# 1. The instruments

| Instrument | Role |
| --- | --- |
| Vector Signal Generator (VSG) | plays IQ waveforms into the radio (Rx tests) |
| Vector Signal Analyzer / Spectrum Analyzer | measures the transmitted signal (Tx tests) |
| Keysight EXM (E6640A) | combined VSG + VSA, software-toggled — your workhorse |
| Power meter | accurate absolute power reference |
| VNA | return loss / VSWR (Module 2) and cable-loss characterization |

# 2. The bench

Figure: PC drives the EXM over LAN; RF flows through a known-loss path to the DUT.

The PC runs RuAuto and drives the EXM by SCPI over LAN; RF flows through a characterized, matched path to the DUT. Tx tests measure DUT output; Rx tests play IQ in.

# 3. IQ capture and playback

Playback (Rx): load waveform into the VSG, set power/freq/sample-rate, set trigger (TDD: external trigger delay to align to the frame), enable ARB + modulation + RF. Capture (debug): the VSA records live IQ to inspect EVM/spectrum/timing.

# 4. The platform you actually bring up

| Element | Notes |
| --- | --- |
| DU | baseband unit driving the radio |
| FHGW | fronthaul gateway 'in a box' = the radio (Xilinx ZU67 RFSoC, 8 TX/8 RX) |
| NETCONF / EEPROM personality | radio configured via NETCONF (netopeer2); EEPROM holds model/flavour |
| Grandmaster (Falcon) | timing: PTP / IEEE-1588v2, GNSS, 10 MHz / 1PPS (LLS-C1) |
| RF switch (Mini-Circuits) | HTTP-controlled path routing between DUT and instruments |
| APC | remote power control / cycling |
| OTA chamber / rotator | TRP/EIRP over-the-air |

# 5. Putting the whole course together

| When you... | You are using... |
| --- | --- |
| read a power or RSSI number | M1 (dB), M4 |
| check VSWR / return loss | M2 |
| reason about sensitivity / noise | M3, M4 |
| look at a constellation / EVM | M5 |
| debug ACLR / spectral regrowth | M6 (chain/CFR/DPD), M7 |
| work on PA power / Vdd | M11 |
| run a PRACH test | M8 |
| work on the AAU / calibration | M9 |
| set up the bench / platform | M10 + RuAuto Field Guide + Validation ref |

# 6. The pitfalls that catch everyone

- Forgetting cable-loss / de-embedding.

- Overloading the analyzer (reference level too low).

- Wrong RBW, or no TDD gating.

- Loose/dirty connectors — torque to spec.

- Keying a PA into an open/short (Module 2).

# 7. Self-check

- Which instrument both generates and analyzes?

- Before trusting a power number, what correction applies?

- For a TDD Tx measurement, what feature is essential?

Answers 1. The Keysight EXM (VSG + VSA). 2. Cable-loss / path de-embedding. 3. Gating to the transmit slot.

# You have the full picture

Language (1-2), chain (3), metrics (4), signal (5), the Tx chain and its cleanup (6-7), PA efficiency (11), the two big test areas and the platform (8-9), tied to the bench (10) and to RuAuto + the references. Re-read in order, do the self-checks, and open the real configs and match each test to its module.