PARALLEL WIRELESS · RCT — STUDY GUIDE

Module 12

Reading the RCT Code: How Tests Are Written

Mentor's note: The practical half of the course. Tests in RuAuto are JSON, not code - so this walks through the REAL ACLR, EVM, power, sensitivity and PRACH definitions from your own FHGW setups, field by field.

# 0. The big idea: tests are DATA, not code

In RuAuto you almost never write a new test in Python. A test is a generic method driven by JSON. To 'write' or change a test you edit config. So reading the config IS reading the test. Every snippet below is taken verbatim from your own setups (RCT-4G-GR-FHGW-TRX1 and RCT-5G-GR-FHGW-TRX4).

## The 4-layer lookup chain

One test run is assembled from four files, all anchored by the setup's INI:

- Entry INI (setup.ini / setup_conf.ini) - lists which JSON files to load: [cws], [te], [tc_cadence], [rrh], [cable_loss], [kpi_thresholds].

- tc_cadence.json - three registries: Test-IDs (per-test parameters), Group-IDs (schedule tests over band/freq/BW/antenna/TM), Cadence-IDs (P0/P1/P2... pick which Groups run).

- cws/test_inputs setup (fdd_test_setup.json / tdd_test_setup.json) - the DUT: IPs, bands, antenna-to-RFIO mapping.

- kpi_thresholds_references/FHGW_Box_kpi_thresholds.json - the pass/fail limits with 3GPP references. For FHGW, the limits live HERE, not in tc_cadence.

4G vs 5G - a structural difference you must know 4G Test-IDs are THIN - just a filter, e.g. "test_Tx_ACLR": { "frequency": ["fm"], "etm": ["1.1"] } - and the KPI number lives in the threshold file. 5G Test-IDs are FAT - each carries test_type, prb, test_duration AND an inline KPIs block. So: for 4G read the threshold file for limits; for 5G read tc_cadence.json.

# 1. How you run one (the CLI)

python run_pytest.py \ --conf-file-path configuration/setup_configuration/RCT-5G-GR-FHGW-TRX4/tdd_test_setup.json \ --test-mode conformance \ --cadence P0 \ source/features/4g/tdd/Tx/conformance_nr_tdd_speed.py

Key flags: --conf-file-path (the setup), --test-mode (conformance is the RCT default), --cadence (P0 CI / P1 nightly / P2 regression / P3 sanity), and -k "test_Tx_ACLR" to run a single test. run_pytest.py wraps pytest, adds the HTML report, and keeps the last 5 log runs.

The flow each test follows: load the 4-layer config -> connect to the DUT through the Steps layer -> switch test model / start PHY (or play the ARB waveform for Rx) -> configure the analyzer mode from test_type -> measure -> apply cable-loss correction -> compare to the KPI -> write a Pass/Pass(Tolerance)/Fail row.

# 2. ACLR - the worked example, end to end

This is the one to know cold. Here is the real 5G Test-ID:

"test_Tx_ACLR": { "tm": "1.1", // E-TM test model 1.1 (QPSK, all RBs) "test_type": "ACP", // -> SCPI sa_set_ACP_mode (adjacent ch power) "prb": "273", // full carrier loaded "trx": "1", // transceiver index "interference": "-1", // no interferer "test_duration": "960000", // microseconds (long: ACLR needs dwell) "sample_time": "3", // average 3 samples "KPIs": { "aclr_kpi": 54, "aclr_kpi_tolerance": 1 } }

Read it: test_type:"ACP" selects the analyzer's adjacent-channel-power personality; aclr_kpi:54 is the minimum required ACLR in dB (higher is better); aclr_kpi_tolerance:1 is the 1 dB grace band for a Pass(Tolerance) verdict.

## The Group that schedules it

"TS_DL_CI": { "IsDL": true, "band": "n78", "frequency": ["M"], "BW": ["40"], "SCS": "30", "tm": ["1.1"], "ext_trig_delay": "0.00998675", "Tests": ["test_Tx_BS_Output_Power", "test_Tx_modulation_analysis", "test_Tx_ACLR"] }

The Group sets the axes (band n78, mid frequency, 40 MHz, 30 kHz SCS, TM 1.1) and lists which Test-IDs run. The Cadence then picks the Group: "P0": ["TS_DL_CI", "TS_UL_CI"].

## The 4G limit (threshold file)

"test_Tx_ACLR": { "3gpp_reference": "6.6.2", "min_limit": 55, // global 3GPP minimum ACLR (dB) "tolerance": 0.8, "band_min_limit": { // per-band / per-run overrides "b0407": 52, "b0307_second_run": 52, "b082028_second_run": 52, "b41_first_run": 52 } }

The lookup key is {band}_{dualband_step} if a dual-band step is set, else just the band - so b0307_second_run relaxes the limit to 52 dB on that specific pass. Validation function: limits_validation_aclr().

ACLR verdict (higher is better) Pass: measured_aclr >= aclr_kpi Pass(Tolerance): aclr_kpi - tol <= measured < aclr_kpi Fail: measured < aclr_kpi - tol 3GPP floor for the offsets: Offset A (+/-BW) < -30 dBc, Offset B (+/-2xBW) < -33 dBc.

# 3. BS output power - test_Tx_BS_Output_Power (6.2)

"test_Tx_BS_Output_Power": { "tm": "1.1", "test_type": "CP", // CP -> sa_set_channel_power_mode "prb": "273", "trx": "1", "KPIs": { "output_powerKPI": ["43", "49"] } } // dBm low/high window

Verdict by difference: diff = configured_power - measured_power; pass if within the window (5G doc cites +/-2.5 dB; 4G channelPower window +/-1.5 dB normal). For TDD the analyzer is gated to the transmit slot:

sa_set_chp_ref_level(rrh.tx_power) sa_set_chp_averaging(10) sa_set_gate_view_state('ON') sa_set_trigger_gate_length(0.001) # 1 ms - measure only the DL slot

# 4. EVM / modulation analysis - test_Tx_modulation_analysis (6.5.2)

One measurement returns several KPIs at once:

"test_Tx_modulation_analysis": { "tm": "1.1", "test_type": "EVM", // -> sa_set_modulation_analysis_mode "prb": "273", "trx": "1", "KPIs": { "output_powerKPI": ["43", "49"], "EVM_KPI": "18.5", // max EVM % (lower better) "Frequency_Error_KPI": ["-185","185"], // Hz bounds "TAE": "90" } } // time alignment error, ns

The 4G limit is per-E-TM (lower EVM allowed for higher-order QAM):

| E-TM | Modulation | Max EVM % |
| --- | --- | --- |
| 1.1 | QPSK | 17.5 |
| 3.1 / 2.0 | 64QAM | 8.0 |
| 3.1a / 2a | 256QAM | 3.5 |
| 3.2 | QPSK/16QAM | 12.5 |

Pass if measured_evm <= min_limit; Pass(Tolerance) if <= min_limit + tolerance (1.0).

# 5. Frequency error (6.5.1) and the masks (6.6.3 / 6.6.4)

Frequency error limit is in ppm of the carrier: ppm:0.05, tolerance_hz:12 -> limit_Hz = 0.05e-6 x f_carrier (+12 Hz). The SEM/spurious limits are stepped tables, e.g. the 4G mask:

"test_lte_Tx_Mask": { // SEM, 6.6.3 "ranges": [ // [df_start MHz, df_stop, lim_start dBm, lim_stop, MBW kHz] [0.0, 5.0, -5.5, -7.5, 100], [5.0, 10.0, -12.5, -12.5, 100], [10.0, null, -13.0, -13, 1000] ] }

Spurious (6.6.4) is similar but in Hz across 9 kHz to 12.75 GHz, with a ul_protection guard at -96 dBm protecting the receive band. test_type SEM -> sa_set_sem_meas, SPUR -> sa_set_spurious_emissions_mode, OBW -> sa_set_OBW_mode.

# 6. Reference sensitivity - test_Rx_Reference_Sensitivity_Level (7.2)

An uplink test: the generator plays an FRC waveform and the framework sweeps power DOWN to find where BLER crosses the threshold.

"test_Rx_Reference_Sensitivity_Level": { "FRC": "A1-5", "waveform_type": "wfm", "RF_port": "RFIO 1", "init_power": -35, // sweep start (dBm) "coarse_step": "2", "fine_step": "0.5", "finest_step": "0.1", // 3-stage "fault_power": "-34", // safety ceiling "BLER_TH": "5", // 5% BLER defines the sensitivity point "KPIs": { "sensitivity_KPI": "-46", "sensitivity_tolerance": "1.0" } }

The 3-stage stepped search (coarse 2 -> fine 0.5 -> finest 0.1 dB) is find_sensitive_point(). The recorded result is the cable-loss-corrected RU input power: UL input = SG power - cable_loss. On the FHGW the limit is DSA-adjusted at runtime: base_dbm + DSA(band) (e.g. base -73 -> band 1 gives -68). A bler_100_max_steps guard aborts early to Fail if BLER is stuck at ~100% (broken UL path).

Sensitivity verdict (lower input is better) Pass: ul_input_power <= sensitivity_KPI Pass(Tolerance): <= sensitivity_KPI + tolerance Fail: otherwise

# 7. PRACH - test_Rx_PRACH (8.4.1), dual mode

"test_Rx_PRACH": { "prach_format": ["B4"], "idx": ["idx158"], "signal": ["PRACH_idx158_40MHZ.wfm"], // the ARB waveform "prach_scs": [30000], "n_rb": [12], "search_mode": "binary", // or "sensitivity" "low_pwr_gap": -5, "high_pwr_gap": 6, "res": 1, // binary bounds/res "coarse_step": 2, "fine_step": 0.5, "finest_step": 0.1, // sensitivity "miss_detect_threshold": 1, // target 1% missed detections "fault_power": -34, "KPIs": { "PRACH_KPI": -68, "PRACH_tolerance": 1 } }

Two search modes (Module 8): binary - fast binary search to res dB (5-7 iterations); sensitivity - coarse 2 -> fine 0.5 -> finest 0.1 dB targeting 1% miss-detect (15-25 iterations). Verdict: Pass if RU input <= PRACH_KPI; Pass(Tolerance) within +tolerance; else Fail. Output columns include miss-detect %, false-alarm %, timing-advance avg/std, PAPR, setup loss.

# 8. Receiver selectivity / blocking (7.4, 7.5)

The 5G setup has the fully-numeric versions (the 4G threshold file still has these as TODO). Example ACS:

"ACS_7_4_1": { "FRC": "A1-5", "test_duration": "8000000", // 8 s (interferer present) "init_power": -46, "coarse_step": "6", "BLER_TH": "5", "KPIs": { "sensitivity_KPI": "-89.6", "sensitivity_tolerance": "1.0" } } "InBandGeneralBlocking_7_4_2A": { "interfer_wave": "Interfering_20MHz_15KHz_100RBs.wfm", "start_interfer_freq_left": "3780", "stop_interfer_freq_left": "3575", "Sweep_gap_interfer": "1", // step the blocker in 1 MHz "KPIs": { "sensitivity_KPI": "-89.6", "sensitivity_tolerance": "1.0" } }

Same pattern: an interferer waveform is added, its frequency swept, and the wanted-signal power at the BLER crossing is compared to the KPI. TS_UL_Regression lists DynamicRange_7_3, ACS_7_4_1, InBand/OutOfBand blocking, ReceiverIntermodulation_7_7, InChannelSelectivity_7_8 - all under cadence P2_FULL.

# 9. Cable loss - applied everywhere

4G cable_loss.json is a dense per-frequency table (antenna -> socket -> {freq: loss_dB}); 5G uses a compact component model (EXM, RRH, Propsim). For Tx tests the loss corrects the measured power; for Rx tests the generator power is raised by the loss and the reported RU input = SG power - cable_loss. This is the de-embedding from Module 1 - the reason every reading is trustworthy.

# 10. The verdict enum (all tests)

| Verdict | Meaning |
| --- | --- |
| Pass | measured meets the raw 3GPP limit (no tolerance used) |
| Pass (Tolerance) | measured falls in the tolerance band (limit -/+ tolerance) |
| Fail | outside the tolerance band |

Surfaced in source as PASS_RESULT / MARGINAL_RESULT / FAIL_RESULT, written to report.html and the Excel workbook with the measured value, the KPI, the tolerance, and the frequency/BW/antenna/TM.

# 11. How to read ANY test yourself (the recipe)

- Open the setup's tc_cadence.json. Find the Test-ID by name (e.g. test_Tx_ACLR).

- Note its test_type (tells you the analyzer mode) and inline KPIs (5G) - or look up the limit in FHGW_Box_kpi_thresholds.json (4G).

- Find the Group whose Tests list contains it - that gives band/freq/BW/antenna/TM.

- Find the Cadence that includes that Group - that is the --cadence you would run.

- Check cable_loss.json for the path correction, then read the verdict rule (higher-better vs lower-better).

Self-check 1. In test_Tx_ACLR, what does test_type:"ACP" control? 2. For 4G, where do you find the ACLR limit - tc_cadence or the threshold file? 3. Sensitivity uses a 3-stage search. What ends the search (defines the point)? 4. PRACH binary vs sensitivity mode - which is faster, which is more precise? Answers: 1) the spectrum-analyzer measurement mode (sa_set_ACP_mode). 2) the threshold file (FHGW_Box_kpi_thresholds.json). 3) BLER crossing BLER_TH (5%). 4) binary is faster (~5-7 iters), sensitivity is more precise (~0.1 dB).

# Takeaways

- A test = Test-ID (params) + Group (axes) + Cadence (when) + KPI (limit). Editing config IS writing the test.

- test_type picks the analyzer mode; 5G keeps KPIs inline, 4G keeps them in the threshold file.

- Rx tests sweep power to a BLER crossing; verdicts are Pass / Pass(Tolerance) / Fail; cable loss is always applied.

Pair this with the RuAuto Field Guide (the framework) and the Conformance Methodology reference (the 3GPP method). Now you can open any setup in the repo and read its tests.