RF ENGINEERING — STUDY GUIDE

Module 3

The RF Chain — the Main RF Flow

Mentor's note: Modules 1-2 gave you the language (dB) and the plumbing (50 ohm). This module is the radio itself. Every block here appears in your AAU block diagram, at every power_monitor tap, and as the thing each conformance test measures. Learn the chain once and the rest of the course is just zooming into individual blocks.

# 1. The whole chain at a glance

Figure: The transmit and receive chains, sharing one antenna through the duplexer.

A radio is two signal paths sharing one antenna. Trace the transmit path left-to-right along the top, and the receive path right-to-left along the bottom.

# 2. Transmit side — building the signal up

| Stage | What it does |
| --- | --- |
| Digital IQ | The waveform as numbers (baseband), measured in dBFS. This is what IQ playback injects. |
| DAC | Digital-to-analog: turns the IQ samples into an analog baseband signal. |
| Mixer + LO | Up-conversion. Multiplies baseband by the Local Oscillator to land on the carrier: f_RF = f_LO + f_baseband. |
| Filter | Removes the unwanted mixer image and out-of-band products, leaving a clean band. |
| PA | Power amplifier: lifts a milliwatt-level signal to tens of watts. The hottest, most non-linear block — the reason DPD exists (Module 6). |
| Duplexer / TDD switch | Routes Tx to the shared antenna (FDD: frequency-split duplexer; TDD: time switch). |

# 3. Receive side — pulling a whisper out of the noise

| Stage | What it does |
| --- | --- |
| LNA | Low-noise amplifier: the FIRST thing the weak (~ -100 dBm) signal hits. High gain, very low noise — it sets the whole receiver's noise figure. |
| Mixer + LO | Down-conversion back to baseband: f_baseband = f_RF - f_LO. |
| Filter | Selects the wanted channel, rejects neighbours. |
| ADC | Analog-to-digital: samples the signal into IQ for the modem. |

# 4. Equation 1 — gain cascades by addition

This is just the Module-1 link budget applied inside the radio. Total chain gain (dB) = the sum of every stage's gain and loss. Add the column:

DAC drive +5 dB filter loss -2 dB PA gain +40 dB ------------------- net +43 dB

# 5. Equation 2 — noise figure, and why the LNA is first

Every stage adds a little noise and degrades the signal-to-noise ratio (SNR). Crucially, WHERE the noise is added matters. Friis's cascade formula:

F_total = F1 + (F2-1)/G1 + (F3-1)/(G1*G2) + ...

The first stage's noise passes through undivided; every later stage's noise is divided by all the gain ahead of it. So a high-gain, low-noise first stage swamps everything downstream. That is the whole reason the LNA is first.

Figure: The first stage dominates the total noise figure; later stages are divided down by the gain ahead of them.

# 6. Noise floor and sensitivity — where the chain meets the test

The receiver noise floor and reference sensitivity are built straight from this chain's noise figure:

Noise floor (dBm) = -174 + 10*log10(BW in Hz) + NF Sensitivity = -174 + 10*log10(BW) + NF + required_SNR

-174 dBm/Hz is thermal noise at room temperature, a physical constant. Stack the contributions and you arrive at the spec number your RCT sensitivity test (7.2) checks:

Figure: Reference sensitivity is the thermal floor plus bandwidth, noise figure, and required SNR.

The cascade is the pass/fail A bad LNA -> worse noise figure -> higher noise floor -> worse sensitivity -> a FAILED test 7.2. When a radio hears down to -100.8 dBm, that number was set by this chain. Nothing 'abstract' here — Friis decides whether the unit ships.

# 7. How this maps to your world

- Your AAU block diagram is this chain replicated per branch (and per band). Modules 6 and 9 zoom into the PA and the multi-branch layout.

- power_monitor reads dBFS at points along the digital end of this chain (DL near the DAC input, UL near the ADC output).

- Conformance tests measure the two ends: Tx tests check what comes OUT of the PA (power, ACLR, EVM); Rx tests check what the chain can HEAR (sensitivity, blocking).

# 8. Self-check

- In which order does a received signal hit these: ADC, LNA, mixer, filter?

- Why is the LNA placed first and made both low-noise AND high-gain?

- A receiver has BW = 10 MHz (10*log = 70 dB), NF = 3 dB, required SNR = 2 dB. What is its sensitivity?

- If f_LO = 2000 MHz and baseband = 10 MHz, what is the transmit carrier frequency?

Answers 1. LNA -> mixer -> filter -> ADC (amplify the weak signal BEFORE doing anything noisy to it). 2. Friis: the first stage's noise is undivided and its high gain divides down all later noise, so it sets the receiver's NF. 3. -174 + 70 + 3 + 2 = -99 dBm. 4. f_RF = f_LO + f_baseband = 2010 MHz.

# Takeaways

- A radio = Tx chain (DAC->mixer->filter->PA) and Rx chain (LNA->mixer->filter->ADC) sharing one antenna.

- Gain cascades by addition (the link budget, inside the box).

- Noise figure is set by the first stage (Friis) — hence the low-noise, high-gain LNA up front.

- Sensitivity = -174 + 10log(BW) + NF + SNR. The chain decides the pass/fail of test 7.2.

Next — Module 4: Key radio metrics. We turn your floating glossary (RSSI, sensitivity, TRP, EIRP, dynamic range) into precise, measurable definitions anchored on this chain.