PARALLEL WIRELESS · RCT

RF & RCT Glossary

Quick definitions for the terms across the course and the RuAuto framework. Module references point to where each is taught.

| Term | Definition |
| --- | --- |
| ACLR | Adjacent Channel Leakage Ratio: wanted-channel power / power leaking into the adjacent channel, in dBc. Test 6.6.2. (M7) |
| ADC / DAC | Analog-to-Digital / Digital-to-Analog Converter: the boundary between digital IQ and analog RF. (M3) |
| AAU | Active Antenna Unit: radio chains + antenna array integrated in one unit. (M9) |
| AWGN | Additive White Gaussian Noise: the standard noise used as an interferer in Rx tests. |
| Back-off | Running a PA below saturation to leave headroom for signal peaks (high PAPR). (M6) |
| Beamforming | Steering radiated energy by setting the phase/amplitude of each antenna element. (M9) |
| BLER | Block Error Rate: fraction of transport blocks decoded with errors; the Rx/performance pass criterion (<=5%). |
| Cable loss / de-embedding | Characterizing the test-path loss and adding it back so power is reported at the connector. (M1, M7) |
| dB | Decibel: a power ratio, 10*log10(Pout/Pin). (M1) |
| dBc | Decibels relative to the carrier (used for ACLR, spurs). (M1) |
| dBFS | Decibels relative to full scale: digital headroom in the pipeline; 0 dBFS = clipping. Not the same as dBm. (M1) |
| dBi | Antenna gain relative to an isotropic radiator; feeds EIRP. (M1) |
| dBm | Absolute power referenced to 1 mW. 0 dBm = 1 mW, +30 dBm = 1 W. (M1) |
| DPD | Digital Pre-Distortion: pre-warping the signal with the PA's inverse so the cascade is linear. (M6) |
| Duplexer | Component that lets Tx and Rx share one antenna (FDD); a switch in TDD. (M3) |
| Dynamic range | Usable span from noise floor to overload. (M4) |
| EARFCN | E-UTRA Absolute Radio Frequency Channel Number: the integer that maps to a carrier frequency. |
| EIRP | Effective Isotropic Radiated Power = Tx power + antenna gain - losses (best direction). (M4) |
| E-TM | E-UTRA Test Model: the defined downlink signal used for Tx conformance tests. (Catalog) |
| EVM | Error Vector Magnitude: how far measured symbols miss ideal, as a percent. Test 6.5.2. (M5) |
| FRC | Fixed Reference Channel: the defined uplink signal used for Rx tests. (Catalog) |
| Friis formula | Noise-figure cascade: the first stage dominates the total NF. (M3) |
| Gamma | Reflection coefficient = (ZL-Z0)/(ZL+Z0). 0 matched, +/-1 total reflection. (M2) |
| IQ | Signal as in-phase (I) and quadrature (Q) components; a point on the 2D plane. (M5) |
| IQ playback | Playing a pre-computed IQ waveform from a generator to stimulate the radio. (M5) |
| LNA | Low-Noise Amplifier: first Rx stage; sets the receiver noise figure. (M3) |
| LO | Local Oscillator: the reference the mixer uses to up/down-convert. (M3) |
| Mixer | Frequency translator: f_RF = f_LO +/- f_baseband. (M3) |
| Noise figure (NF) | How much a stage degrades SNR; sets the noise floor and sensitivity. (M3) |
| OBUE | Operating Band Unwanted Emissions: spectrum must stay under a mask. Test 6.6.3. (M7) |
| OFDM | Multi-subcarrier modulation; subcarrier x symbol = resource element. (M5) |
| P1dB | 1 dB compression point: where PA gain has dropped 1 dB from linear. (M6) |
| PAPR | Peak-to-Average Power Ratio: OFDM peaks far above average (~10 dB). (M6) |
| PA | Power Amplifier: boosts Tx to tens of watts; the main non-linear block. (M3, M6) |
| PRACH | Physical Random Access Channel: the UE's first uplink contact. Test 8.4. (M8) |
| RB / PRB | Resource Block: 12 subcarriers; the scheduling unit. (M5) |
| RBW / VBW | Resolution / Video Bandwidth on a spectrum analyzer. (M7) |
| Return loss | -20*log10¦Gamma¦: how far below incident the reflection is. Bigger = better. (M2) |
| RSSI | Received Signal Strength Indicator: total in-band power at the Rx input. (M4) |
| Sensitivity | Minimum input power meeting the error-rate target. Test 7.2. (M3, M4) |
| SEM | Spectrum Emission Mask: the stepped ceiling the Tx spectrum must fit under. (M7) |
| SCPI | Standard Commands for Programmable Instruments: text commands over LAN to the EXM. (M10) |
| TRP | Total Radiated Power: power integrated over the whole sphere (OTA). (M4) |
| VSWR | Voltage Standing Wave Ratio = (1+¦Gamma¦)/(1-¦Gamma¦). Near 1 = good match. (M2) |
| Zadoff-Chu | Constant-amplitude sequence with a sharp autocorrelation; the PRACH signature. (M8) |