PARALLEL WIRELESS · RCT — STUDY GUIDE

Reference

Band Plan & Front-End Components

Mentor's note: Every module talks about 'the PA', 'the filter', 'the switch'. This names the real parts in your AAU and the bands they serve, from the schematics you shared.

Grounds the whole course in the real hardware: which bands the AAU serves, and the actual front-end parts that implement each chain.

# 1. The band plan

Figure: The bands the platform serves; B41 is TDD, the rest FDD.

| Band | Rough freq | Duplex |
| --- | --- | --- |
| B28 | ~700 MHz | FDD |
| B20 | ~800 MHz | FDD |
| B8 | ~900 MHz | FDD |
| B3 | ~1800 MHz | FDD |
| B1 / B4 | ~2100 MHz | FDD |
| B7 | ~2600 MHz | FDD |
| B41 | 2496-2690 MHz | TDD |

FDD bands use paired uplink/downlink frequencies (a duplexer separates them). B41 is TDD: uplink and downlink share the frequency in alternating time slots (a switch, and gated measurements).

# 2. The board split

| Block | Role |
| --- | --- |
| FHGW (RFSoC) | digital: DUC/DDC, power scaling, CFR, DPD, DAC/ADC; the brains |
| Mid-board | filtering, splitting/combining, calibration coupling between FHGW and RFFE |
| RFFE (RF front end) | the high-power analog: PAs, LNAs, switches, circulators, antenna feed |

# 3. Front-end components you will meet

| Component | What it does |
| --- | --- |
| Doherty PA (carrier + peaking) | high-efficiency PA: a main amplifier plus a peaking amplifier that engages on peaks |
| Driver / MMIC | pre-amplifier stages feeding the PA |
| Circulator | routes Tx out to the antenna and Rx in to the LNA; isolates reflections |
| SAW / BPF filters | band selection and rejection (F0180, F1490, BFCN-xxxx, etc.) |
| Phase shifter (PE44820DS) | 0-360 deg phase control per element - the knob for beamforming |
| PIN-diode switches (SPDT/SP4T) | route paths (BGS12P2L6, PE42420, PE42445); must be absorptive, >40 dB isolation |
| Directional couplers (10/20/25 dB) | tap a small sample for power detection and DPD observation |
| RF power detector (LT5581) | measures power for monitoring/leveling |
| Balun / pads / V-ATT / term | single-to-differential, fixed/variable attenuation, 50 ohm termination |

Why Doherty matters A Doherty PA keeps efficiency high across a wide power range by adding a 'peaking' amplifier only when the signal peaks demand it - directly relevant to the efficiency work in Module 11.

# 4. Antenna arrays

Sub-array layouts differ by band group: a 4-2-2-4 element arrangement (12 elements, 4 columns) for B1/B3/B41, and 3-3 (6 elements, 2 columns) for B8/B20/B28 - which is why beamforming gain is higher (~+12 dB) on the former.