PARALLEL WIRELESS · RCT — STUDY GUIDE

Reference

3GPP TS 36.141 Conformance Test Methodology

Mentor's note: The Test Catalog lists WHAT is tested; this teaches HOW. E-TMs, FRCs, frequency points, the full DL/UL suite, and the measurement traps (cable loss, TDD gating).

This reference teaches the METHOD behind the test list in the 4G RCT Test Catalog. It is distilled from your full 4G test plan (3GPP TS 36.141).

# 1. The test signals

Transmitter tests transmit an E-UTRA Test Model (E-TM); receiver tests are stimulated by a Fixed Reference Channel (FRC) and judged on BLER (<= 5%).

| E-TM | Modulation / use |
| --- | --- |
| 1.1 | QPSK, all RBs - power, ACLR, OBUE, spurious, intermod |
| 1.2 | QPSK boosted/deboosted |
| 2 / 2A | 64QAM / 256QAM, 1 RB - dynamic-range lower limit |
| 3.1 / 3.1A | 64QAM / 256QAM, all RBs - EVM, frequency error |
| 3.2 / 3.3 | QPSK/16QAM mixed - EVM |

# 2. The three test frequencies

For each (band, bandwidth) you test at three points across the band: bottom (fb = DL_min + BW/2), middle (fm), and top (ft). This catches frequency-dependent problems (filters, gain flatness).

# 3. Downlink (transmitter) test suite

| Test | Method in one line |
| --- | --- |
| BS output power | measure mean power per carrier at the connector (E-TM1.1) |
| Total power dynamic range | power(full RBs, E-TM3.1) - power(1 RB, E-TM2) |
| ACLR (+ OTA) | wanted-channel power / adjacent-channel power, in dBc |
| DL reference signal power | measured RS power accuracy |
| EVM (+ OTA) | modulation error vs ideal, per modulation limit (E-TM3.1) |
| Frequency error | carrier vs assigned frequency over 1 subframe |
| Time alignment error | delay between antennas (Tx diversity / MIMO) |
| Modulation analysis | constellation / EVM diagnostics |
| OBUE / SEM | spectrum must fit under the stepped mask |
| Spurious emissions | limits across 1 MHz - 12.75 GHz |

# 4. Uplink (receiver) test suite

These are broader than just sensitivity — each is a distinct measurement of how the receiver behaves in the presence of impairments or interferers:

| Test | What it proves |
| --- | --- |
| Reference sensitivity (+ OTA) | min input power meeting BLER (FRC A1-3) |
| RSSI readback calibration | the radio's reported RSSI matches the true input power |
| Adjacent channel selectivity (ACS) | reject an interferer just outside the channel |
| In-channel selectivity (ICS) | decode wanted signal beside a stronger in-band signal |
| Narrowband blocking | reject a 1-RB interferer near the band edge |
| Wideband blocking / blocking sweep | reject in-band LTE / out-of-band CW interferers |
| Receiver intermodulation | reject IM products from two interferers |

# 5. Measurement mechanics that bite

- Cable-loss de-embedding: apply the per-frequency loss (cable_loss.json) via the analyzer's gain-correction (for example :CORR:BTS:GAIN <-cable_loss>).

- TDD gating (B41): the analyzer must be gated to the transmit slot - for example standard preset config 3, special-subframe config 8 (SSFR 8), gate delay ~3.1 ms, gate length ~2.8 ms.

- Pass windows: for example power +/-1.5 dB, ACLR judged against a PW gate (about -45 dBc).

# 6. The platform

The radio is an FHGW (fronthaul gateway) built on a Xilinx ZU67 RFSoC with 8 TX + 8 RX ports; antenna mapping differs per band group (b0103, b0307, b0407, b082028, b41). The Keysight EXM E6640A is software-toggled between signal analyzer (Tx tests) and signal generator (Rx tests). Control is SCPI over LAN; the radio is configured via NETCONF.

How it maps to RuAuto Each test here is a Test-ID in tc_cadence.json with its KPI limit; the E-TM/FRC become the test-model/waveform fields; cadences P0-P4 pick which groups run. See the RuAuto Field Guide.