PARALLEL WIRELESS · RCT — STUDY GUIDE

Reference

Validation Team & CI/CD Process

Mentor's note: The process around the RF: how the team plans, reviews code, runs CI, pre-checks a unit, and brings up a lab. From the Validation Team Handbook.

How the validation team actually works day to day - the process around the RF. Distilled from the Validation Team Handbook.

# 1. Ways of working

The team runs Agile/Scrum: sprints, daily stand-ups, and PI (program increment) planning. Work is tracked as tickets and delivered in small, reviewed increments.

# 2. Code and review (ru-pytest)

- Source lives in Bitbucket; changes go through Pull Requests with review.

- Branch flow: private branch -> DEV -> Integration/CI. Never commit straight to shared branches.

- Code quality is enforced: PEP8, black formatting, Pylint.

# 3. CI/CD and cadences

Jenkins runs the tests automatically. Cadence IDs select scope: P0 (CI / smoke), P1 (nightly), up to P2-P4 (regression / full). Per-band jobs exist (for example RCT-GR-CI-B1B3, B82028, B3B7, B4B7, N78, B41).

# 4. PreCheck tests

Before a real run, quick PreChecks confirm the unit is sane:

| PreCheck | Confirms |
| --- | --- |
| Version | correct software/firmware loaded |
| Power Monitor DL | downlink digital power (dBFS) is in range |
| CFR Input Power | CFR input level is correct |
| DPD Gain Register / DPD Input Power & PAPR | DPD is configured and fed correctly |
| Activation File / Factory XMLs | right configuration and calibration data present |
| EEPROM Model Name | the unit identifies as the expected model |

# 5. BKC and dashboards

A Best Known Configuration (BKC) records the known-good setup (software, settings, calibration) so runs are reproducible. Results stream to Kibana dashboards for trends and triage.

# 6. Lab bring-up

| Element | Notes |
| --- | --- |
| DU | the baseband unit that drives the radio |
| FHGW | fronthaul gateway 'in a box' - the radio under test |
| Grandmaster (Falcon) | timing source: PTP / IEEE-1588v2, GNSS, 10 MHz / 1PPS (LLS-C1) |
| RF switch (Mini-Circuits) | HTTP-controlled path routing between DUT and instruments |
| UE (OnePlus 12R, rooted, SIM 00101) | for end-to-end / E2E checks |
| APC power control | remote power cycling of equipment |
| OTA chamber / rotator | over-the-air TRP/EIRP measurements |

Where this connects The PreChecks (Power Monitor, CFR, DPD) are Modules 1/6/11 in action; the cadences and configs are the RuAuto Field Guide; the bench is Module 10.