PARALLEL WIRELESS — RCT / RU-PYTEST

The RuAuto Test Framework

A New Engineer's Field Guide: Architecture, How Tests Are Formulated, and the Full Test Catalog

Scope note: This guide is built from the docs/ and configuration/ trees of the repo you shared. Those describe the framework in unusual detail and quote the relevant code. One caveat: this extract did not include the source/*.py implementation tree, so code paths like source/features/... and infrastructure/utils.py are described from the documentation that references them, not read line-by-line. The JSON/INI configs that drive the tests ARE present and are quoted directly.

# Contents

# 1. What RuAuto is

RuAuto (internally RU-pytest) is Parallel Wireless's automated test harness for Radio Units / base stations across 2G, 3G, 4G and 5G. Its job: take a radio on the bench, drive it and the test instruments through a complete 3GPP conformance test, measure the results, compare them against the spec limits, and produce a Pass/Fail report — repeatably and without a human turning knobs.

## The technology stack

| Layer | Technology | Role |
| --- | --- | --- |
| Engine | Python + pytest | Primary test runner. Always launched via run_pytest.py (not bare pytest). |
| Alt engine | Robot Framework | Secondary; run_robot.py. Robot keywords are thin wrappers over the Python Steps layer. |
| Config | JSON + INI | The 'backbone' — defines every test, limit, instrument and cable loss. Data, not code. |
| Trace tools | Lua (Wireshark) | pw_2g..5g.lua dissectors parse protocol traces for analysis (not in the live test path). |
| Integrations | K8s, Elasticsearch, QXDM, TestRail | 5G pod deploy, results storage/PM counters, UE logging, test management. |

# 2. The 3-layer architecture (the golden rule)

RuAuto enforces a strict separation. A test never talks to a radio directly — it goes through Steps, which drive Products. Learn this and the whole codebase becomes navigable:

TEST LAYER source/features/ test_*.py / *.robot -- assertions ONLY ¦ calls STEPS LAYER source/steps/ CwsSteps, VrpSteps, AttenuatorSteps, OamKpiSteps -- workflows ¦ uses PRODUCT LAYER source/products/ VRU, CWS, NR, HNG, RRH -- SSH/CLI/XML to the device

The right Product class is auto-selected from config flags (technology + product_type + duplex + fhgw/docker/dual_band) by a decorator registry (@register_product mapping a frozen CwsKey dataclass to a class). Product classes form a deep inheritance chain — Cws (base) -> Vru -> NR_tdd — so 5G inherits the entire 2G/3G/4G code base.

Why you care When a test fails, this tells you WHERE to look: a wrong number -> Test layer (limit/assertion); a radio that won't configure -> Steps/Product layer; a missing instrument -> config (env_setup.json).

# 3. How a test run is orchestrated, end to end

A run flows like this:

- python run_pytest.py --conf-file-path <setup>.ini --test-mode conformance --cadence P1 <test_file.py>

- The wrapper creates a timestamped log dir (logs/pw_test_<timestamp>/) and keeps only the last 5 runs.

- conftest.py registers ~58 CLI options; the test_params fixture calls test_config.load_configurations(), which merges every referenced JSON file (resolving $ref pointers) into one config singleton.

- The test connects via cws_steps.cws_connect(...) (returns an integer handle), which instantiates the right Product class.

- The test iterates the cadence -> groups -> (band x freq x BW x antenna x TM) -> tests; configures the radio; drives the instruments; measures; validates against the limit; writes verdicts.

- Results land in report.html, multi-sheet .xlsx workbooks, CSV KPI files, and optionally Elasticsearch.

The --test-mode switch (CONFORMANCE / E2E / SYSTEM / NON_CONFORMANCE) changes which configs load, whether devices reboot, the product variant, and timeouts. CONFORMANCE is the RCT lab default.

# 4. How a test is FORMULATED (the core idea)

This is the most important section. A conformance test is not a hard-coded function with fixed numbers. It is a generic, parameterized method driven entirely by JSON. The key file per setup is tc_cadence.json, which has three blocks:

| Block | What it is |
| --- | --- |
| Test-IDs | The DEFINITION of each test: its parameters, instrument settings, and KPI (pass/fail) limits. Loosely grouped by 3GPP chapter. |
| Group-IDs | Axis sets: which band, frequencies (B/M/T = bottom/mid/top), bandwidths, antennas, transmission modes, SCS — plus a 'Tests' list naming which Test-IDs run on that axis. |
| Cadence-IDs | Named presets (P0 CI, P1 Nightly, P2_FULL Regression, P3 Sanity) listing which Groups to run. --cadence=P1 selects one. |

So a run iterates: cadence -> groups -> (band x freq x BW x antenna x TM) -> tests-in-group -> look up the Test-ID -> execute -> validate. Adding a test point is usually a JSON edit, not new Python.

## Where the numbers come from — four merged layers

| Layer | Directory | Holds |
| --- | --- | --- |
| 1. Hardware | setup_configuration/<lab>/ | device IPs, credentials, instrument inventory, cable-loss tables |
| 2. RF source | cws_sources_<tech>/ | cell/RF parameters, EARFCN<->frequency raster (earfcn.json) |
| 3. Test inputs | test_inputs_<tech>/ | test-specific knobs |
| 4. KPI limits | kpi_thresholds_references/*.json | the 3GPP-mapped pass/fail limits |

Later layers override earlier ones in the merge. The universal pattern for any single test:

set up radio (mplane/CLI update, switch test model, start PHY/L1-L2) -> generate/playback signal (DL: RU transmits a 3GPP test model; UL: signal generator plays an IQ waveform into the RU) -> measure (TX: spectrum/signal analyzer; RX: RU L1/L2 stats) -> correct for cable/path loss -> compare to limit -> verdict: Pass / Pass(Tolerance) / Fail + write a result row

# 5. Three real tests, walked end to end

## A. TX Power & ACLR (5G NR, TS 38.104 sec.6)

From the real RCT-5G-GR-FHGW-TRX4/tc_cadence.json. Test-ID definitions are pure data:

"test_Tx_BS_Output_Power": { "tm":"1.1","test_type":"CP","prb":"273","trx":"1", "KPIs": { "output_powerKPI": ["43","49"] } }, "test_Tx_ACLR": { "tm":"1.1","test_type":"ACP","prb":"273", "KPIs": { "aclr_kpi": 54, "aclr_kpi_tolerance": 1 } }

Flow: bring up the carrier on the group's band/BW/freq (e.g. n78, 40 MHz, mid), switch to test model TM 1.1, run PHY. The RU transmits; the Keysight EXM acts as a signal analyzer. For channel power the framework sets the reference level, averages 10 sweeps, and for TDD enables gating (gate length 1 ms) so it measures only the downlink slot. Measured power is corrected by extract_cable_loss(...). Validation: channel power -> diff = configured - measured, tolerance +/-2.5 dB; ACLR -> measured leakage must sit below the limit. A row is written with Frequency, Bandwidth, Antenna, TM, Measured Power, KPI, Tolerance, Verdict.

The same TX file also does EVM/freq error (EVM 18.5%, freq error +/-185 Hz), time alignment (TAE 65 ns), ON/OFF power, OBUE, spurious, and intermodulation. The instrument is reconfigured per test via a mode map ("ACP"->sa_set_ACP_mode, "EVM"->sa_set_modulation_analysis_mode, "SEM"->sa_set_sem_meas, etc.).

## B. PRACH detection (4G/5G, TS 36.141 sec.8.4)

An UPLINK test: the EXM is now a signal generator playing back a pre-recorded PRACH IQ waveform, and the RU's own PRACH detector is the measuring instrument. The framework sweeps the generator power down to find the detection threshold. Two search modes:

- binary — fast binary search between [KPI+low_gap, KPI+high_gap], converging to res dB.

- sensitivity — 3-resolution stepped search (coarse 2 -> fine 0.5 -> finest 0.1 dB), targeting 1% miss-detection, capped by a fault_power -34 dBm safety limit.

Real params (5G): prach_format ["B4"], signal ["PRACH_idx158_100MHZ.wfm"], prach_scs 30000, PRACH_KPI -70, tolerance 1. On the FHGW the limit comes from the threshold file (test_Rx_PRACH.base_dbm = -87) with a per-band DSA (digital step attenuator) added at runtime: base_dbm + DSA(band). Verdict: Pass if RU input power <= KPI; Pass(Tolerance) within +tolerance; else Fail. Output includes miss-detection %, false-alarm %, timing-advance avg/std, SNR.

## C. Static reference sensitivity (2G, TS 51.021 sec.7.3)

The minimum uplink input power at which decoded data still meets the error-rate limit (FER/RBER/BER/BLER) per logical channel, under static (no-fade) conditions. Flow: all 4 TRX on at full power; a Keysight E6640A plays a per-channel IQ waveform (e.g. AFS12_2_swapped_zeros_f26.bin) into the BTS uplink port at controlled power; a 3-stage stepped search (coarse 1.0 -> fine 0.5 -> finest 0.1 dB) sweeps power down while the BTS reports error rate. UL input power = generator power - cable loss.

Limits come from 3GPP Table 7.3-6, mapped per codec: e.g. TCH/EFS -> -104 dBm, FER <= 0.1%; AFS12.2 -> -104 dBm, FER <= 0.001%; PDTCH/MCS1-4 -> BLER <= 10%. Verdict: Pass if measured <= reference; Pass(Tolerance) within +0.7 dB; else Fail. The full matrix is ~1,984 points (SFH x ARFCN B/M/T x SISO/MRC/IRC x antenna x channel x codec).

# 6. The full test catalog & 3GPP mapping

| Family | Tests | 3GPP spec |
| --- | --- | --- |
| 5G NR TX | channel power, EVM/freq error, dynamic range, TAE, occupied BW, ACLR, OBUE, spurious, intermod, ON/OFF, PA protection | TS 38.104 sec.6.2-6.7 |
| 5G NR RX | reference sensitivity (incl. mMIMO), dynamic range, ACS, in-band blocking, out-of-band blocking, receiver intermod, in-channel selectivity | TS 38.104 sec.7 |
| 4G LTE TX | power, ACLR, EVM, SEM, frequency error | TS 36.104 / 36.141 sec.6 |
| 4G LTE RX | unified RX: sensitivity, ACS, blocking; E-UTRA interferer ranges | TS 36.104 / 36.141 sec.7 |
| NB-IoT + LTE | combined TX (EVM, freq error, OBW, ACLR, power boost) & RX (sensitivity, RSSI, blocking) | TS 36.104 sec.7.2/7.3, 36.141 |
| PRACH | test_Rx_PRACH dual-mode (binary / sensitivity) | TS 36.141 sec.8.4 |
| 2G GSM | static reference sensitivity, PVT, ACP | TS 51.021 sec.6.4/6.5/7.3 |
| Pre-checks | power monitor, PAPR, DPD, EEPROM integrity, n78 smoke | vendor + 3GPP |

# 7. Key RF measurement mechanics

- IQ playback: RX/UL tests inject a stimulus by ARB waveform playback on a Keysight generator (.bin/.wfm files named for the channel/FRC). Sequence: reset -> connect RFIO port -> set power/freq/sample-rate -> load+select waveform -> set trigger/delay -> enable ARB+modulation+RF. TDD uses an external trigger delay to align to the frame.

- Instrument control: pure SCPI over TCP to Keysight boxes (E6640A 'EXM', E6550A 'EXF', MXG/MXA/PXA, plus fading emulators). One EXM does double duty — generator for RX tests, analyzer for TX tests. Inventoried in env_setup.json with model, hostname, port, direction, role (REGULAR/INTERFERER).

- Cable / path-loss correction: per-(BS, TRX, frequency) loss tables (e.g. cable_loss_out-final_DL_UL.json) so reported power is at the RU connector. FHGW units add a per-band DSA; a 33 dB 'vvdn_factor' offset is added to 3GPP minimums for FHGW boxes.

- KPI thresholds: in kpi_thresholds_references/*.json, keyed by test name with 3gpp_reference, test_parameters (search steps, durations, bler thresholds), and kpi (base_dbm, tolerance, per-band expected values). Verdicts are 3-valued: Pass / Pass(Tolerance) / Fail.

# 8. The config system, concretely

A setup directory = one lab bench / one RU. Its setup_conf.ini is the index; each section names a JSON file (verbatim from RCT-5G-GR-FHGW-TRX4):

[cws] config_file = tdd_test_setup.json # BBU/DU node [te] config_file = env_setup.json # instruments [tc_cadence] config_file = tc_cadence.json # tests+groups+cadences [rrh] config_file = rrh_setup.json # the RU itself [cable_loss] config_file = cable_loss.json [kpi_thresholds] config_file = ../kpi_thresholds_references/FHGW_Box_kpi_thresholds.json

JSON files compose via {"$ref": "other.json#/pointer"}. The merged result is reached as test_config.get_config_value('a.b.c'). A Group + Cadence that schedule the ACLR test above:

"TS_DL_CI": { "IsDL":true,"band":"n78","frequency":["M"],"BW":["40"], "SCS":"30","tm":["1.1"],"ext_trig_delay":"0.00998675", "Tests":["test_Tx_BS_Output_Power","test_Tx_modulation_analysis","test_Tx_ACLR"] } "Cadence-IDs": { "P0": ["TS_DL_CI","TS_UL_CI"], "P1": ["TS_DL_Nightly", ...] }

# 9. Your reading list (in order)

Start with the docs — they are unusually thorough and quote the code:

| File | Why |
| --- | --- |
| docs/ARCHITECTURE.md | The 3-layer model + registries. The mental model for everything. |
| docs/CONFIGURATION.md | Merge hierarchy, setup.ini sections, $ref, FHGW flags. |
| docs/RUNNING_TESTS.md | How to actually launch a run (cadence/marker selection, PM collection). |
| docs/TEST_PATTERNS.md | pytest fixtures (test_args, test_params), device-connection pattern. |
| docs/3GPP_TS51021_7_3_STATIC_REF_SENSITIVITY.md | The gold-standard end-to-end test walkthrough. Best single 'how a test is formed' doc. |
| docs/5G_TX_CONFORMANCE_TESTS.md | Full 5G TX catalog + the instrument method names. |
| docs/PRACH_8_4_DUAL_MODE.md | The two PRACH search algorithms and their JSON knobs. |
| docs/NBIOT_COMBINED_TX.md | The most JSON-driven catalog — shows how generic the runner is. |
| docs/GLOSSARY.md | RF + framework acronyms. |
| setup_configuration/RCT-5G-GR-FHGW-TRX4/tc_cadence.json | A complete real Test-IDs/Group-IDs/Cadence-IDs example. |
| kpi_thresholds_references/FHGW_Box_kpi_thresholds.json | Real pass/fail limits with 3GPP refs + DSA/vvdn notes. |
| conformance_4g/fdd_test_setup.json | The cleanest legacy 4G example. |

How this ties to your RF training Module 1 (dB) -> every limit and the cable-loss correction in these tests. Module 2 (VSWR) -> why cable_loss.json and matched paths exist. Module 6 (DPD) -> the ACLR test is literally how you prove the DPD is working. Module 7 (ACLR/spectrum) -> the sa_set_ACP_mode / SEM measurements above. Module 8 (PRACH) -> test_Rx_PRACH; Module 9 (AAU) -> the FHGW/TRX4 multi-branch setups.