Table of Contents

# PW Test Plan — 4G LTE

# 1 Introduction

The RU under test will support some subset of the rows below. Test only the bands, BWs, and freq points that your contract specifies. This reference is shared across every PW 4G LTE conformance test in this set.

## Supported bands (E-UTRA FR1)

| Band | Duplex | DL range (MHz) | Common BWs (MHz) |
| --- | --- | --- | --- |
| B1 | FDD | 2110 – 2170 | 5, 10, 15, 20 |
| B3 | FDD | 1805 – 1880 | 1.4, 3, 5, 10, 15, 20 |
| B4 | FDD | 2110 – 2155 | 1.4, 3, 5, 10, 15, 20 |
| B7 | FDD | 2620 – 2690 | 5, 10, 15, 20 |
| B8 | FDD | 925 – 960 | 1.4, 3, 5, 10 |
| B20 | FDD | 791 – 821 | 5, 10, 15, 20 |
| B28 | FDD | 758 – 803 | 3, 5, 10, 15, 20 |
| B41 | TDD | 2496 – 2690 | 5, 10, 15, 20 |

## Frequency points per (band, BW)

Three test frequencies per (band, BW) are required by 3GPP TS 36.141 §4.9.1:

| Point | Formula | Meaning |
| --- | --- | --- |
| fb (bottom) | DL_min + BW/2 | lowest carrier that fits in the band |
| fm (middle) | (DL_min + DL_max) / 2 | mid-band carrier |
| ft (top) | DL_max − BW/2 | highest carrier that fits in the band |

## E-TM (E-UTRA Test Model) selection

3GPP TS 36.141 defines six E-UTRA Test Models. Each conformance test selects one (or more) of these. Pick the model required by your specific test:

| E-TM | PRB allocation | Purpose | testmac rctdl ID |
| --- | --- | --- | --- |
| 1.1 | Full BW, QPSK | Reference for max output power, OFF leakage | 6111 |
| 1.2 | Full BW, 16-QAM | Total power dynamic range | 6112 |
| 2 | Single PRB, 64-QAM | Min carrier output, freq accuracy | 6113 |
| 3.1 | Full BW, 64-QAM | ACLR (with 64-QAM) | 6114 |
| 3.2 | Full BW, 16-QAM | ACLR (with 16-QAM) | 6115 |
| 3.3 | Full BW, QPSK | ACLR (with QPSK) | 6116 |

## FHGW + BOB antenna mapping (per band group)

Every FHGW exposes 8 TX + 8 RX physical ports (Tx1…Tx8, Rx1…Rx8) through the Break-Out Board. The set of ports that actively transmit/receive depends on the band group — different products wire different subsets:

| Band group | Band | Active TX ports | Active RX ports | FHGW product (example) |
| --- | --- | --- | --- | --- |
| b0103 (B1+B3 FDD) | B1 | Tx4, Tx1, Tx8, Tx5 | Rx1, Rx2, Rx5, Rx6 | Integrity-ME-0-B1B3-00 |
| b0103 (B1+B3 FDD) | B3 | Tx3, Tx2, Tx7, Tx6 | Rx3, Rx4, Rx7, Rx8 | Integrity-ME-0-B1B3-00 |
| b0307 (B3+B7 FDD) | B3 | Tx6, Tx7 | Rx6, Rx7 | Integrity-ST-0-B3B7B78-00-0 |
| b0307 (B3+B7 FDD) | B7 | Tx5, Tx8 | Rx5, Rx8 | Integrity-ST-0-B3B7B78-00-0 |
| b0407 (B4+B7 FDD) | B4 | Tx6, Tx7 | Rx6, Rx7 | Integrity-MM-0-B4B7n78 |
| b0407 (B4+B7 FDD) | B7 | Tx5, Tx8 | Rx5, Rx8 | Integrity-MM-0-B4B7n78 |
| b082028 (B8 + B20/B28 FDD) | B8 | Tx1, Tx2, Tx3, Tx4 | Rx1, Rx2, Rx5, Rx6 | Integrity-ME-0-B1B3B8B41 |
| b082028 (B8 + B20/B28 FDD) | B20 | Tx5, Tx6, Tx7, Tx8 | Rx1, Rx2, Rx3, Rx4 | Integrity-ME-0-B1B3B8B41 |
| b082028 (B8 + B20/B28 FDD) | B28 | Tx5, Tx6, Tx7, Tx8 | Rx3, Rx4, Rx7, Rx8 | Integrity-ME-0-B1B3B8B41 |
| b41 (B41 TDD) | B41 | Tx5, Tx6, Tx7, Tx8 | Rx1, Rx2, Rx5, Rx6 | Integrity-ME-0-B1B3B8B28n41 |

Inactive chassis ports (not part of the active band group) stay idle and must be terminated with 50 Ω.

## Rated output power (P_rated)

P_rated (or P_rated,c per carrier) is the configured carrier Tx power that conformance tests compare against. PW's automation runs the FHGW characterisation flow (no RFFE attached), so P_rated is the expected conducted-RF output power at the FHGW Box SMA ports (BOB), per band group and band:

| Band group | Band | P_rated (dBm) |
| --- | --- | --- |
| b0103 | B1 | −16.7 |
| b0103 | B3 | −15.7 |
| b0307 | B3 | −23.4 |
| b0307 | B7 | −26.4 |
| b0407 | B4 | −24.5 |
| b0407 | B7 | −26.4 |
| b082028 | B8 | −18.0 |
| b082028 | B20 | −19.2 |
| b082028 | B28 | −27.3 |
| b41 | B41 | −23.0 |

## Common BW selections per band group (informational)

The manual procedure supports any BW listed in Supported bands for the chosen band. The table below shows PW's typical operating point per band group, when no specific BW is requested — informational only:

| Band group | Typical BW (MHz) |
| --- | --- |
| b0103 (B1+B3 FDD) | 10 |
| b0307 (B3+B7 FDD) | 20 |
| b0407 (B4+B7 FDD) | 20 |
| b41 (B41 TDD) | 10 |
| b082028 (B8 + B20/B28 FDD) | 10 |

To run a different BW, edit the FHGW's oru_startup_slave.xml (<channel-bandwidth> + <center-of-channel-bandwidth> fields) and re-push it before §5.2 — see the fb / ft caveat in §5.3 for the same XML-edit pattern.

## 3GPP Specification archives

Every test in this set cites clauses from the same 3GPP specs. Download URLs (all open access):

| Spec | Title | Archive |
| --- | --- | --- |
| TS 36.141 | E-UTRA Base Station (BS) conformance testing | https://www.3gpp.org/ftp/Specs/archive/36_series/36.141/ |
| TS 36.104 | E-UTRA Base Station (BS) radio transmission and reception | https://www.3gpp.org/ftp/Specs/archive/36_series/36.104/ |
| 3GPP portal (all series, all releases) | — | https://www.3gpp.org/specifications-technologies |

Each archive folder lists every release of the spec as a downloadable ZIP/PDF. Pick the release that matches the conformance plan your test is being run against (e.g. Rel-17 → 36141-h{nn}0.zip).

# 2 Management and Control

## 2.1 Update FHGW EEPROM model name

Run this only when the personality-verify step (§5.1 step 11 in any per-test procedure below) reports a flavour that doesn't match the band group under test — i.e. the chassis is booted in the wrong combo. The unified FHGW image already carries every flavour's configs, so this procedure just rewrites the EEPROM model name and reboots; no image flash needed.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| E.1 | SSH into the DU | DU console | ssh root@<DU_IP> | DU shell prompt |
| E.2 | Find the FHGW IP under the fronthaul interface | DU shell | arp -n | FHGW IP appears under ens1f3 (or your fronthaul iface) |
| E.3 | SSH into the FHGW. Default password is root. | DU shell | ssh root@<FHGW_IP> | FHGW shell prompt. Banner shows the currently-loaded model name (e.g. root@Integrity-ME-0-B1B3-00-2:~#) |
| E.4 | Confirm the unified image carries the target flavour (optional) | FHGW shell | ls /etc/configs/*-startup.xml | Listing includes the target Integrity-*-startup.xml |
| E.5 | Open the EEPROM flash-scripts folder and list available flavours | FHGW shell | cd /run/media/mmcblk0p4/ls flash_eeprom_*.sh | One flash_eeprom_<FLAVOUR>.sh per supported flavour |
| E.6 | Edit the script for the target flavour. Update the model-name string to the target Integrity-* name. | FHGW shell → editor | vi flash_eeprom_<FLAVOUR>.sh | File saved |
| E.7 | Write EEPROM | FHGW shell | ./flash_eeprom_<FLAVOUR>.sh | Script prints OK / done; no I²C errors |
| E.8 | Reboot the FHGW | FHGW shell | reboot | FHGW returns in ~60 s. SSH back in — banner shows the new model name. |
| E.9 | Verify the new personality | FHGW shell | personality-app --flavour | Output equals the band group you wanted. Resume §5.1 from step 11. |

Constraint: the FHGW chassis must already be hardware-capable of the target flavour. Different platform classes host different flavour sets:

- ME class (8 active DACs): hosts b0103, b082028

- MM class (4 active DACs, rfdc_ids 4-7): hosts b0307, b0407, n78, n78l

- B41 class (4 active DACs, tile 0): hosts b41

Rewriting EEPROM to a flavour outside the chassis's hardware class boots into a personality that won't match the silicon. Verify with the lab before crossing classes.

## 2.2 Install new FW

The FHGW firmware ships as a unified image tarball; the on-device upgrade script applies it without disturbing the EEPROM model name (so the band-group personality survives the upgrade). The script is currently named upgrade.sh; a rename to ru-upgrade.sh is landing with the next ru-app release — use whichever name your radio carries.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| F.1 | SCP the FW tarball to the FHGW. Route via the DU's fronthaul iface. | DU console | scp <FW>.tar.gz root@<FHGW_IP>:/tmp/ | 100% ... done — file transferred |
| F.2 | SSH into the FHGW. Default password is root. | DU shell | ssh root@<FHGW_IP> | FHGW shell prompt |
| F.3 | Run the upgrade script. Newer ru-app radios ship with ru-upgrade.sh; older ones with upgrade.sh — use whichever exists on your radio. | FHGW shell | cd /tmp./ru-upgrade.sh <FW>.tar.gz | Progress streams; Upgrade complete (exit 0) |
| F.4 | Reboot the FHGW | FHGW shell | reboot | SSH drops; chassis comes back in ~60–120 s |
| F.5 | Verify the running version | FHGW shell | cat /etc/fhgw_version | Version string matches the tarball you installed (e.g. 4.0.4-build123) |
| F.6 | Re-verify the personality after reboot | FHGW shell | personality-app --flavour | Same band-group personality as before the upgrade (e.g. b0103) |

The upgrade preserves the EEPROM — the chassis comes back in the same band-group personality it had before. To change the personality, follow §2.1 Update FHGW EEPROM model name — that's a separate procedure.

# 3 Downlink Test Suite

All downlink tests run on the same conducted-RF bench — equipment, connectivity, and operating rules below. Each test's body covers what's specific to that test.

Table 3-0 — Test Equipment

| Test Equipment | Notes |
| --- | --- |
| DU | Linux server running L1 / L2 / testmac |
| FHGW Box | Unit under test — FHGW compute board (Xilinx ZU67 RFSoC, 8 TX + 8 RX ports) + BOB break-out board, no RFFE attached. Low-power RF output. |
| PTP Grand Master | Microchip Falcon-RX/812/G/A — LLS-C1 (feeds DU only) |
| Keysight EXM | E6640A Wireless Test Set (LTE-FDD / LTE-TDD personality, SA mode) |
| NUC | Test controller PC |
| Splitter | 1 input × 2 outputs (×4 for 4-antenna bands) |
| RF Cable | SMA-to-SMA, 50 Ω |

Figure 3-0 — Conducted-RF test bench (LTE Tx)

Operating rules

- One TX port at a time. Other active TX ports terminated with 50 Ω. Inactive chassis ports (not part of this band group) stay idle.

- Each port has its own calibrated cable-loss value. Measure all of them before starting — wrong cable loss is the #1 cause of mismatched per-port readings.

- No need to cross-lock the EXM to the Falcon-RX domain. The EXM runs on its own internal frequency reference.

Per-test operating notes

- 3.2 Total Power Dynamic Range uses the same per-antenna cable loss as 3.1 (measure once, reuse for both E-TM 1.2 and E-TM 2 sweeps). Switch E-TMs cleanly: issue the second rctdl after the first burst has fully settled (≥10 s).

## §5 Procedure — common parts (LTE Downlink)

§5.1, §5.2, and §5.3 below are the shared §5 Procedure steps for every downlink test in this chapter. The "Where to run" column refers to the endpoints listed in How to send commands below. Run them as part of each test's §5 — the test's flowchart calls each section out in order (§5.1 → §5.2 → §5.2.5 → §5.3 → §5.4). The test-specific §5.2.5 and §5.4 live in each test's body.

### How to send commands

The "Where to run" column in every procedure table refers to one of these command endpoints. The same Keysight EXM (E6640A) acts as both SA (Tx tests) and SG (Rx tests) — same physical box, software-toggled mode.

| Where the command runs | How to send it |
| --- | --- |
| SA / SG (Keysight EXM E6640A) | Connect EXM to your PC (LAN / USB / GPIB) → Keysight IO Libraries Suite → Connection Expert → right-click EXM → Interactive IO. Paste the SCPI line. Commands ending in ? are Send & Read; everything else is Send Command. Mode (SA vs SG) toggled via :INST:SEL …. R&S / Anritsu use different mnemonics — adapt. |
| PC | Your test-controller PC. Used for the initial ssh to reach the DU. |
| DU shell | SSH into the DU's Linux host. ./l1.sh / ./l2.sh are foreground — open separate SSH sessions (DU shell #1, #2, #3) for parallel processes. |
| testmac console | After ./l2.sh starts, type commands at the testmac> prompt in that shell. |
| FHGW shell | SSH from the DU into the FHGW (ssh root@<FHGW_IP>). Used for personality-app, file edits, launching netopeer2-cli, and launching the LRCP ./Server. Typically two FHGW SSH sessions are needed in parallel (one for netopeer2-cli + edits, one for the LRCP lrcp> prompt). |
| netopeer2-cli session | Set up once in §5.1 step 12 — an interactive NETCONF client. Subsequent > edit-config commands target the same persistent session. |
| LRCP prompt | Set up once in §5.1 step 15 (Tx tests only) — the FHGW's LRCP server CLI (./Server in /opt/LRCP). Accepts r <filename> at the lrcp> prompt to reload a config file. |
| RF switch (HTTP) | Mini-Circuits SP6T responds to HTTP GET requests on the switch IP. curl or any HTTP client. |

### 5.1 Bench bring-up (do once at session start)

These steps configure the bench. Do them once when you start your session — they don't change with band, BW, freq, or antenna.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | SSH into the DU | PC | ssh root@<DU_IP> | DU shell prompt |
| 2 | Find the FHGW IP under the fronthaul interface | DU shell | arp -n | FHGW IP appears under ens1f3 (your fronthaul iface) |
| 3 | Reset SA and select the LTE personality. FDD bands (B1/B3/B4/B7/B8/B20/B28) use LTEAFDD; TDD band B41 uses LTEATDD. | SA | *RST*OPC?INST:SEL LTEAFDD | *OPC? returns 1; title bar shows LTE-FDD (or LTE-TDD) |
| 4 | Configure Channel Power mode with averaging | SA | :CONF:CHPower:INIT:CONT ON:CHPower:AVERage:STATe ON:CHPower:AVERage:COUNt 20:CHPower:AVERage:TYPE RMS | Continuous sweep; avg counter visible |
| 5 | Sanity-check the RF switch is reachable | RF switch (HTTP) | GET http://<SWITCH_IP>/MN? | MN=RC-2SP6T-26 (or your switch's model string) |
| 6 | Calibrate cable loss per (TX antenna, freq point). Skip steps 6–10 if cable_loss.json is already populated for this bench. Disconnect FHGW from the splitter input; connect a Signal Generator in its place. SA stays on the switch output. | bench (physical) | — | SG drives the chain the FHGW normally drives |
| 7 | Output a CW tone at a known reference level (e.g. −10 dBm) at the freq point you're calibrating. For B3 / fm: 1842.5 MHz. | SG | :FREQ <FREQ_HZ>:POW -10:OUTP ON | RF-ON LED lit on SG; tone visible on SA |
| 8 | Route the RF switch to ant_i, then read channel power. Substitute <BANK> and <PORT> from your setup config. | RF switch then SA | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT>:READ:CHPower:CHPower? | Numeric dBm value P_meas_cal (e.g. −15.3) |
| 9 | Compute cable loss = SG_ref − P_meas_cal. Record in cable_loss.json keyed by (antenna_port, freq_hz). Store as a positive dB value; the SA gets the negated value in §5.4 step 2. | spreadsheet / notes | e.g. −10 − (−15.3) = 5.3 dB | One entry in the loss table |
| 10 | Repeat steps 7–9 for every (antenna × freq point) in your test plan. When done: turn SG off, disconnect SG, reconnect FHGW to splitter input. | bench | :OUTP OFF | Full per-(port, freq) loss table; FHGW back online |
| 11 | SSH into the FHGW from the DU, cd into the netopeer working directory, and verify the personality matches your band group. If the personality is wrong, fix it before the next steps — they push band-group-specific config. | FHGW shell | ssh root@<FHGW_IP>cd /root/ORAN-CLI/netopeer2personality-app --flavour | Personality equals the band group under test (e.g. b0103, b0307, b0407, b082028, b41). If anything else: STOP — rewrite the EEPROM model name and reboot (see Update FHGW EEPROM model name) or use a different bench. |
| 12 | Launch netopeer2-cli, connect to the FHGW's NETCONF server (loopback), and push the radio mode config (LTE) | FHGW shell → netopeer2-cli | netopeer2-cli> connect --host localhost --login root> edit-config --target running --config=ConfigID_LTE.xml --rpc 40 | netopeer2-cli > prompt opens; NETCONF OK; no alarm |
| 13 | Push FHGW PTP profile (G.8275.1 telecom) | netopeer2-cli session (from step 12) | > edit-config --target running --config=set-ru-ptp-config-G8275-1.xml --rpc 40 | NETCONF OK; FHGW PTP syncs to grandmaster |
| 14 | Push processing-elements (eCPRI mapping) | netopeer2-cli session | > edit-config --target running --config=set-processing-elements.xml --rpc 40 | NETCONF OK |
| 15 | Open a separate SSH session to the FHGW and launch the LRCP server. Used by §5.2 step 3b to reload Tx power per band combo. Leave the lrcp> prompt open. | FHGW shell #2 (new SSH session) | ssh root@<FHGW_IP>cd /opt/LRCP./Server | lrcp> prompt appears |
| 16 | Start L1 (PHY) stack. L1 stays in foreground — leave this shell open. | DU shell #1 | ./l1.sh -rct | welcome to application console banner within ~60 s |
| 17 | Start L2 (testmac) stack | DU shell #3 (new SSH session) | ./l2.sh | testmac> prompt appears |

✓ Bench is now armed. → Continue to §5.2 for the first band in your test plan.

### 5.2 Per-band-combo setup (repeat for each band combo)

Do these steps once per band combo — a (band, band-group) pair, e.g. B3 in band-group b0103. The band-group is the booted FHGW personality (fixed per session — to change it, see Update FHGW EEPROM model name). Pushing this combo's ACTIVE_*.xml is a full snapshot: it activates this band's carriers AND deactivates the other band's carriers in the same group. BW + freq config lives in §5.2.5 (next).

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Activate the A0/A1 carriers for your combo. Look up your <A0A1_FILE> in the ACTIVE_*.xml file lookup below. | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<A0A1_FILE> | NETCONF OK |
| 2 | Activate the A2/A3 carriers. 4-antenna combos only (b0103, b082028, b41) — skip when the lookup row below shows a single file (b0307, b0407). Look up your <A2A3_FILE> in the same lookup row. | netopeer2-cli session | > edit-config --target running --config=<A2A3_FILE> | NETCONF OK (merges with step 1 — does not replace it) |
| 3a | Edit the FHGW Tx power config. Substitute <P_RATED_INT> = int(<P_RATED>) — drop the decimal of the <P_RATED> value from the umbrella's Rated output power table (e.g. −15.7 → −15). The automation rounds the same way. Filename: lrcp_rru_cfg_4ant.js for 4-antenna combos (b0103, b082028, b41); lrcp_rru_cfg_2ant.js for 2-antenna combos (b0307, b0407). | FHGW shell → editor | cd /opt/LRCPvi lrcp_rru_cfg_<N>ant.jsset MaxAntOutputPower1 = <P_RATED_INT>set MaxAntOutputPower2 = <P_RATED_INT>set DlAtt = <P_RATED_INT × 100> (centi-dB) | File saved |
| 3b | Reload the LRCP config | LRCP prompt (from §5.1 step 15) | r lrcp_rru_cfg_<N>ant.js | LRCP confirms OK 4×; radio_broadcast_ready flag set |

ACTIVE_*.xml file lookup (used by steps 1 and 2 above):

| Band combo | Files to push |
| --- | --- |
| B1 (b0103) | configs/b0103/ACTIVE_B01C00A0A1.xml + configs/b0103/ACTIVE_B01C00A2A3.xml |
| B3 (b0103) | configs/b0103/ACTIVE_B03C00A0A1.xml + configs/b0103/ACTIVE_B03C00A2A3.xml |
| B3 (b0307) | configs/b0307/ACTIVE_B03C00A0A1.xml (2-antenna combo — single file) |
| B7 (b0307) | configs/b0307/ACTIVE_B07C00A0A1.xml (2-antenna combo — single file) |
| B4 (b0407) | configs/b0407/ACTIVE_B04C00A0A1.xml (2-antenna combo — single file) |
| B7 (b0407) | configs/b0407/ACTIVE_B07C00A0A1.xml (2-antenna combo — single file) |
| B8 (b082028) | configs/b082028/ACTIVE_B08C00A0A1.xml + configs/b082028/ACTIVE_B08C00A2A3.xml |
| B20 (b082028) | configs/b082028/ACTIVE_B20C00A0A1.xml + configs/b082028/ACTIVE_B20C00A2A3.xml |
| B28 (b082028) | configs/b082028/ACTIVE_B28C00A0A1.xml + configs/b082028/ACTIVE_B28C00A2A3.xml |
| B41 (b41) | configs/b41/ACTIVE_B41C00A0A1.xml + configs/b41/ACTIVE_B41C00A2A3.xml |

Two-file rows (4-antenna combos) push the A0/A1 file first, then the A2/A3 file — each edit-config merges with the previous (the second is not a replacement). Single-file rows (2-antenna combos: b0307, b0407) push the one file only. The XML is a full snapshot — pushing a combo's file(s) activates this band's carriers AND deactivates the other band's carriers in the same group automatically.

✓ Carriers active on this band; Tx power set. → Continue to §5.2.5 to pick a BW.

### 5.3 Per-freq-point setup (repeat for each fb / fm / ft)

Do these steps once per freq point within the current BW. Note: fb / fm / ft are re-derived from the current BW per the umbrella's Frequency points formulas — e.g. for B3 at 10 MHz fb = 1810 MHz; at 20 MHz fb = 1815 MHz. The E-TM stream from §5.2.5 step 4 keeps running — don't refire rctdl between freqs.

Note — fb / ft caveat. §5.3 only retunes the SA. The FHGW's EARFCN is pinned by the §5.2.5 step 1 startup XML you pushed (typically fm). For a true fb / ft measurement, re-push the startup XML with the target <center-of-channel-bandwidth> (i.e. re-do §5.2.5 step 1 with the new EARFCN). Alternatively, treat each (BW, freq) pair as its own pass through §5.2.5.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Set the SA carrier reference frequency for this freq point. Pick <FREQ_HZ> from the umbrella's Frequency points worked example for your band — Hz, not MHz. | SA | :CCARrier:REFerence <FREQ_HZ> (e.g. for B3 / fm: :CCARrier:REFerence 1842500000) | Center-freq readout updates; carrier centered in trace |
| 2 | Wait ~15 s for PHY to settle | — | — | SA reading stabilises within ±0.2 dB |

✓ Carrier centered. → Continue to §5.4 for the first antenna. Auto-level happens per antenna in §5.4 step 3 because each antenna's cable loss is different.

## 3.1 Tx — BS Output Power

3GPP references: TS 36.141 §6.2 (conformance) + TS 36.104 §6.2 (radio requirement) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, E-TMs) live in the PW Conformance — Common Test Parameters Reference above. This test uses E-TM 1.1.

### 1. Purpose

Verify the FHGW's per-antenna transmit output power matches P_rated for the configured band group. Conducted-RF, low-power test — output flows through a passive cable chain into the EXM. No chamber, no OTA.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / E-TM tables.

Table 3-1 — BS Output Power Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared TX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Frequency | fb / fm / ft per (band, BW) — see Frequency points in the Reference | MHz |
| Power | P_rated — per band group, see Rated output power in the Reference | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference for all valid options (Common BW selections per band group is informational only) | MHz |
| E-TM | 1.1 (rctdl 6111) | — |

### 4. Pass/Fail criteria (PW automation gate)

Each antenna passes iff |P_meas − P_rated| ≤ Δ, where Δ = ±1.5 dB (PW's internal gate, tighter than the 3GPP TS 36.141 §6.2.3.1 conformance limit of ±2.7 dB / ±3.0 dB).

Worked example — B3, P_rated = −15.7 dBm:

- P_meas = −15.3 dBm → |diff| = 0.4 → PASS

- P_meas = −17.5 dBm → |diff| = 1.8 → FAIL

Test verdict: PASS only if every antenna passes.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SA init,     │
  │    personality verify, RF switch ping,   │
  │    cable-loss cal per (port, freq)       │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  │    • set FHGW Tx power                   │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │      (channel-bandwidth + EARFCN)        │
  │    • set SA BW preset                    │
  │    • (TDD only) DL-slot gating           │
  │    • fire rctdl 6111 <BW>                │
  │      (E-TM stream stays running across   │
  │       all freqs at this BW)              │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SA carrier reference freq       │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna    ◄─── next antenna │  ← this test
  │    • route RF switch                     │
  │    • apply cable loss                    │
  │    • auto-level SA                       │
  │    • read channel power                  │
  │    • verdict                             │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     │  (rctdl keeps running)          │
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     │  (e.g. 5 → 10 → 20 MHz at this band)     │           push new startup XML,
     │  Same activated band — no ACTIVE_*.xml.  │           re-set SA BW preset, refire rctdl
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group to test?   │── yes ──► back to §5.2
     │  (e.g. did B1 in b0103, now want B3)     │           push different ACTIVE_*.xml,
     │  Same booted personality — no reboot.    │           re-set Tx power
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     │  (e.g. did all of b0103, now want b0307) │           (see "Update FHGW EEPROM
     │  Different personality — REBOOT needed.  │           model name" appendix),
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every antenna
                 at every (band, freq) passes)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. Each BW change pushes a fresh FHGW startup XML carrying <channel-bandwidth> and <center-of-channel-bandwidth> (the EARFCN that defines fm), then re-arms the SA and fires rctdl.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm in Hz (from the umbrella's Frequency points formula). | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Set the SA BW preset to match your BW. Substitute the BW token: B1M4 / B3M / B5M / B10M / B15M / B20M. | SA | :RAD:STAN:PRES B10M | Span auto-adjusts |
| 3 | Enable DL-slot gating. TDD bands only (B41) — skip on FDD. Values are for TDD config 3, special-subframe config 8; for other configs derive gate delay/length from TS 36.211 §4. | SA | :RAD:STAN:PRES:CONF 3:RAD:STAN:PRES:SSFR 8:TRIG:SOUR:GATE:OUTP:PAT:GATE:STAT ON:GATE:VIEW:STAT ON:GATE:VIEW:SWE 20 ms:GATE:DEL 3.1 ms:GATE:LEN 2.8 ms | Gated sweep; only DL window integrated |
| 4 | Fire E-TM 1.1 at the configured BW. Substitute the BW in MHz at the end (1.4 / 3 / 5 / 10 / 15 / 20) — must match step 1a's <channel-bandwidth> and step 2's preset. | testmac console | rctdl 6111 10 | Carrier appears on SA; channel-power stable at ~P_rated. The E-TM stream stays running for all freq points below — do not refire between freqs. |

✓ FHGW transmitting E-TM 1.1 at this BW. → Continue to §5.3 for the first freq point.

#### 5.4 Per-antenna measurement (repeat for each antenna)

Do these steps once per antenna in this band's active TX set. The carrier is already up (§5.2.5 step 4) and the SA is centered on this freq point (§5.3). All that's left is routing the right antenna to the SA and reading the channel power.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RF switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 (HTTP body) |
| 2 | Apply ant_i's calibrated cable-loss offset to the SA. The loss value comes from §5.1 steps 6–10 (or cable_loss.json) for the current freq. Enter as NEGATIVE dB (a 5 dB cable loss → -5). | SA | :CORR:BTS:GAIN <-CABLE_LOSS_DB> (e.g. for 5 dB loss: :CORR:BTS:GAIN -5) | SA trace shows carrier at ~P_rated (cable-loss compensated) |
| 3 | Auto-level the SA for this antenna. Must be after step 2 — each antenna's cable loss is different, so the ref-level must be re-set per antenna. | SA | :SENSe:POWer:RF:RANGe:OPTimize IMMediate | Trace fills upper third of screen; no IF/RF overload indicator |
| 4 | Verify signal sync (visual check on SA front panel) | SA panel | — | Sync OK green; no red Frame Sync Lost / Burst Search Failed / Demod Error. If any fail → re-verify §5.2. |
| 5 | Trigger a single averaged sweep | SA | :INIT:IMM*WAI | "Single sweep complete" within ~1 s |
| 6 | Read integrated channel power | SA | :READ:CHPower:CHPower? | Numeric dBm value = P_meas_i (cable-loss corrected per step 2) |
| 7 | Compare against the pass window from §4 | spreadsheet / notes | P_rated − Δ ≤ P_meas_i ≤ P_rated + Δ where Δ = ±1.5 dB | P_meas_i inside window → ant_i PASS. Example: P_rated = −15.7, P_meas_i = −15.3 → window [−17.2, −14.2] → inside → PASS. |

Note — step 2 sign convention: :CORR:BTS:GAIN is an external-gain correction. A 5 dB cable loss is entered as -5 (loss = negative gain). The SA then adds +5 dB to its raw readings to refer the result back to the FHGW port.

#### Loop control (when do I go where?)

After step 7, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq) | → back to §5.4 step 1 with the next ant_i |
| All antennas done for this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point (the E-TM stream from §5.2.5 step 4 is still running — don't refire) |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW (e.g. 10 → 20 MHz at B3). Push a new startup XML, re-set SA BW preset, refire E-TM. No ACTIVE_*.xml change — same activated band. |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band (e.g. B1 → B3 in b0103). Push a different ACTIVE_*.xml, re-set Tx power. No reboot — same booted personality. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 for the first band in the new group. |
| All band-groups done | STOP. Overall verdict for the test = PASS iff every antenna at every (band, BW, freq) passes. |

Loose-coupling check (after one full antenna sweep): briefly disconnect the SA from one antenna and reconnect to another — both should show comparable channel power (within ±0.5 dB of each other). Confirms no vendor per-antenna calibration is off.

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| Reading is ~10 dB low | Cable loss not subtracted, or wrong attenuator | Re-verify cable/attenuator calibration with a known source |
| Reading is ~3 dB low on TDD | DL duty cycle not accounted for | Use a TDD-aware channel-power measurement (INST:SEL LTEATDD) or gate on DL slots only |
| Reading drifts > 0.5 dB over a minute | RU thermal back-off or insufficient averaging | Let the RU warm up 15+ min; increase SA average count |
| One antenna reads radically different | Vendor's per-antenna calibration off, or cable swap | Swap cable + attenuator between two antennas to isolate the variable |
| IF-overload indicator on SA | Reference level too low or attenuator too small | Raise SA ref level by 5 dB and retry; if still overloading, add more external attenuation |
| Channel power flickers ±2 dB | E-TM source not stable, or RU not actually forced to max | Confirm RU is in test/max-power mode, not normal traffic |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §6.2 BS Output Power, §6.2.3.1 Test requirements, §4.9.1 Test frequencies (fb / fm / ft), Annex C E-UTRA Test Models (E-TM 1.1).

- TS 36.104 §6.2 BS output power (radio requirement — the ±2.0 dB number that 36.141's ±2.7 dB is built on).

## 3.2 Tx — Total Power Dynamic Range

3GPP references: TS 36.141 §6.3.2 (conformance) + TS 36.104 §6.3.2 (radio requirement) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, E-TMs, antenna mapping, P_rated) live in the Common Test Parameters Reference above. This test measures the delta between E-TM 1.2 (full BW, 16-QAM) and E-TM 2 (single PRB, 64-QAM).

### 1. Purpose

Verify the FHGW Box's transmit dynamic range for Bx at the test frequency. The test runs two back-to-back measurements on the same antenna — first with E-TM 1.2 (full BW upper-bound burst), then with E-TM 2 (single PRB lower-bound burst). The dynamic range is the delta, which must meet a BW-dependent minimum.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / E-TM / antenna / P_rated tables.

Table 3-1 — Total Power Dynamic Range Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared TX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Frequency | fb / fm / ft per (band, BW) — see Frequency points in the Reference | MHz |
| Power | P_rated — per band group, see Rated output power in the Reference | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference for all valid options (Common BW selections per band group is informational only) | MHz |
| E-TM (upper-bound burst) | 1.2 (rctdl 6112) — full BW, 16-QAM | — |
| E-TM (lower-bound burst) | 2 (rctdl 6113) — single PRB, 64-QAM | — |

### 4. Pass/Fail criteria (3GPP)

Per TS 36.141 §6.3.2, each antenna's dynamic range (P_meas(E-TM 1.2) − P_meas(E-TM 2)) must meet a BW-dependent minimum:

| Channel BW (MHz) | Minimum dynamic range (dB) |
| --- | --- |
| 5 | 13.9 |
| 10 | 16.9 |
| 15 | 18.7 |
| 20 | 20.0 |

Per-antenna pass: dynamic_range_i ≥ min_limit(BW) − 0.4 dB. (PW applies a 0.4 dB runtime tolerance to absorb measurement noise — see FHGW_Box_kpi_thresholds.json → test_Tx_dynamic_range.limits.)

Test verdict: PASS only if every antenna passes.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test — the inner loop fires E-TM 1.2 + E-TM 2 per antenna (rctdl fires per antenna, not per BW).

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SA init,     │
  │    personality verify, RF switch ping,   │
  │    cable-loss cal per (port, freq)       │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  │    • set FHGW Tx power                   │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │      (channel-bandwidth + EARFCN)        │
  │    • set SA BW preset                    │
  │    • (TDD only) DL-slot gating           │
  │    (rctdl fires per antenna in §5.4)     │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SA carrier reference freq       │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna    ◄─── next antenna │  ← this test
  │    • route RF switch                     │
  │    • apply cable loss                    │
  │    • auto-level SA                       │
  │    • rctdl 6112 (E-TM 1.2) → P_max       │
  │    • rctdl 6113 (E-TM 2)   → P_min       │
  │    • dyn_range = P_max − P_min           │
  │    • verdict (≥ min_limit?)              │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     │  (e.g. 5 → 10 → 20 MHz at this band)     │           push new startup XML,
     │  Same activated band — no ACTIVE_*.xml.  │           re-set SA BW preset
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group to test?   │── yes ──► back to §5.2
     │  (e.g. did B1 in b0103, now want B3)     │           push different ACTIVE_*.xml,
     │  Same booted personality — no reboot.    │           re-set Tx power
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     │  (e.g. did all of b0103, now want b0307) │           (see "Update FHGW EEPROM
     │  Different personality — REBOOT needed.  │           model name" appendix),
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every antenna
                 at every (band, freq) meets
                 the dynamic-range minimum)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. Each BW change pushes a fresh FHGW startup XML carrying <channel-bandwidth> and <center-of-channel-bandwidth> (the EARFCN that defines fm), then re-arms the SA. Unlike BS Output Power, rctdl does not fire here — it fires per antenna in §5.4 because Total Power Dynamic Range needs both E-TM 1.2 and E-TM 2 bursts on the same antenna.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm in Hz (from the umbrella's Frequency points formula). | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Set the SA BW preset to match your BW. Substitute the BW token: B1M4 / B3M / B5M / B10M / B15M / B20M. | SA | :RAD:STAN:PRES B10M | Span auto-adjusts |
| 3 | Enable DL-slot gating. TDD bands only (B41) — skip on FDD. Values are for TDD config 3, special-subframe config 8; for other configs derive gate delay/length from TS 36.211 §4. | SA | :RAD:STAN:PRES:CONF 3:RAD:STAN:PRES:SSFR 8:TRIG:SOUR:GATE:OUTP:PAT:GATE:STAT ON:GATE:VIEW:STAT ON:GATE:VIEW:SWE 20 ms:GATE:DEL 3.1 ms:GATE:LEN 2.8 ms | Gated sweep; only DL window integrated |

✓ FHGW ready at this BW; SA configured. → Continue to §5.3 for the first freq point.

#### 5.4 Per-antenna measurement (repeat for each antenna — two E-TM bursts per antenna)

Do these steps once per antenna in the band's active TX set. Each antenna gets two measurements: E-TM 1.2 for P_max, then E-TM 2 for P_min. Dynamic range = P_max − P_min.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RF switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 |
| 2 | Apply ant_i's calibrated cable-loss offset to the SA. Value from §5.1 cable-loss cal (steps 6–10). Enter as NEGATIVE dB (a 5 dB cable loss → -5). | SA | :CORR:BTS:GAIN <-CABLE_LOSS_DB> (e.g. :CORR:BTS:GAIN -5) | SA trace shows carrier at ~P_rated (cable-loss compensated) |
| 3 | Auto-level the SA for this antenna. Must be after step 2 — each antenna's cable loss is different. Fire step 4 first if the SA RF is dark. | SA | :SENSe:POWer:RF:RANGe:OPTimize IMMediate | Trace fills upper third; no IF/RF overload |
| 4 | Burst 1 — E-TM 1.2 (upper bound). Substitute the BW in MHz at the end (1.4 / 3 / 5 / 10 / 15 / 20). | testmac console | rctdl 6112 10 | Carrier appears on SA at ~P_rated |
| 5 | Trigger a single averaged sweep and read | SA | :INIT:IMM*WAI:READ:CHPower:CHPower? | Numeric dBm = P_max_i |
| 6 | Burst 2 — E-TM 2 (lower bound). Same BW substitution as step 4. Wait ~10 s after firing. | testmac console | rctdl 6113 10 | Carrier still on SA at a much lower level |
| 7 | Trigger a single averaged sweep and read | SA | :INIT:IMM*WAI:READ:CHPower:CHPower? | Numeric dBm = P_min_i |
| 8 | Compute the dynamic range and verdict for ant_i | spreadsheet / notes | dynamic_range_i = P_max_i − P_min_icheck dynamic_range_i ≥ min_limit(BW) − 0.4 dB (see §4) | If ≥ → ant_i PASS |

Note — step 2 sign convention: :CORR:BTS:GAIN is an external-gain correction. A 5 dB cable loss is entered as -5 (loss = negative gain). The SA then adds +5 dB to its raw readings to refer the result back to the FHGW port.

#### Loop control (when do I go where?)

After step 8, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq) | → back to §5.4 step 1 with the next ant_i |
| All antennas done for this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW (e.g. 10 → 20 MHz at B3). Push a new startup XML, re-set SA BW preset. No ACTIVE_*.xml change — same activated band. |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band (e.g. B1 → B3 in b0103). Push a different ACTIVE_*.xml, re-set Tx power. No reboot — same booted personality. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 for the first band in the new group. |
| All band-groups done | STOP. Overall verdict = PASS iff every antenna at every (band, BW, freq) meets the dynamic-range minimum. |

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| Dynamic range ~3 dB short on TDD (B41) | UL/DL gating not accounted for in one of the bursts | Re-verify the SA personality is LTEATDD for B41; consider DL-slot gating |
| P_max_i ≈ P_min_i (no dynamic range) | The second rctdl didn't actually change the burst pattern | Wait longer between bursts (≥15 s); verify with rctdl status between the two rctdl commands |
| P_min_i reads radically different across antennas | Single-PRB allocation may be on a different sub-band for each antenna | Confirm E-TM 2 PRB position is consistent across the sweep |
| Both bursts hit the same dBm value | Auto-level may have changed reference level between bursts | Lock the SA ref level after the first measurement (§5.4 step 2) to avoid SA-side rescaling |
| Reading flickers > 0.5 dB | Insufficient averaging | Increase :CHPower:AVERage:COUNt to ≥ 50 |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §6.3.2 Total Power Dynamic Range (conformance), Annex C E-UTRA Test Models (E-TM 1.2, E-TM 2).

- TS 36.104 §6.3.2 (radio requirement — the dynamic-range minimums table).

- PW automation thresholds: configuration/setup_configuration/kpi_thresholds_references/FHGW_Box_kpi_thresholds.json → test_Tx_dynamic_range.limits

## 3.3 Tx — ACLR

3GPP references: TS 36.141 §6.5.4 (Adjacent Channel Leakage Ratio — conformance) + TS 36.104 §6.6.2 (radio requirement) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, E-TMs) live in the PW Conformance — Common Test Parameters Reference above. This test uses E-TM 1.1 in PW automation — <TBD: confirm whether PW automation uses ETM 1.1 or 3.x for ACLR; 3GPP §6.5.4 actually requires E-TM 3.1 / 3.2 / 3.3 (64-QAM, 16-QAM, QPSK)>.

### 1. Purpose

Verify the FHGW's adjacent channel leakage ratio (ACLR) at each declared TX antenna meets the per-carrier limit. Conducted-RF test: the FHGW transmits a single LTE carrier and the SA integrates power in the on-channel window and in the adjacent (±1 BW) and alternate (±2 BW) channel windows, reporting the ratio in dBc.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / E-TM tables.

Table 3-1 — ACLR Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared TX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Frequency | fb / fm / ft per (band, BW) — see Frequency points in the Reference | MHz |
| Power | P_rated — per band group, see Rated output power in the Reference | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference for all valid options (Common BW selections per band group is informational only) | MHz |
| E-TM | 1.1 (rctdl 6111) per PW automation — <TBD: confirm whether PW automation uses ETM 1.1 or 3.x for ACLR; 3GPP §6.5.4 requires E-TM 3.1 / 3.2 / 3.3> | — |
| Adjacent channel offsets | ±1 × BW (adjacent), ±2 × BW (alternate) — total 4 measured channels | MHz |

### 4. Pass/Fail criteria (PW automation gate)

Each antenna passes iff the worst (lowest) of the 4 adjacent-channel ratios is at or above the PW gate. The PW gate is sourced from kpi_thresholds.test_Tx_ACLR.min_limit — <TBD: confirm PW gate value from kpi_thresholds.json; current expectation is ≥ −45 dBc, matching the 3GPP TS 36.141 §6.5.4 conformance limit>.

ACLR is reported in dBc (decibels relative to the on-channel carrier power), so a more-negative number is worse leakage. The pass test is ACLR_meas ≥ PW_gate (i.e. −42 dBc passes against a −45 dBc gate; −47 dBc fails).

Worked example — assume PW gate = −45 dBc, B3 / fm / 10 MHz:

- Adjacent channels (±1 BW): ACLR = −47.2 dBc (lower), −48.1 dBc (upper)

- Alternate channels (±2 BW): ACLR = −52.4 dBc (lower), −53.0 dBc (upper)

- Worst of the 4 = −47.2 dBc → −47.2 ≥ −45 is FALSE → FAIL

Second example, same gate:

- Adjacent: −44.1 / −44.5 dBc; alternate: −50.0 / −50.2 dBc

- Worst = −44.1 dBc → −44.1 ≥ −45 is TRUE → PASS

Test verdict: PASS only if every antenna passes (worst of 4 ratios ≥ PW gate at every (band, BW, freq)).

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SA init,     │
  │    personality verify, RF switch ping,   │
  │    cable-loss cal per (port, freq)       │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  │    • set FHGW Tx power                   │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │      (channel-bandwidth + EARFCN)        │
  │    • set SA to ACP measurement mode      │
  │      with ±1/±2 BW channel definitions   │
  │    • (TDD only) DL-slot gating           │
  │    • fire rctdl 6111 <BW>                │
  │      (E-TM stream stays running across   │
  │       all freqs at this BW)              │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SA carrier reference freq       │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna    ◄─── next antenna │  ← this test
  │    • route RF switch                     │
  │    • apply cable loss                    │
  │    • auto-level SA                       │
  │    • read 4 ACLR values (±1, ±2 BW)      │
  │    • verdict = worst of 4 vs PW gate     │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     │  (rctdl keeps running)          │
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     │  (e.g. 5 → 10 → 20 MHz at this band)     │           push new startup XML,
     │  Same activated band — no ACTIVE_*.xml.  │           re-set SA ACP config, refire rctdl
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group to test?   │── yes ──► back to §5.2
     │  (e.g. did B1 in b0103, now want B3)     │           push different ACTIVE_*.xml,
     │  Same booted personality — no reboot.    │           re-set Tx power
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     │  (e.g. did all of b0103, now want b0307) │           (see "Update FHGW EEPROM
     │  Different personality — REBOOT needed.  │           model name" appendix),
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every antenna
                 at every (band, BW, freq) passes)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. Each BW change pushes a fresh FHGW startup XML carrying <channel-bandwidth> and <center-of-channel-bandwidth> (the EARFCN that defines fm), then re-arms the SA for ACP measurement and fires rctdl.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm in Hz (from the umbrella's Frequency points formula). | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2a | Switch the SA from Channel Power to ACP (Adjacent Channel Power) mode | SA | :CONF:ACPower:INIT:CONT ON | ACP measurement screen active |
| 2b | Define the ACP channel set: 1 reference channel + 2 adjacent (±1 BW) + 2 alternate (±2 BW). Substitute <BW_HZ> (e.g. 10000000 for 10 MHz). | SA | :ACPower:OFFSet:LIST:STATe ON,ON:ACPower:OFFSet:LIST:BANDwidth <BW_HZ>,<BW_HZ>:ACPower:OFFSet:LIST <BW_HZ>,<BW_HZ × 2>:ACPower:AVERage:STATe ON:ACPower:AVERage:COUNt 20 | Two offset markers (±1 BW, ±2 BW) visible; averaging on |
| 2c | Set the SA BW preset to match the reference channel BW. Substitute the BW token: B1M4 / B3M / B5M / B10M / B15M / B20M. | SA | :RAD:STAN:PRES B10M | Span auto-adjusts to ~5 × BW so ±1, ±2 channels fit on screen |
| 3 | Enable DL-slot gating. TDD bands only (B41) — skip on FDD. Values are for TDD config 3, special-subframe config 8; for other configs derive gate delay/length from TS 36.211 §4. | SA | :RAD:STAN:PRES:CONF 3:RAD:STAN:PRES:SSFR 8:TRIG:SOUR:GATE:OUTP:PAT:GATE:STAT ON:GATE:VIEW:STAT ON:GATE:VIEW:SWE 20 ms:GATE:DEL 3.1 ms:GATE:LEN 2.8 ms | Gated sweep; only DL window integrated |
| 4 | Fire E-TM 1.1 at the configured BW. Substitute the BW in MHz at the end (1.4 / 3 / 5 / 10 / 15 / 20) — must match step 1a's <channel-bandwidth> and step 2c's preset. <TBD: confirm rctdl ID — if PW automation uses E-TM 3.1 / 3.2 / 3.3 instead, substitute 6114 / 6115 / 6116>. | testmac console | rctdl 6111 10 | Carrier appears on SA; reference-channel power stable at ~P_rated; adjacent/alternate channel windows fill in. The E-TM stream stays running for all freq points below — do not refire between freqs. |

✓ FHGW transmitting at this BW; SA armed for ACP. → Continue to §5.3 for the first freq point.

#### 5.4 Per-antenna measurement (repeat for each antenna)

Do these steps once per antenna in this band's active TX set. The carrier is already up (§5.2.5 step 4), the SA is armed for ACP (§5.2.5 step 2b), and the carrier is centered on this freq point (§5.3). All that's left is routing the right antenna to the SA, applying the per-antenna cable-loss offset, and reading the 4 adjacent-channel ratios.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RF switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 (HTTP body) |
| 2 | Apply ant_i's calibrated cable-loss offset to the SA. The loss value comes from §5.1 steps 6–10 (or cable_loss.json) for the current freq. Enter as NEGATIVE dB (a 5 dB cable loss → -5). ACLR is a ratio so absolute loss cancels — but ref-level setting in step 3 needs this to put the carrier in-range. | SA | :CORR:BTS:GAIN <-CABLE_LOSS_DB> (e.g. for 5 dB loss: :CORR:BTS:GAIN -5) | SA trace shows reference channel at ~P_rated (cable-loss compensated) |
| 3 | Auto-level the SA for this antenna. Must be after step 2 — each antenna's cable loss is different, so the ref-level must be re-set per antenna. | SA | :SENSe:POWer:RF:RANGe:OPTimize IMMediate | Trace fills upper third of screen; no IF/RF overload indicator. Adjacent channels typically sit ~45 dB below the reference — verify they're above the SA's displayed noise floor; if not, decrease ref level by another 5 dB and retry. |
| 4 | Verify signal sync (visual check on SA front panel) | SA panel | — | Sync OK green; no red Frame Sync Lost / Burst Search Failed / Demod Error. If any fail → re-verify §5.2. |
| 5 | Trigger a single averaged sweep | SA | :INIT:IMM*WAI | "Single sweep complete" within ~1 s |
| 6 | Read the 4 ACLR ratios (lower-alternate, lower-adjacent, upper-adjacent, upper-alternate). Returns a comma-separated list of dBc values relative to the reference channel. <TBD: confirm exact SCPI for 4-channel ACP readout on Keysight E6640A LTE personality — :READ:ACPower:ACPower? typically returns a 5- or 6-element list; check whether the first element is on-channel power (dBm) or the first lower-alternate ratio (dBc)>. | SA | :READ:ACPower:ACPower? | Numeric list, e.g. −52.4, −47.2, −48.1, −53.0 (dBc) |
| 7 | Take the worst (least-negative) of the 4 ratios; compare against the PW gate from §4. | spreadsheet / notes | min_ratio = max(ACLR_array) (least-negative); pass iff min_ratio ≥ PW_gate | min_ratio ≥ PW_gate → ant_i PASS. Example: ACLR = [−52.4, −47.2, −48.1, −53.0], worst = −47.2 dBc, PW gate = −45 dBc → −47.2 ≥ −45 is FALSE → FAIL. |

Note — step 2 sign convention: :CORR:BTS:GAIN is an external-gain correction. A 5 dB cable loss is entered as -5 (loss = negative gain). The SA then adds +5 dB to its raw readings to refer the result back to the FHGW port. Since ACLR is a ratio between two channels measured in the same trace, the correction cancels in the ratio — it's needed only to put the on-channel carrier in the SA's calibrated range.

#### Loop control (when do I go where?)

After step 7, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq) | → back to §5.4 step 1 with the next ant_i |
| All antennas done for this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point (the E-TM stream from §5.2.5 step 4 is still running — don't refire) |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW (e.g. 10 → 20 MHz at B3). Push a new startup XML, re-arm ACP for the new BW, refire E-TM. No ACTIVE_*.xml change — same activated band. |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band (e.g. B1 → B3 in b0103). Push a different ACTIVE_*.xml, re-set Tx power. No reboot — same booted personality. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 for the first band in the new group. |
| All band-groups done | STOP. Overall verdict for the test = PASS iff every antenna at every (band, BW, freq) passes. |

Loose-coupling check (after one full antenna sweep): briefly disconnect the SA from one antenna and reconnect to another — both should show comparable ACLR (within ±1 dBc of each other on the worst-of-4). Confirms no vendor per-antenna calibration anomaly.

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| Adjacent ratio reads ~−20 to −30 dBc (very poor) | E-TM source not running, or reference channel measuring noise not carrier | Confirm rctdl 6111 <BW> is alive on testmac; re-verify §5.2.5 step 4; check Sync OK on SA |
| Lower adjacent passes, upper fails (or vice versa) by > 3 dB | Asymmetric PA distortion or a band-edge effect (running too close to fb/ft) | Verify carrier is actually centered (§5.3 step 1); try the fm freq point first to isolate edge effects |
| All 4 ratios read floor (~ −60 to −70 dBc) | SA noise-floor limited — adjacent channels below SA's sensitivity | Increase averaging count; decrease ref level via auto-level (§5.4 step 3); add pre-amplifier |
| Worst ratio drifts > 1 dBc over a minute | RU thermal back-off or insufficient averaging | Let the RU warm up 15+ min; increase SA average count to 50 |
| One antenna's ACLR much worse than the others (> 3 dBc gap) | Vendor's per-antenna PA bias off, or cable swap | Swap cable between two antennas to isolate the variable; if the gap moves with the antenna, the PA is at fault |
| Reference channel reads ~3 dB low on TDD | DL duty cycle not accounted for | Enable DL-slot gating (§5.2.5 step 3); use INST:SEL LTEATDD |
| ACP markers don't appear / SCPI returns only 1 value | Offset list not enabled or BW mismatch | Re-run §5.2.5 step 2b with correct <BW_HZ>; confirm :ACPower:OFFSet:LIST:STATe ON,ON |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §6.5.4 Adjacent Channel Leakage Power Ratio (ACLR), §6.5.4.2 Test requirements, §4.9.1 Test frequencies (fb / fm / ft), Annex C E-UTRA Test Models (E-TM 3.1 / 3.2 / 3.3 per spec; E-TM 1.1 per PW automation — see §3 TBD).

- TS 36.104 §6.6.2 ACLR (radio requirement — the −44.2 dBc / −45 dBc category B numbers that 36.141's test limit is built on).

## 3.4 Tx — ACLR OTA

3GPP references: TS 36.141 §6.5.4 (ACLR — conformance) + Annex F (OTA test procedures) + TS 36.104 §6.6.2 (radio requirement) — see §4 and §7.

Scope: this document is the per-test procedure for the over-the-air (OTA) ACLR variant — FHGW radiates into a calibrated anechoic chamber and the SA measures adjacent-channel ratios at chamber boresight via a reference antenna, corrected for the chamber's path-loss table. Common parameter tables (bands, frequency points, E-TMs) live in the PW Conformance — Common Test Parameters Reference above. This test uses E-TM 3.1a per the conformance plan — <TBD: confirm rctdl ID for E-TM 3.1a; 6114 is E-TM 3.1 in the umbrella>.

### 1. Purpose

Verify the FHGW's radiated adjacent channel leakage ratio (ACLR) at chamber boresight meets the per-carrier limit. The FHGW transmits a single LTE carrier through its antenna array; the SA — connected to a calibrated reference antenna inside the chamber — integrates power in the on-channel window and in the adjacent (±1 BW) and alternate (±2 BW) channel windows, after applying the per-(port, freq, angle) path-loss correction.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / E-TM tables.

Table 3-1 — ACLR OTA Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared TX ports for the band group, measured per-port or as composite array — <TBD: per-port vs composite> | — |
| Frequency | fb / fm / ft per (band, BW) — see Frequency points in the Reference | MHz |
| Power | P_rated (referred to FHGW antenna port; the radiated EIRP is P_rated + G_ant) | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference | MHz |
| E-TM | 3.1a (<TBD: confirm rctdl ID for E-TM 3.1a; 6114 covers E-TM 3.1>) | — |
| Positioner | Azimuth = 0°, Elevation = 0° (boresight) for conformance pass | deg |
| Adjacent channel offsets | ±1 × BW (adjacent), ±2 × BW (alternate) — total 4 measured channels | MHz |

### 4. Pass/Fail criteria (PW automation gate)

Each TX antenna passes iff the worst (lowest) of the 4 adjacent-channel ratios — measured radiatively at chamber boresight and path-loss corrected — is at or above the PW gate. The PW gate is sourced from kpi_thresholds.test_Tx_ACLR_OTA.min_limit — <TBD: confirm PW gate for OTA variant from kpi_thresholds.json; may match the conducted ≥ −45 dBc, or may be relaxed for OTA per 36.141 Annex F>.

ACLR is reported in dBc (decibels relative to the on-channel carrier power), so a more-negative number is worse leakage. In OTA the path-loss correction is identical for the reference channel and the adjacent channels (path loss is freq-flat across ±2 BW to within ~0.1 dB for typical chambers), so the ratio is preserved — but absolute level setting still needs the correction so the carrier lands in the SA's calibrated range.

Worked example — assume PW gate = −45 dBc, B3 / fm / 10 MHz, boresight:

- Reference channel (path-loss corrected, referred to FHGW antenna port): −15.4 dBm

- Adjacent (±1 BW): −47.0 dBc (lower), −47.8 dBc (upper)

- Alternate (±2 BW): −52.1 dBc (lower), −52.7 dBc (upper)

- Worst of 4 = −47.0 dBc → −47.0 ≥ −45 is FALSE → FAIL

Test verdict: PASS only if every measured antenna (or the composite array, depending on plan) passes at every (band, BW, freq) at boresight.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SA init,     │
  │    personality verify, chamber + posi-   │
  │    tioner ready, path-loss table loaded  │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  │    • set FHGW Tx power                   │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │      (channel-bandwidth + EARFCN)        │
  │    • set SA to ACP measurement mode      │
  │      with ±1/±2 BW channel definitions   │
  │    • (TDD only) DL-slot gating           │
  │    • fire rctdl <E-TM 3.1a> <BW>         │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SA carrier reference freq       │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna    ◄─── next antenna │  ← this test
  │    • move positioner to boresight        │
  │    • apply path-loss correction          │
  │    • auto-level SA                       │
  │    • read 4 ACLR values (±1, ±2 BW)      │
  │    • verdict = worst of 4 vs PW gate     │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group?           │── yes ──► back to §5.2
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every antenna
                 passes at boresight, every (band, BW, freq))

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. Each BW change pushes a fresh FHGW startup XML carrying <channel-bandwidth> and <center-of-channel-bandwidth> (the EARFCN that defines fm), then re-arms the SA for ACP and fires the E-TM 3.1a stream.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm in Hz (from the umbrella's Frequency points formula). | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2a | Switch the SA from Channel Power to ACP mode | SA | :CONF:ACPower:INIT:CONT ON | ACP measurement screen active |
| 2b | Define the ACP channel set: 1 reference + 2 adjacent (±1 BW) + 2 alternate (±2 BW). Substitute <BW_HZ>. | SA | :ACPower:OFFSet:LIST:STATe ON,ON:ACPower:OFFSet:LIST:BANDwidth <BW_HZ>,<BW_HZ>:ACPower:OFFSet:LIST <BW_HZ>,<BW_HZ × 2>:ACPower:AVERage:STATe ON:ACPower:AVERage:COUNt 20 | Two offset markers visible; averaging on |
| 2c | Set the SA BW preset. Substitute the BW token: B1M4 / B3M / B5M / B10M / B15M / B20M. | SA | :RAD:STAN:PRES B10M | Span auto-adjusts to ~5 × BW |
| 3 | Enable DL-slot gating. TDD bands only (B41) — skip on FDD. Values are for TDD config 3, special-subframe config 8. | SA | :RAD:STAN:PRES:CONF 3:RAD:STAN:PRES:SSFR 8:TRIG:SOUR:GATE:OUTP:PAT:GATE:STAT ON:GATE:VIEW:STAT ON:GATE:VIEW:SWE 20 ms:GATE:DEL 3.1 ms:GATE:LEN 2.8 ms | Gated sweep; only DL window integrated |
| 4 | Fire E-TM 3.1a at the configured BW. Substitute the BW in MHz at the end (1.4 / 3 / 5 / 10 / 15 / 20) — must match step 1a's <channel-bandwidth> and step 2c's preset. <TBD: confirm rctdl ID for E-TM 3.1a; the umbrella lists 6114 for E-TM 3.1 — 3.1a may share that ID or have a separate variant>. | testmac console | rctdl <TBD: 6114?> 10 | Carrier radiates from antenna array; reference-channel power stable on SA after path-loss correction. Stream stays running across all freqs at this BW — do not refire between freqs. |

✓ FHGW radiating at this BW; SA armed for ACP. → Continue to §5.3 for the first freq point.

#### 5.4 Per-antenna measurement (repeat for each antenna)

Do these steps once per antenna in this band's active TX set (or once for the composite array if running array-level OTA — see §2 Operating rules TBD). The carrier is already up (§5.2.5 step 4), the SA is armed for ACP (§5.2.5 step 2b), and centered on this freq point (§5.3). All that's left is moving the positioner to boresight, applying the per-(port, freq, angle) path-loss correction, and reading the 4 adjacent-channel ratios.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Move the positioner to boresight (az = 0°, el = 0°). For per-port runs, also orient the FHGW so ant_i faces the reference antenna; for array-level runs, leave the FHGW at its mounted pose. | positioner controller | <TBD: positioner SCPI/HTTP control — e.g. :POSition:AZIMuth 0 ; :POSition:ELEVation 0 ; or HTTP GET http://<POSI_IP>/move?az=0&el=0> | Positioner reports az = 0.0, el = 0.0. Wait <TBD: chamber settling time — typically 5–30 s>. |
| 2 | Apply the path-loss correction for (ant_i, current_freq, az=0, el=0) from the loaded table. Enter as NEGATIVE dB (a 40 dB path loss → -40). The SA then adds +40 dB to its raw readings to refer the result back to the FHGW antenna port. | SA | :CORR:BTS:GAIN <-PATH_LOSS_DB> (e.g. for 40 dB: :CORR:BTS:GAIN -40) | SA trace shows reference channel at ~P_rated + G_ant (path-loss compensated, referred to FHGW antenna port) |
| 3 | Auto-level the SA. Must be after step 2 — each (port, freq, angle) combination has a different path loss, so the ref-level must be re-set per measurement point. | SA | :SENSe:POWer:RF:RANGe:OPTimize IMMediate | Trace fills upper third of screen; no IF/RF overload. Verify adjacent channels are above SA's noise floor; if not, increase averaging or decrease ref level. |
| 4 | Verify signal sync (visual check on SA front panel) | SA panel | — | Sync OK green; no red Frame Sync Lost / Burst Search Failed / Demod Error. If any fail → re-verify §5.2 and chamber connectivity. |
| 5 | Trigger a single averaged sweep | SA | :INIT:IMM*WAI | "Single sweep complete" within ~1 s |
| 6 | Read the 4 ACLR ratios (lower-alternate, lower-adjacent, upper-adjacent, upper-alternate). | SA | :READ:ACPower:ACPower? (<TBD: confirm exact SCPI for 4-channel ACP readout on Keysight E6640A LTE personality — may need :CALC:ACP:RES? or vendor-specific variant>) | Numeric list, e.g. −52.1, −47.0, −47.8, −52.7 (dBc) |
| 7 | Take the worst (least-negative) of the 4 ratios; compare against the PW gate from §4. | spreadsheet / notes | min_ratio = max(ACLR_array); pass iff min_ratio ≥ PW_gate | min_ratio ≥ PW_gate → ant_i PASS. Example: ACLR = [−52.1, −47.0, −47.8, −52.7], worst = −47.0 dBc, PW gate = −45 dBc → −47.0 ≥ −45 is FALSE → FAIL. |

Note — step 2 sign convention: :CORR:BTS:GAIN is an external-gain correction. A 40 dB path loss is entered as -40 (loss = negative gain). Since ACLR is a ratio between two channels measured in the same trace and path loss is essentially flat across ±2 BW, the correction cancels in the ratio — it's needed only to put the on-channel carrier in the SA's calibrated range and to refer the absolute reference-channel reading back to the FHGW antenna port.

#### Loop control (when do I go where?)

After step 7, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq) | → back to §5.4 step 1 with the next ant_i (positioner re-aim if per-port) |
| All antennas done for this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point (E-TM stream from §5.2.5 step 4 is still running — don't refire) |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW. Push a new startup XML, re-arm ACP for the new BW, refire E-TM. No ACTIVE_*.xml change — same activated band. |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band. Push a different ACTIVE_*.xml, re-set Tx power. No reboot — same booted personality. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 for the first band in the new group. |
| All band-groups done | STOP. Overall verdict for the test = PASS iff every antenna (or the composite array) passes at boresight at every (band, BW, freq). |

Off-boresight pattern sweep (optional) — if your conformance plan calls for ACLR across the radiation pattern (e.g. ±30° az / ±30° el grid), loop §5.4 over the angle grid with the path-loss correction re-fetched from the table per angle. The PW gate still applies on a per-angle basis; reporting is per-angle.

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| Reference channel reads 30–60 dB low | Path-loss correction not applied or wrong (port, freq, angle) row | Re-verify §5.4 step 2 — confirm :CORR:BTS:GAIN reflects the actual path loss for this (port, freq, az=0, el=0) |
| Adjacent ratio reads ~−20 to −30 dBc (very poor) | E-TM source not running, or reference channel measuring noise not carrier | Confirm rctdl <ETM_ID> <BW> is alive on testmac; re-verify §5.2.5 step 4; check Sync OK on SA |
| All 4 ratios read noise floor (~ −55 to −65 dBc) | Path-loss correction too high, putting adjacents below SA sensitivity, OR chamber reflections smearing the carrier | Increase averaging count; verify path-loss table loaded correctly; check chamber for stray reflections (loose absorber tiles?) |
| Worst ratio differs by > 3 dBc from conducted result on same FHGW | Antenna pattern asymmetry, or off-boresight orientation | Verify positioner is truly at az = 0°, el = 0°; verify FHGW pose; cross-check by running an azimuth sweep to find the actual peak |
| One antenna's ACLR much worse than the others (> 3 dBc gap, per-port mode) | Antenna element fault, or PA bias asymmetry on that branch | Repeat just the bad antenna; if reproducible, escalate as a HW finding |
| Reference channel reads ~3 dB low on TDD | DL duty cycle not accounted for | Enable DL-slot gating (§5.2.5 step 3); use INST:SEL LTEATDD |
| Reading drifts > 1 dBc over a minute | Chamber thermal drift, or positioner micro-motion | Let chamber stabilise 15+ min after a positioner move; verify positioner is in hold after motion complete |
| ACP markers don't appear / SCPI returns only 1 value | Offset list not enabled or BW mismatch | Re-run §5.2.5 step 2b with correct <BW_HZ> |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §6.5.4 Adjacent Channel Leakage Power Ratio (ACLR), §6.5.4.2 Test requirements, §4.9.1 Test frequencies (fb / fm / ft), Annex C E-UTRA Test Models (E-TM 3.1a), Annex F OTA test procedures (chamber, path-loss correction, positioner conventions).

- TS 36.104 §6.6.2 ACLR (radio requirement).

## 3.5 Tx — BS Output Power OTA

3GPP references: TS 36.141 §6.2 (BS Output Power — conformance) + Annex F (OTA test procedures) + TS 36.104 §6.2 (radio requirement) — see §4 and §7.

Scope: this document is the per-test procedure for the over-the-air (OTA) BS Output Power variant — FHGW radiates into a calibrated anechoic chamber and the SA measures EIRP-equivalent channel power at chamber boresight via a reference antenna, corrected for the chamber's path-loss table. Common parameter tables (bands, frequency points, E-TMs) live in the PW Conformance — Common Test Parameters Reference above. This test uses E-TM 3.1a per the conformance plan — <TBD: confirm rctdl ID for E-TM 3.1a; the umbrella lists 6114 for E-TM 3.1>.

### 1. Purpose

Verify the FHGW's per-antenna radiated transmit output power matches P_rated (referred to the FHGW antenna port after correcting for chamber path loss) for the configured band group. The FHGW transmits a single LTE carrier through its antenna array; the SA — connected to a calibrated reference antenna inside the chamber — integrates the on-channel power at boresight and the path-loss correction refers the result back to the FHGW antenna port.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / E-TM tables.

Table 3-1 — BS Output Power OTA Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared TX ports for the band group, measured per-port or as composite array — <TBD: per-port vs composite> | — |
| Frequency | fb / fm / ft per (band, BW) — see Frequency points in the Reference | MHz |
| Power | P_rated (referred to FHGW antenna port; radiated EIRP = P_rated + G_ant) | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference | MHz |
| E-TM | 3.1a (<TBD: confirm rctdl ID for E-TM 3.1a; 6114 covers E-TM 3.1>) | — |
| Positioner | Azimuth = 0°, Elevation = 0° (boresight) for conformance pass | deg |

### 4. Pass/Fail criteria (PW automation gate)

Each TX antenna passes iff |P_meas − P_rated| ≤ Δ, where P_meas is the path-loss-corrected channel power referred to the FHGW antenna port and Δ = ±1.5 dB (PW's internal gate, matching the conducted variant). <TBD: confirm whether the PW gate for OTA matches the conducted ±1.5 dB or is relaxed per 36.141 Annex F — typical OTA tolerance budgets are ±2.5 to ±4.0 dB to account for chamber uncertainty>.

The PW gate is sourced from kpi_thresholds.test_Tx_BS_Output_Power_OTA.{min_limit, max_limit} — <TBD: confirm PW gate values from kpi_thresholds.json>.

Worked example — B3, P_rated = −15.7 dBm, PW gate = ±1.5 dB, boresight:

- P_meas = −15.3 dBm (after applying PL(Tx6, fm=1842.5 MHz, az=0, el=0) = 42.1 dB as an external-gain correction on the SA) → |diff| = 0.4 → PASS

- P_meas = −17.6 dBm (same correction) → |diff| = 1.9 → FAIL

Test verdict: PASS only if every measured antenna (or the composite array, depending on plan) passes at every (band, BW, freq) at boresight.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SA init,     │
  │    personality verify, chamber + posi-   │
  │    tioner ready, path-loss table loaded  │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  │    • set FHGW Tx power                   │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │      (channel-bandwidth + EARFCN)        │
  │    • set SA BW preset                    │
  │    • (TDD only) DL-slot gating           │
  │    • fire rctdl <E-TM 3.1a> <BW>         │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SA carrier reference freq       │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna    ◄─── next antenna │  ← this test
  │    • move positioner to boresight        │
  │    • apply path-loss correction          │
  │    • auto-level SA                       │
  │    • read channel power                  │
  │    • verdict = |P_meas − P_rated| vs Δ   │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group?           │── yes ──► back to §5.2
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every antenna
                 passes at boresight, every (band, BW, freq))

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. Each BW change pushes a fresh FHGW startup XML carrying <channel-bandwidth> and <center-of-channel-bandwidth> (the EARFCN that defines fm), then re-arms the SA and fires the E-TM 3.1a stream.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm in Hz (from the umbrella's Frequency points formula). | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Set the SA BW preset to match your BW. Substitute the BW token: B1M4 / B3M / B5M / B10M / B15M / B20M. | SA | :RAD:STAN:PRES B10M | Span auto-adjusts |
| 3 | Enable DL-slot gating. TDD bands only (B41) — skip on FDD. Values are for TDD config 3, special-subframe config 8; for other configs derive gate delay/length from TS 36.211 §4. | SA | :RAD:STAN:PRES:CONF 3:RAD:STAN:PRES:SSFR 8:TRIG:SOUR:GATE:OUTP:PAT:GATE:STAT ON:GATE:VIEW:STAT ON:GATE:VIEW:SWE 20 ms:GATE:DEL 3.1 ms:GATE:LEN 2.8 ms | Gated sweep; only DL window integrated |
| 4 | Fire E-TM 3.1a at the configured BW. Substitute the BW in MHz at the end (1.4 / 3 / 5 / 10 / 15 / 20) — must match step 1a's <channel-bandwidth> and step 2's preset. <TBD: confirm rctdl ID for E-TM 3.1a; the umbrella lists 6114 for E-TM 3.1 — 3.1a may share that ID or have a separate variant>. | testmac console | rctdl <TBD: 6114?> 10 | Carrier radiates from antenna array; channel-power stable on SA after path-loss correction. The E-TM stream stays running for all freq points below — do not refire between freqs. |

✓ FHGW radiating E-TM 3.1a at this BW. → Continue to §5.3 for the first freq point.

#### 5.4 Per-antenna measurement (repeat for each antenna)

Do these steps once per antenna in this band's active TX set (or once for the composite array if running array-level OTA — see §2 Operating rules TBD). The carrier is already up (§5.2.5 step 4) and the SA is centered on this freq point (§5.3). All that's left is moving the positioner to boresight, applying the per-(port, freq, angle) path-loss correction, and reading the channel power.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Move the positioner to boresight (az = 0°, el = 0°). For per-port runs, also orient the FHGW so ant_i faces the reference antenna; for array-level runs, leave the FHGW at its mounted pose. | positioner controller | <TBD: positioner SCPI/HTTP control — e.g. :POSition:AZIMuth 0 ; :POSition:ELEVation 0 ; or HTTP GET http://<POSI_IP>/move?az=0&el=0> | Positioner reports az = 0.0, el = 0.0. Wait <TBD: chamber settling time — typically 5–30 s>. |
| 2 | Apply the path-loss correction for (ant_i, current_freq, az=0, el=0) from the loaded table. Enter as NEGATIVE dB (a 40 dB path loss → -40). The SA then adds +40 dB to its raw readings to refer the result back to the FHGW antenna port. | SA | :CORR:BTS:GAIN <-PATH_LOSS_DB> (e.g. for 40 dB: :CORR:BTS:GAIN -40) | SA trace shows carrier at ~P_rated (path-loss compensated, referred to FHGW antenna port) |
| 3 | Auto-level the SA. Must be after step 2 — each (port, freq, angle) combination has a different path loss, so the ref-level must be re-set per measurement point. | SA | :SENSe:POWer:RF:RANGe:OPTimize IMMediate | Trace fills upper third of screen; no IF/RF overload indicator |
| 4 | Verify signal sync (visual check on SA front panel) | SA panel | — | Sync OK green; no red Frame Sync Lost / Burst Search Failed / Demod Error. If any fail → re-verify §5.2 and chamber connectivity. |
| 5 | Trigger a single averaged sweep | SA | :INIT:IMM*WAI | "Single sweep complete" within ~1 s |
| 6 | Read integrated channel power | SA | :READ:CHPower:CHPower? | Numeric dBm value = P_meas_i (path-loss corrected per step 2, referred to FHGW antenna port) |
| 7 | Compare against the pass window from §4 | spreadsheet / notes | P_rated − Δ ≤ P_meas_i ≤ P_rated + Δ where Δ = ±1.5 dB (or PW OTA gate per §4 TBD) | P_meas_i inside window → ant_i PASS. Example: P_rated = −15.7, P_meas_i = −15.3 → window [−17.2, −14.2] → inside → PASS. |

Note — step 2 sign convention: :CORR:BTS:GAIN is an external-gain correction. A 40 dB path loss is entered as -40 (loss = negative gain). The SA then adds +40 dB to its raw readings to refer the result back to the FHGW antenna port.

#### Loop control (when do I go where?)

After step 7, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq) | → back to §5.4 step 1 with the next ant_i (positioner re-aim if per-port) |
| All antennas done for this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point (E-TM stream from §5.2.5 step 4 is still running — don't refire) |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW (e.g. 10 → 20 MHz at B3). Push a new startup XML, re-set SA BW preset, refire E-TM. No ACTIVE_*.xml change — same activated band. |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band (e.g. B1 → B3 in b0103). Push a different ACTIVE_*.xml, re-set Tx power. No reboot — same booted personality. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 for the first band in the new group. |
| All band-groups done | STOP. Overall verdict for the test = PASS iff every antenna (or the composite array) passes at boresight at every (band, BW, freq). |

Off-boresight pattern sweep (optional) — if your conformance plan calls for output-power characterisation across the radiation pattern (e.g. ±30° az / ±30° el grid), loop §5.4 over the angle grid with the path-loss correction re-fetched from the table per angle. Reporting is per-angle EIRP and per-angle pass/fail vs the PW gate.

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| Reading is 30–60 dB low | Path-loss correction not applied or wrong (port, freq, angle) row | Re-verify §5.4 step 2 — confirm :CORR:BTS:GAIN reflects the actual path loss for this (port, freq, az=0, el=0) |
| Reading is ~3 dB low on TDD | DL duty cycle not accounted for | Use a TDD-aware channel-power measurement (INST:SEL LTEATDD); enable DL-slot gating (§5.2.5 step 3) |
| Reading drifts > 1 dB over a minute | Chamber thermal drift, RU thermal back-off, or positioner micro-motion | Let chamber + RU stabilise 15+ min after motion; verify positioner is in hold state; increase SA average count |
| One antenna reads radically different than the others (per-port mode) | FHGW pose not re-aimed for this antenna, or antenna element fault | Repeat with the antenna explicitly oriented to boresight; if still off, escalate as a HW finding |
| IF-overload indicator on SA | Reference level too low, or path-loss correction too small (under-estimating loss) | Raise SA ref level by 5 dB and retry; if still overloading, re-verify the path-loss table value |
| Channel power flickers ±2 dB | E-TM source not stable, chamber reflections, or insufficient averaging | Confirm RU is in test mode and rctdl <ETM> <BW> is running; check chamber for stray reflections; raise SA average count |
| Reading differs by > 2 dB from the conducted variant on the same FHGW | Antenna pattern asymmetry, or off-boresight orientation | Verify positioner is truly at az = 0°, el = 0°; cross-check by running a short azimuth sweep to find the actual radiation peak |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §6.2 BS Output Power, §6.2.3.1 Test requirements, §4.9.1 Test frequencies (fb / fm / ft), Annex C E-UTRA Test Models (E-TM 3.1a), Annex F OTA test procedures (chamber, path-loss correction, positioner conventions).

- TS 36.104 §6.2 BS output power (radio requirement — the ±2.0 dB number that 36.141's ±2.7 dB is built on).

## 3.6 Tx — DL Reference Signal Power

3GPP references: TS 36.141 §6.5.5 (conformance — DL RS power accuracy) + TS 36.104 §6.5.5 (radio requirement) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, E-TMs) live in the PW Conformance — Common Test Parameters Reference above. This test uses E-TM 1.1 (rctdl 6111) — same stream as BS Output Power, different SA readout.

### 1. Purpose

Verify the FHGW's per-antenna DL Reference Signal (DLRS / CRS) power matches the carrier-derived expected value for the configured band group. "DL Reference Signal Power" is the average power of the Cell-specific Reference Signal REs across the OFDM symbols that carry them — not the total channel power. The two differ because the CRS carries power on only a small fraction of subcarriers; total channel power equals CRS power × N_RB × 12 (CRS-EPRE convention). Conducted-RF, low-power test — output flows through a passive cable chain into the EXM. No chamber, no OTA.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / E-TM tables.

Table 3-1 — DLRS Power Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared TX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Frequency | fb / fm / ft per (band, BW) — see Frequency points in the Reference | MHz |
| Power (expected DLRS) | P_rated − 10·log10(N_RB × 12) per (band, BW) — see worked example in §4 | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference for all valid options | MHz |
| E-TM | 1.1 (rctdl 6111) | — |

#### N_RB lookup (number of Resource Blocks per BW)

| BW (MHz) | N_RB | N_RB × 12 (CRS subcarriers) | 10·log10(N_RB × 12) (dB) |
| --- | --- | --- | --- |
| 1.4 | 6 | 72 | 18.6 |
| 3 | 15 | 180 | 22.6 |
| 5 | 25 | 300 | 24.8 |
| 10 | 50 | 600 | 27.8 |
| 15 | 75 | 900 | 29.5 |
| 20 | 100 | 1200 | 30.8 |

### 4. Pass/Fail criteria (PW automation gate)

Each antenna passes iff |P_DLRS_meas − P_DLRS_expected| ≤ Δ, where:

- P_DLRS_expected = P_rated − 10·log10(N_RB × 12) (per band, per BW — see §3 N_RB table)

- Δ = <TBD: PW DLRS gate from kpi_thresholds.test_Tx_DLRS — likely ±3 dB hardcoded> (source survey note: "Config range or ±3 dB hardcoded"). The 3GPP TS 36.141 §6.5.5 conformance limit is ±2.7 dB; PW's gate is at least that tight or tighter.

Worked example — B3 at 10 MHz, P_rated = −15.7 dBm, N_RB = 50:

- P_DLRS_expected = −15.7 − 10·log10(50 × 12) = −15.7 − 10·log10(600) = −15.7 − 27.8 = −43.5 dBm

- PW gate window (assuming Δ = ±3 dB): [−46.5, −40.5] dBm

- P_DLRS_meas = −43.2 dBm → |diff| = 0.3 → PASS

- P_DLRS_meas = −47.0 dBm → |diff| = 3.5 → FAIL

Test verdict: PASS only if every antenna passes at every (band, BW, freq).

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SA init,     │
  │    personality verify, RF switch ping,   │
  │    cable-loss cal per (port, freq)       │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  │    • set FHGW Tx power                   │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │      (channel-bandwidth + EARFCN)        │
  │    • switch SA to CEVM mode              │
  │    • set SA BW preset                    │
  │    • (TDD only) DL-slot gating           │
  │    • fire rctdl 6111 <BW>                │
  │      (E-TM stream stays running across   │
  │       all freqs at this BW)              │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SA carrier reference freq       │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna    ◄─── next antenna │  ← this test
  │    • route RF switch                     │
  │    • apply cable loss                    │
  │    • trigger CEVM sweep                  │
  │    • read DLRS power                     │
  │    • compare vs P_rated − 10·log10(N×12) │
  │    • verdict                             │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     │  (rctdl keeps running)          │
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     │  (e.g. 5 → 10 → 20 MHz at this band)     │           push new startup XML,
     │  Same activated band — no ACTIVE_*.xml.  │           re-set SA BW preset, refire rctdl
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group to test?   │── yes ──► back to §5.2
     │  (e.g. did B1 in b0103, now want B3)     │           push different ACTIVE_*.xml,
     │  Same booted personality — no reboot.    │           re-set Tx power
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     │  (e.g. did all of b0103, now want b0307) │           (see "Update FHGW EEPROM
     │  Different personality — REBOOT needed.  │           model name" appendix),
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every antenna
                 at every (band, BW, freq) passes)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. Each BW change pushes a fresh FHGW startup XML carrying <channel-bandwidth> and <center-of-channel-bandwidth> (the EARFCN that defines fm), then switches the SA to CEVM mode and fires rctdl. The DLRS readout is a CEVM output — :CONF:CHPower is replaced by :CONF:CEVM here.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm in Hz (from the umbrella's Frequency points formula). | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2a | Switch the SA to CEVM (Cell EVM) measurement mode. DLRS power is a CEVM output, not a channel-power output — this replaces the §5.1 step 4 :CONF:CHPower setup for this test. | SA | :CONF:CEVM:INIT:CONT ON:CEVM:AVERage:STATe ON:CEVM:AVERage:COUNt 20 | CEVM result table appears; DLRS Power row populated |
| 2b | Set the SA BW preset to match your BW. Substitute the BW token: B1M4 / B3M / B5M / B10M / B15M / B20M. | SA | :RAD:STAN:PRES B10M | Span auto-adjusts |
| 3 | Enable DL-slot gating. TDD bands only (B41) — skip on FDD. Values are for TDD config 3, special-subframe config 8; for other configs derive gate delay/length from TS 36.211 §4. | SA | :RAD:STAN:PRES:CONF 3:RAD:STAN:PRES:SSFR 8:TRIG:SOUR:GATE:OUTP:PAT:GATE:STAT ON:GATE:VIEW:STAT ON:GATE:VIEW:SWE 20 ms:GATE:DEL 3.1 ms:GATE:LEN 2.8 ms | Gated sweep; only DL window integrated |
| 4 | Fire E-TM 1.1 at the configured BW. Substitute the BW in MHz at the end (1.4 / 3 / 5 / 10 / 15 / 20) — must match step 1a's <channel-bandwidth> and step 2b's preset. | testmac console | rctdl 6111 10 | Carrier appears on SA; CEVM DLRS Power row populates within ~3 s. The E-TM stream stays running for all freq points below — do not refire between freqs. |

✓ FHGW transmitting E-TM 1.1 at this BW; SA in CEVM mode. → Continue to §5.3 for the first freq point.

#### 5.4 Per-antenna measurement (repeat for each antenna)

Do these steps once per antenna in this band's active TX set. The carrier is already up (§5.2.5 step 4) and the SA is centered on this freq point (§5.3). All that's left is routing the right antenna to the SA, applying its cable loss, triggering the CEVM measurement, and reading the DLRS power row.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RF switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 (HTTP body) |
| 2 | Apply ant_i's calibrated cable-loss offset to the SA. The loss value comes from §5.1 steps 6–10 (or cable_loss.json) for the current freq. Enter as NEGATIVE dB (a 5 dB cable loss → -5). | SA | :CORR:BTS:GAIN <-CABLE_LOSS_DB> (e.g. for 5 dB loss: :CORR:BTS:GAIN -5) | SA trace shows carrier cable-loss compensated |
| 3 | Verify signal sync (visual check on SA front panel) | SA panel | — | Sync OK green; no red Frame Sync Lost / Burst Search Failed / Demod Error. If any fail → re-verify §5.2. |
| 4 | Trigger a single averaged CEVM sweep | SA | :INIT:IMM*WAI | "Single sweep complete" within ~1 s; CEVM result table refreshes |
| 5 | Read DLRS power from the CEVM result table | SA | <TBD: exact SCPI for DLRS readout> (working name: :READ:CEVM:DLRS:POWer?; the Keysight LTE CEVM measurement returns DLRS power as one of its sub-results — confirm the exact mnemonic against your firmware's SCPI reference) | Numeric dBm value = P_DLRS_meas_i (cable-loss corrected per step 2) |
| 6 | Compute P_DLRS_expected = P_rated − 10·log10(N_RB × 12) using the §3 N_RB table for the current BW | spreadsheet / notes | e.g. B3 / 10 MHz: −15.7 − 27.8 = −43.5 | P_DLRS_expected in dBm |
| 7 | Compare against the pass window from §4 | spreadsheet / notes | P_DLRS_expected − Δ ≤ P_DLRS_meas_i ≤ P_DLRS_expected + Δ where Δ = <TBD: PW DLRS gate> | P_DLRS_meas_i inside window → ant_i PASS. Example: P_DLRS_expected = −43.5, P_DLRS_meas_i = −43.2 → window [−46.5, −40.5] (Δ = 3 dB) → inside → PASS. |

Note — step 2 sign convention: :CORR:BTS:GAIN is an external-gain correction. A 5 dB cable loss is entered as -5 (loss = negative gain). The SA then adds +5 dB to its raw readings to refer the result back to the FHGW port.

#### Loop control (when do I go where?)

After step 7, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq) | → back to §5.4 step 1 with the next ant_i |
| All antennas done for this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point (the E-TM stream from §5.2.5 step 4 is still running — don't refire) |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW (e.g. 10 → 20 MHz at B3). Push a new startup XML, re-set SA BW preset, refire E-TM. No ACTIVE_*.xml change — same activated band. Note: P_DLRS_expected changes with BW (N_RB changes) — recompute. |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band (e.g. B1 → B3 in b0103). Push a different ACTIVE_*.xml, re-set Tx power. No reboot — same booted personality. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 for the first band in the new group. |
| All band-groups done | STOP. Overall verdict for the test = PASS iff every antenna at every (band, BW, freq) passes. |

Sanity check (after one full antenna sweep): the per-antenna spread on DLRS power should be tight — the underlying carrier is fed through the same FHGW DAC chain, and any per-antenna drift you see here usually shows up identically in the BS Output Power test. If one antenna's DLRS reading drifts far from the others (> ±1.5 dB), suspect a cable / connector / per-antenna gain issue, not a CRS modulation problem.

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| DLRS reading is ~10 dB low | Cable loss not subtracted, or wrong attenuator | Re-verify cable/attenuator calibration with a known source |
| DLRS reading is ~27.8 dB low at 10 MHz (matches the BW offset) | Comparing measured DLRS to P_rated directly without subtracting 10·log10(N_RB × 12) | Recompute P_DLRS_expected using §3 N_RB table |
| CEVM result table shows -- for DLRS Power | SA hasn't sync'd to the carrier yet, or CEVM averaging hasn't completed first burst | Confirm Sync OK green; re-trigger :INIT:IMM; wait one full average cycle |
| Reading is ~3 dB low on TDD | DL duty cycle not accounted for | Verify gating (§5.2.5 step 3) is enabled and the gate window is centered on a DL slot |
| DLRS drifts > 0.5 dB over a minute | RU thermal back-off or insufficient averaging | Let the RU warm up 15+ min; increase CEVM average count |
| One antenna reads radically different DLRS | Vendor's per-antenna calibration off, or cable swap | Swap cable + attenuator between two antennas to isolate the variable. Cross-check against that antenna's BS Output Power reading — if both move together, it's the chain; if only DLRS moves, suspect a CRS-power scaling bug in the FHGW config. |
| Demod Error / Frame Sync Lost on SA | EARFCN mismatch between FHGW and SA | Re-verify §5.2.5 step 1 (FHGW <center-of-channel-bandwidth>) matches §5.3 step 1 (SA :CCARrier:REFerence) |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §6.5.5 DL RS power accuracy, §4.9.1 Test frequencies (fb / fm / ft), Annex C E-UTRA Test Models (E-TM 1.1).

- TS 36.104 §6.5.5 DL RS power (radio requirement — defines DLRS as per-RE CRS power and sets the ±2.0 dB radio limit that 36.141's ±2.7 dB derives from).

- TS 36.211 §6.10 Reference signals (CRS structure, RE mapping — used to derive the N_RB × 12 scaling factor).

## 3.7 Tx — EVM

3GPP references: TS 36.141 §6.5.2 (Error Vector Magnitude) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, E-TMs) live in the PW Conformance — Common Test Parameters Reference above. This test uses two E-TMs: E-TM 2.0 and E-TM 3.1 — each ETM is measured as its own sweep through §5.3 → §5.4.

### 1. Purpose

Verify the per-antenna modulation quality of the FHGW transmitter. EVM (Error Vector Magnitude) measures the RMS distance between the demodulated constellation points and their ideal positions, expressed as a % of the ideal vector magnitude. Two E-TMs are required:

- E-TM 2.0 — single-PRB, 64-QAM. Stresses the transmitter at a narrow allocation; sensitive to phase noise and LO leakage.

- E-TM 3.1 — full-BW, 64-QAM. Stresses the transmitter at full PA load; sensitive to PA non-linearity.

Conducted-RF, low-power test — output flows through a passive cable chain into the EXM. No chamber, no OTA.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / E-TM tables.

Table 3-1 — EVM Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared TX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Frequency | fb / fm / ft per (band, BW) — see Frequency points in the Reference | MHz |
| Power | P_rated — per band group, see Rated output power in the Reference (auto-level handles per-antenna cable loss) | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference for all valid options (Common BW selections per band group is informational only) | MHz |
| E-TM | 2.0 (rctdl 6113) — single-PRB 64-QAM3.1 (rctdl 6114) — full-BW 64-QAM | — |
| Measurement | Composite EVM (CEVM) on EXM | % |

### 4. Pass/Fail criteria (PW automation gate)

Each (antenna, ETM) passes iff EVM_meas ≤ EVM_gate[ETM], where the PW automation gate is tighter than the 3GPP TS 36.104 §6.5.2.1 conformance limit:

| E-TM | PW gate (EVM%) | 3GPP TS 36.104 limit (EVM%) |
| --- | --- | --- |
| 2.0 (single PRB, 64-QAM) | <TBD: per-ETM EVM% gate from kpi_thresholds.test_Tx_EVM> | ≤ 8.0 % |
| 3.1 (full BW, 64-QAM) | <TBD: per-ETM EVM% gate from kpi_thresholds.test_Tx_EVM> | ≤ 8.0 % |

Worked example — B3, E-TM 3.1, PW gate <TBD> (assume 6.0% for the example):

- EVM_meas = 4.2 % → 4.2 ≤ 6.0 → PASS

- EVM_meas = 7.3 % → 7.3 > 6.0 → FAIL (would still PASS the 3GPP 8.0% limit, but fails the PW automation gate)

Test verdict: PASS only if every (antenna, ETM) combination passes.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test. EVM runs §5.4 twice per (band, BW, freq) — once at E-TM 2.0, once at E-TM 3.1.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SA init,     │
  │    personality verify, RF switch ping,   │
  │    cable-loss cal per (port, freq)       │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  │    • set FHGW Tx power                   │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW + per ETM                 │  ← this test
  │    For ETM in {2.0, 3.1}:                │
  │    • edit + push oru_startup_slave.xml   │
  │    • set SA BW preset, CEVM mode         │
  │    • (TDD only) DL-slot gating           │
  │    • fire rctdl <ID> <BW>                │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SA carrier reference freq       │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna    ◄─── next antenna │  ← this test
  │    • route RF switch                     │
  │    • apply cable loss                    │
  │    • trigger CEVM sweep                  │
  │    • read EVM%                           │
  │    • verdict vs ETM-specific gate        │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more ETMs to run at this BW?             │── yes ──► back to §5.2.5
     │  (do 2.0, then 3.1)                      │           refire rctdl with next ETM
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group?           │── yes ──► back to §5.2
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every (antenna, ETM)
                 at every (band, BW, freq) passes)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW + per-ETM setup (repeat for each BW, then for each ETM at that BW)

EVM requires two ETMs (E-TM 2.0 and E-TM 3.1) per (band, BW). Inner loop: fix BW, sweep ETM. Outer loop: next BW.

For the current BW × ETM combination:

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm in Hz (from the umbrella's Frequency points formula). Only re-do this on the first ETM at a new BW — the FHGW XML is BW-dependent, not ETM-dependent. | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW. First-ETM-at-this-BW only — skip on the second ETM at the same BW. | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Switch the SA from Channel Power mode to Composite EVM (CEVM) mode | SA | :CONF:CEVM:INIT:CONT ON:CEVM:AVERage:STATe ON:CEVM:AVERage:COUNt 10 | EXM screen shows constellation + EVM% readout |
| 3 | Set the SA BW preset to match your BW. Substitute the BW token: B1M4 / B3M / B5M / B10M / B15M / B20M. | SA | :RAD:STAN:PRES B10M | Span auto-adjusts; demodulator re-locks |
| 4 | Enable DL-slot gating. TDD bands only (B41) — skip on FDD. Values are for TDD config 3, special-subframe config 8; for other configs derive gate delay/length from TS 36.211 §4. | SA | :RAD:STAN:PRES:CONF 3:RAD:STAN:PRES:SSFR 8:TRIG:SOUR:GATE:OUTP:PAT:GATE:STAT ON:GATE:DEL 3.1 ms:GATE:LEN 2.8 ms | Gated demod; only DL window analysed |
| 4a | Fire the current ETM at the configured BW. First pass: ETM 2.0 → rctdl 6113 <BW>. Second pass: ETM 3.1 → rctdl 6114 <BW>. The <BW> token is the BW in MHz at the end (1.4 / 3 / 5 / 10 / 15 / 20) and must match step 1a's <channel-bandwidth> and step 3's preset. | testmac console | rctdl <RCTDL_ID> <BW_MHZ> (e.g. rctdl 6113 10) | Carrier appears on SA; constellation shape matches expected modulation (single PRB visible for 2.0, full BW for 3.1) |

✓ FHGW transmitting current E-TM at this BW; SA in CEVM mode. → Continue to §5.3 for the first freq point. When all freq points + antennas are done for this ETM, return here and refire rctdl with the next ETM (re-do step 4a only).

#### 5.4 Per-antenna measurement (repeat for each antenna, per ETM)

Do these steps once per antenna per ETM at the current (band, BW, freq). The carrier is already up at the current ETM (§5.2.5 step 4a) and the SA is in CEVM mode (§5.2.5 step 2). All that's left is routing the right antenna, triggering a sweep, and reading EVM%.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RF switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 (HTTP body) |
| 2 | Apply ant_i's calibrated cable-loss offset to the SA. The loss value comes from §5.1 steps 6–10 (or cable_loss.json) for the current freq. Enter as NEGATIVE dB (a 5 dB cable loss → -5). EVM is a relative metric, but the SA still uses this to pick a safe ref-level. | SA | :CORR:BTS:GAIN <-CABLE_LOSS_DB> (e.g. for 5 dB loss: :CORR:BTS:GAIN -5) | Ref level adjusts; carrier sits in the linear region of the digitiser |
| 3 | Auto-level the SA for this antenna. Must be after step 2 — each antenna's cable loss is different. | SA | :SENSe:POWer:RF:RANGe:OPTimize IMMediate | Trace fills upper third of screen; no IF/RF overload indicator |
| 4 | Verify the CEVM demodulator is locked (visual check on SA front panel) | SA panel | — | Sync OK green; constellation visible; no red Demod Error / Frame Sync Lost. If any fail → re-verify §5.2.5. |
| 5 | Trigger a single averaged CEVM sweep | SA | :INIT:IMM*WAI | "Single sweep complete" within ~2 s |
| 6 | Read composite EVM (RMS, all subcarriers / symbols, average over the sweep) | SA | :READ:CEVM:RSEVM? (<TBD: confirm exact CEVM SCPI mnemonic for the Keysight E6640A — alternates:* :FETCh:CEVM:RSEVM?*,* :READ:CEVM[1]? *with field index>) | Numeric % value = EVM_meas_i (e.g. 4.2) |
| 7 | Compare against the ETM-specific gate from §4 | spreadsheet / notes | EVM_meas_i ≤ EVM_gate[<CURRENT_ETM>] | EVM_meas_i inside window → (ant_i, current ETM) PASS. Example: ETM 3.1, EVM_gate = 6.0%, EVM_meas_i = 4.2% → 4.2 ≤ 6.0 → PASS. |

Note — step 2 sign convention: :CORR:BTS:GAIN is an external-gain correction. A 5 dB cable loss is entered as -5 (loss = negative gain). The SA then adds +5 dB to its raw readings.

#### Loop control (when do I go where?)

After step 7, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq, ETM) | → back to §5.4 step 1 with the next ant_i |
| All antennas done for this ETM at this freq, more freq points (fb/fm/ft) at this BW for the SAME ETM | → back to §5.3 step 1 for the next freq point (the E-TM stream from §5.2.5 step 4a is still running — don't refire) |
| All freq points done for this ETM, more ETMs at this BW (e.g. did 2.0, now do 3.1) | → back to §5.2.5 step 4a — refire rctdl with the next ETM ID. Carrier stream switches; §5.3 freq sweep restarts. |
| All ETMs done at this BW, more BWs for this band combo | → back to §5.2.5 step 1a for the next BW. Push a new startup XML, re-set SA BW preset, refire E-TM. |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band. Push a different ACTIVE_*.xml. No reboot. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1. |
| All band-groups done | STOP. Overall verdict = PASS iff every (antenna, ETM) at every (band, BW, freq) passes. |

Loose-coupling check (after one full antenna sweep at ETM 3.1): briefly disconnect the SA from one antenna and reconnect to another — EVM% should be comparable (within ±1.0% of each other). Confirms no vendor per-antenna calibration drift.

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| EVM% reads --- or NaN | Demodulator not locked | Verify §5.2.5: BW preset matches <channel-bandwidth>; CEVM mode active; carrier centered on SA; sync indicator green |
| EVM% ETM 2.0 high (>10%), ETM 3.1 OK | LO leakage or phase-noise issue at single-PRB allocation | Check PTP lock on FHGW; warm up FHGW 15+ min; verify EXM internal reference is stable |
| EVM% ETM 3.1 high (>10%), ETM 2.0 OK | PA non-linearity / compression at full-BW load | Confirm Tx power matches P_rated (cross-check via BS Output Power test); verify DlAtt / MaxAntOutputPower per §5.2 step 3a; thermal back-off — re-run after 15+ min |
| One antenna reads radically different EVM | Vendor per-antenna calibration off, or 50 Ω termination missing on another active port | Verify all other active TX ports are 50 Ω terminated; swap cable between two antennas to isolate the variable |
| EVM% flickers ±2% sweep-to-sweep | Insufficient averaging | Raise :CEVM:AVERage:COUNt to 20+; ensure FHGW E-TM stream is steady-state |
| EVM% spikes only at fb or ft (not fm) | EARFCN not actually retuned — only the SA was retuned | See fb / ft caveat in §5.3 — re-push startup XML with the target <center-of-channel-bandwidth> (treat each (BW, freq) as its own pass through §5.2.5) |
| TDD: EVM% high, channel power low | DL-slot gating disabled or wrong special-subframe config | Verify §5.2.5 step 4 ran on B41; check :GATE:DEL / :GATE:LEN match the TDD config used by L1 |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §6.5.2 Error Vector Magnitude (EVM), §6.5.2.1 Test requirements, §4.9.1 Test frequencies (fb / fm / ft), Annex C E-UTRA Test Models (E-TM 2.0, E-TM 3.1).

- TS 36.104 §6.5.2 Modulation quality (radio requirement — defines the 8.0% conformance limit per E-TM).

## 3.8 Tx — EVM OTA

3GPP references: TS 36.141 §6.5.2 (Error Vector Magnitude) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, E-TMs) live in the PW Conformance — Common Test Parameters Reference above. OTA variant — the FHGW transmits through a radiated path (chamber + positioner + reference antenna) rather than a cabled splitter. This test uses E-TM 3.1a (full-BW, 64-QAM, the 3GPP OTA variant of 3.1) — <TBD: confirm rctdl ID for E-TM 3.1a — typical value <TBD>>.

### 1. Purpose

Verify the per-antenna modulation quality of the FHGW transmitter over the air, through the deployed RFFE + antenna array. EVM (Error Vector Magnitude) measures the RMS distance between the demodulated constellation points and their ideal positions, expressed as a % of the ideal vector magnitude.

The OTA variant uses an anechoic chamber, a 3-axis positioner, and a calibrated reference horn antenna. Path-loss is captured per (port, freq, az, el) in a chamber-cal table; the SA reading is corrected by that path-loss before the EVM% verdict.

This test uses E-TM 3.1a — full-BW 64-QAM tuned for OTA radiated requirements (3GPP TS 36.141 Annex C).

### 3. Test parameters

Below are the test parameters and values for this specific test.

Table 3-1 — EVM OTA Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared TX ports for the band group (measured one at a time via positioner rotation) — see FHGW + BOB antenna mapping in the Reference | — |
| Frequency | fb / fm / ft per (band, BW) — see Frequency points in the Reference | MHz |
| Power | P_rated — per band group, see Rated output power in the Reference. Note: for OTA, the radiated EIRP differs from the conducted P_rated by the antenna gain + RFFE gain; the chamber path-loss table absorbs this. | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference for all valid options | MHz |
| E-TM | 3.1a (rctdl <TBD: rctdl ID for E-TM 3.1a>) — full-BW 64-QAM, OTA variant | — |
| Measurement | Composite EVM (CEVM) on EXM | % |

### 4. Pass/Fail criteria (PW automation gate)

Each antenna passes iff EVM_meas ≤ EVM_gate, where the PW automation gate is tighter than the 3GPP TS 36.104 §6.5.2.1 conformance limit:

| E-TM | PW gate (EVM%) | 3GPP TS 36.104 limit (EVM%) |
| --- | --- | --- |
| 3.1a (full BW, 64-QAM, OTA) | <TBD: EVM% gate from kpi_thresholds.test_Tx_EVM_OTA> | ≤ 8.0 % |

Worked example — B3, E-TM 3.1a, PW gate <TBD> (assume 6.5% for the example):

- EVM_meas = 5.1 % → 5.1 ≤ 6.5 → PASS

- EVM_meas = 7.8 % → 7.8 > 6.5 → FAIL (would still PASS the 3GPP 8.0% limit, but fails the PW automation gate)

Test verdict: PASS only if every antenna passes.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SA init,     │
  │    personality verify, positioner ping,  │
  │    load chamber path-loss table          │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  │    • set FHGW Tx power                   │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW                           │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │    • set SA BW preset, CEVM mode         │
  │    • (TDD only) DL-slot gating           │
  │    • fire rctdl <ETM_3.1a_ID> <BW>       │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SA carrier reference freq       │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna    ◄─── next antenna │  ← this test
  │    • rotate positioner to boresight      │
  │    • apply chamber path-loss             │
  │    • trigger CEVM sweep                  │
  │    • read EVM%                           │
  │    • verdict                             │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group?           │── yes ──► back to §5.2
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every antenna
                 at every (band, BW, freq) passes)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. Each BW change pushes a fresh FHGW startup XML carrying <channel-bandwidth> and <center-of-channel-bandwidth> (the EARFCN that defines fm), then re-arms the SA and fires rctdl with the OTA E-TM 3.1a.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm in Hz. | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Set the SA BW preset to match your BW. Substitute the BW token: B5M / B10M / B15M / B20M (OTA bands typically don't use the narrow BWs). | SA | :RAD:STAN:PRES B10M | Span auto-adjusts; demodulator re-locks |
| 3 | Enable DL-slot gating. TDD bands only (B41) — skip on FDD. Values for TDD config 3, special-subframe config 8. | SA | :RAD:STAN:PRES:CONF 3:RAD:STAN:PRES:SSFR 8:TRIG:SOUR:GATE:OUTP:PAT:GATE:STAT ON:GATE:DEL 3.1 ms:GATE:LEN 2.8 ms | Gated demod; only DL window analysed |
| 4 | Fire E-TM 3.1a at the configured BW. Substitute the BW in MHz at the end (5 / 10 / 15 / 20) — must match step 1a's <channel-bandwidth> and step 2's preset. The <RCTDL_3_1a> token is the rctdl ID for E-TM 3.1a: <TBD: rctdl ID — typical value <TBD>>. | testmac console | rctdl <RCTDL_3_1a> <BW_MHZ> (e.g. rctdl <TBD> 10) | Carrier appears on SA; constellation shape matches full-BW 64-QAM (3.1a) |

✓ FHGW transmitting E-TM 3.1a at this BW; SA in CEVM mode. → Continue to §5.3 for the first freq point.

#### 5.4 Per-antenna measurement (repeat for each antenna)

Do these steps once per antenna in this band's active TX set. The carrier is already up (§5.2.5 step 4) and the SA is centered (§5.3). All that's left is rotating the positioner to this antenna's boresight angle, applying the chamber path-loss for the (port, freq, angle) tuple, and reading EVM%.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Look up the boresight angle (az, el) for ant_i at the current freq from the deployment pattern data. <TBD: PW per-antenna boresight angle table> | PC | <TBD: lookup command — e.g. boresight ant_i <FREQ_HZ> > | (<AZ_DEG>, <EL_DEG>) |
| 1b | Rotate the positioner to (az, el). Wait for motion-complete acknowledgement before continuing — the antenna pattern can null EVM if the rotation hasn't settled. | positioner | <TBD: positioner SCPI/HTTP move command — e.g. MOVE AZ <AZ_DEG> EL <EL_DEG> > | Positioner reports move complete / at position; ~5–15 s settle |
| 2 | Apply the chamber path-loss offset for (ant_i, freq, az, el) to the SA. The path-loss value comes from the chamber-cal table loaded in §5.1 step 6. Enter as NEGATIVE dB (a 35 dB path loss → -35). The SA then adds +35 dB to its raw readings to refer the result back to the antenna aperture. | SA | :CORR:BTS:GAIN <-PATH_LOSS_DB> (e.g. for 35 dB chamber loss: :CORR:BTS:GAIN -35) | Ref level adjusts; carrier sits in the linear region of the digitiser |
| 3 | Auto-level the SA for this antenna. Must be after step 2 — each antenna's path-loss is different. | SA | :SENSe:POWer:RF:RANGe:OPTimize IMMediate | Trace fills upper third of screen; no IF/RF overload indicator |
| 4 | Verify the CEVM demodulator is locked (visual check on SA front panel) | SA panel | — | Sync OK green; constellation visible; no red Demod Error / Frame Sync Lost. If any fail → re-verify §5.2.5 + re-check positioner boresight. |
| 5 | Trigger a single averaged CEVM sweep | SA | :INIT:IMM*WAI | "Single sweep complete" within ~2 s |
| 6 | Read composite EVM (RMS, all subcarriers / symbols, average over the sweep) | SA | :READ:CEVM:RSEVM? (<TBD: confirm exact CEVM SCPI mnemonic for the Keysight E6640A>) | Numeric % value = EVM_meas_i (e.g. 5.1) |
| 7 | Compare against the gate from §4 | spreadsheet / notes | EVM_meas_i ≤ EVM_gate | EVM_meas_i inside window → ant_i PASS. Example: EVM_gate = 6.5%, EVM_meas_i = 5.1% → 5.1 ≤ 6.5 → PASS. |

Note — step 2 sign convention: :CORR:BTS:GAIN is an external-gain correction. A 35 dB chamber path-loss is entered as -35 (loss = negative gain).

#### Loop control (when do I go where?)

After step 7, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq) | → back to §5.4 step 1a with the next ant_i (positioner rotates to that antenna's boresight) |
| All antennas done at this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1a for the next BW |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band. No reboot. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1. |
| All band-groups done | STOP. Overall verdict = PASS iff every antenna at every (band, BW, freq) passes. |

Boresight sanity check (after one full antenna sweep): briefly rotate ±2° off boresight and confirm EVM degrades smoothly — sharp discontinuities indicate chamber multipath or a positioner-cal mismatch.

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| EVM% reads --- or NaN | Demodulator not locked, or positioner off-boresight | Verify §5.2.5: BW preset matches <channel-bandwidth>; CEVM mode active; positioner reports at position; SA sync indicator green |
| EVM% high (>10%) on every antenna | Chamber path-loss table mis-loaded, or wrong band | Re-verify §5.1 step 6 — confirm the loaded table covers the current (band, freq) range. Confirm RFFE is connected. |
| EVM% degrades after positioner moves | Positioner settle time too short, or cable flex | Add 5+ s post-move delay before §5.4 step 5; verify chamber cable routing has no kinks near the rotating axis |
| One antenna reads radically different EVM | Boresight lookup wrong for that antenna | Cross-check the boresight (az, el) against the deployment pattern data — antennas in opposite sectors point in opposite directions |
| EVM% spikes only at fb or ft (not fm) | EARFCN not actually retuned — only the SA was retuned | See fb / ft caveat in §5.3 — re-push startup XML with the target <center-of-channel-bandwidth> |
| TDD: EVM% high, channel power low | DL-slot gating disabled | Verify §5.2.5 step 3 ran on B41 |
| Reading is OK in conducted, FAIL in OTA on same FHGW | RFFE + antenna impairment | EVM_OTA − EVM_conducted = RFFE + antenna contribution. Compare against vendor RFFE EVM contribution spec. |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §6.5.2 Error Vector Magnitude (EVM), §6.5.2.1 Test requirements, §4.9.1 Test frequencies (fb / fm / ft), Annex C E-UTRA Test Models (E-TM 3.1a), Annex M OTA test methods.

- TS 36.104 §6.5.2 Modulation quality (radio requirement — defines the 8.0% conformance limit).

## 3.9 Tx — Frequency Error

3GPP references: TS 36.141 §6.5.1 (Frequency Error) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, E-TMs) live in the PW Conformance — Common Test Parameters Reference above. This test uses two E-TMs: E-TM 2.0 and E-TM 3.1 — each ETM is measured as its own sweep through §5.3 → §5.4.

### 1. Purpose

Verify the per-antenna carrier frequency accuracy of the FHGW transmitter. The FHGW is PTP-locked to the LLS-C1 grandmaster; the EXM demodulates the carrier and reports the residual offset (in Hz) between the recovered RF center and the configured EARFCN. Two E-TMs are required:

- E-TM 2.0 — single-PRB, 64-QAM. The narrow allocation makes carrier-tracking errors more visible.

- E-TM 3.1 — full-BW, 64-QAM. Confirms freq accuracy under full PA load.

Conducted-RF, low-power test — output flows through a passive cable chain into the EXM.

### 3. Test parameters

Below are the test parameters and values for this specific test.

Table 3-1 — Frequency Error Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared TX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Frequency | fb / fm / ft per (band, BW) — see Frequency points in the Reference | MHz |
| Power | P_rated — per band group, see Rated output power in the Reference | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference for all valid options | MHz |
| E-TM | 2.0 (rctdl 6113) — single-PRB 64-QAM3.1 (rctdl 6114) — full-BW 64-QAM | — |
| Measurement | Carrier frequency offset (CEVM mode reports this alongside EVM) | Hz |

### 4. Pass/Fail criteria (PW automation gate)

Each (antenna, ETM) passes iff |f_err_meas| ≤ f_err_gate, where:

| Band class | PW automation gate | Effective limit at fm |
| --- | --- | --- |
| FDD bands (B1, B3, B4, B7, B8, B20, B28) | <TBD: kpi_thresholds.test_Tx_FrequencyError.limit_fdd> — typically ±0.05 ppm × fm_Hz + ±12 Hz uncertainty | e.g. B3 / fm = 1842.5 MHz → ±0.05 ppm = ±92 Hz; with uncertainty, gate ≈ ±104 Hz |
| TDD bands (B41) | <TBD: kpi_thresholds.test_Tx_FrequencyError.limit_tdd> — typically ±0.1 ppm × fm_Hz + ±12 Hz uncertainty | e.g. B41 / fm = 2593 MHz → ±0.1 ppm = ±259 Hz; with uncertainty, gate ≈ ±271 Hz |

The 3GPP TS 36.104 §6.5.1 conformance limits are ±0.05 ppm (FDD wide-area BS) / ±0.1 ppm (TDD wide-area BS). PW's automation gate is the 3GPP limit plus the EXM's documented frequency measurement uncertainty of ±12 Hz at typical warm-up. Tighter benches (GPSDO-locked) may use a stricter gate.

Worked example — B3 / fm = 1842.5 MHz, E-TM 3.1, PW gate ±104 Hz:

- f_err_meas = +43 Hz → |43| = 43 ≤ 104 → PASS

- f_err_meas = −187 Hz → |187| = 187 > 104 → FAIL

Test verdict: PASS only if every (antenna, ETM) combination passes.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test. Frequency Error runs §5.4 twice per (band, BW, freq) — once at E-TM 2.0, once at E-TM 3.1.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SA init,     │
  │    personality verify, RF switch ping,   │
  │    cable-loss cal per (port, freq)       │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  │    • set FHGW Tx power                   │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW + per ETM                 │  ← this test
  │    For ETM in {2.0, 3.1}:                │
  │    • edit + push oru_startup_slave.xml   │
  │    • set SA BW preset, CEVM mode         │
  │    • (TDD only) DL-slot gating           │
  │    • fire rctdl <ID> <BW>                │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SA carrier reference freq       │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna    ◄─── next antenna │  ← this test
  │    • route RF switch                     │
  │    • apply cable loss                    │
  │    • trigger CEVM sweep                  │
  │    • read freq error (Hz)                │
  │    • verdict vs band-class gate          │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more ETMs to run at this BW?             │── yes ──► back to §5.2.5
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group?           │── yes ──► back to §5.2
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every (antenna, ETM)
                 at every (band, BW, freq) passes)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW + per-ETM setup (repeat for each BW, then for each ETM at that BW)

Frequency Error requires two ETMs (E-TM 2.0 and E-TM 3.1) per (band, BW). Inner loop: fix BW, sweep ETM. Outer loop: next BW.

For the current BW × ETM combination:

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm in Hz. Only re-do this on the first ETM at a new BW. | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW. First-ETM-at-this-BW only — skip on the second ETM at the same BW. | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Switch the SA from Channel Power mode to Composite EVM (CEVM) mode — this is where frequency error is reported | SA | :CONF:CEVM:INIT:CONT ON:CEVM:AVERage:STATe ON:CEVM:AVERage:COUNt 10 | EXM screen shows constellation + EVM% + freq error readout |
| 3 | Set the SA BW preset to match your BW. Substitute the BW token: B1M4 / B3M / B5M / B10M / B15M / B20M. | SA | :RAD:STAN:PRES B10M | Span auto-adjusts; demodulator re-locks |
| 4 | Enable DL-slot gating. TDD bands only (B41) — skip on FDD. Values are for TDD config 3, special-subframe config 8. | SA | :RAD:STAN:PRES:CONF 3:RAD:STAN:PRES:SSFR 8:TRIG:SOUR:GATE:OUTP:PAT:GATE:STAT ON:GATE:DEL 3.1 ms:GATE:LEN 2.8 ms | Gated demod; only DL window analysed |
| 4a | Fire the current ETM at the configured BW. First pass: ETM 2.0 → rctdl 6113 <BW>. Second pass: ETM 3.1 → rctdl 6114 <BW>. The <BW> token is the BW in MHz at the end (1.4 / 3 / 5 / 10 / 15 / 20) and must match step 1a + step 3. | testmac console | rctdl <RCTDL_ID> <BW_MHZ> (e.g. rctdl 6113 10) | Carrier appears on SA; constellation locked; freq-error readout starts updating |

✓ FHGW transmitting current E-TM at this BW; SA in CEVM mode. → Continue to §5.3 for the first freq point. When all freq points + antennas are done for this ETM, return here and refire rctdl with the next ETM.

#### 5.4 Per-antenna measurement (repeat for each antenna, per ETM)

Do these steps once per antenna per ETM at the current (band, BW, freq). The carrier is already up at the current ETM (§5.2.5 step 4a) and the SA is in CEVM mode (§5.2.5 step 2). All that's left is routing the right antenna, triggering a sweep, and reading the carrier frequency offset.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RF switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 (HTTP body) |
| 2 | Apply ant_i's calibrated cable-loss offset to the SA. The loss value comes from §5.1 steps 6–10 for the current freq. Enter as NEGATIVE dB. Cable loss does not affect frequency error directly, but the SA still needs the right ref level to keep the demodulator in the linear region. | SA | :CORR:BTS:GAIN <-CABLE_LOSS_DB> (e.g. for 5 dB loss: :CORR:BTS:GAIN -5) | Ref level adjusts |
| 3 | Auto-level the SA for this antenna. Must be after step 2. | SA | :SENSe:POWer:RF:RANGe:OPTimize IMMediate | Trace fills upper third of screen; no IF/RF overload indicator |
| 4 | Verify the CEVM demodulator is locked (visual check on SA front panel) | SA panel | — | Sync OK green; freq-error display showing a stable value. If Demod Error or Frame Sync Lost → re-verify §5.2.5. |
| 5 | Trigger a single averaged CEVM sweep | SA | :INIT:IMM*WAI | "Single sweep complete" within ~2 s |
| 6 | Read the carrier frequency error | SA | :READ:CEVM:CARRier:FERRor? (<TBD: confirm exact SCPI mnemonic for the Keysight E6640A — alternates:* :FETCh:CEVM:FERRor?*,* :READ:CEVM[1]? *with field index for freq error>) | Numeric Hz value = f_err_meas_i (e.g. +43) |
| 7 | Compare against the band-class gate from §4 | spreadsheet / notes | ` | f_err_meas_i |

#### Loop control (when do I go where?)

After step 7, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq, ETM) | → back to §5.4 step 1 with the next ant_i |
| All antennas done for this ETM, more freq points for the SAME ETM | → back to §5.3 step 1 for the next freq point |
| All freq points done for this ETM, more ETMs at this BW (e.g. did 2.0, now do 3.1) | → back to §5.2.5 step 4a — refire rctdl with the next ETM ID |
| All ETMs done at this BW, more BWs for this band combo | → back to §5.2.5 step 1a for the next BW |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band. No reboot. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1. |
| All band-groups done | STOP. Overall verdict = PASS iff every (antenna, ETM) at every (band, BW, freq) passes. |

PTP sanity check (after one full antenna sweep): on the FHGW shell, confirm PTP is still LOCKED (pmc -u -b 0 'GET PORT_DATA_SET'). If the FHGW slipped out of PTP lock mid-test, every measurement after the slip is suspect — re-run the affected (band, BW, freq) tuples.

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| Freq error reads --- or NaN | Demodulator not locked | Verify §5.2.5: BW preset matches <channel-bandwidth>; CEVM mode active; carrier centered on SA |
| Freq error consistent +/− N Hz across all antennas of the same band | EXM internal reference offset | Warm up EXM ≥ 30 min; if still offset, check against a 10 MHz GPSDO or 1 PPS source |
| Freq error drifts > 50 Hz over a minute | FHGW PTP lock unstable, or oscillator warm-up | Check PTP LOCKED state on FHGW; warm up FHGW 15+ min; check grandmaster status |
| Freq error wildly different on one antenna (others OK) | Per-port LO leak / image, not actual carrier offset | Confirm the demodulator is locked on the carrier, not on a spur; raise SA ref level by 5 dB |
| Freq error larger at fb / ft than at fm | EARFCN not actually retuned for fb / ft — measuring at fm with the SA pointing at fb / ft | See fb / ft caveat in §5.3 — re-push startup XML with the target <center-of-channel-bandwidth> |
| Freq error spec violation only at one band, others OK | Per-band LO multiplier issue | Compare against EVM (same test path) — if EVM is also bad, the band's LO chain is the issue |
| TDD: freq error reads NaN | DL-slot gating disabled or wrong special-subframe config | Verify §5.2.5 step 4 ran on B41 |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §6.5.1 Frequency Error, §6.5.1.4 Test requirements, §4.9.1 Test frequencies (fb / fm / ft), Annex C E-UTRA Test Models (E-TM 2.0, E-TM 3.1).

- TS 36.104 §6.5.1 Frequency error (radio requirement — ±0.05 ppm FDD wide-area / ±0.1 ppm TDD wide-area BS).

## 3.10 Tx — Time Alignment Error

3GPP references: TS 36.141 §6.5.6 (conformance — Time Alignment Error between Tx antennas) + TS 36.104 §6.5.3 (radio requirement) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, E-TMs) live in the PW Conformance — Common Test Parameters Reference above. This test uses E-TM 1.1 (rctdl 6111) and is inter-antenna — it does not produce a per-antenna verdict like power tests, but a per-pair (or full-set) max-delta verdict.

### 1. Purpose

Verify that the FHGW transmits all antennas of the same cell time-aligned within the 3GPP intra-band contiguous limit (≤ 65 ns max pairwise skew per TS 36.141 §6.5.6.2). Drift between TX antennas degrades MIMO precoding, beamforming, and inter-cell carrier aggregation. Unlike BS Output Power and DLRS — which produce one verdict per antenna — Time Alignment produces one verdict per measurement set: the largest pairwise delta across all measured antennas of the cell must stay under the limit. Conducted-RF, low-power test — output flows through a passive cable chain into the EXM. No chamber, no OTA.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / E-TM tables.

Table 3-1 — Time Alignment Error Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared TX ports for the band group (sampled one at a time, then compared pairwise) — see FHGW + BOB antenna mapping in the Reference. Minimum of 2 antennas needed for a pairwise delta. | — |
| Frequency | fb / fm / ft per (band, BW) — see Frequency points in the Reference | MHz |
| Power | P_rated — see Rated output power in the Reference (informational — power level doesn't affect the time-offset readout, but the FHGW is set to its nominal P_rated for a real-traffic-like burst) | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference for all valid options | MHz |
| E-TM | 1.1 (rctdl 6111) | — |

### 4. Pass/Fail criteria (PW automation gate)

This test produces one verdict per measurement set, not per antenna. After sampling Time Offset T_i for each antenna i of the active TX set, compute the maximum pairwise delta:

Δ_max = max( |T_i − T_j| )  for all (i, j), i ≠ j

PASS iff Δ_max ≤ Δ_limit, where:

- Δ_limit = <TBD: PW gate from kpi_thresholds.test_Tx_TimeAlignment — likely 65 ns or tighter> (source survey note: "Hardcoded <65 nsec inter-antenna delta").

- 3GPP TS 36.141 §6.5.6.2 sets ≤ 65 ns for intra-band contiguous Tx (the case that applies to all FHGW band groups here). Inter-band CA cases use looser limits — not in scope for this manual procedure.

Worked example — B3 at 10 MHz, 4 active TX antennas (b0103 B3 → Tx3, Tx2, Tx7, Tx6):

| Antenna | T_i (ns) |
| --- | --- |
| Tx3 | 120 |
| Tx2 | 145 |
| Tx7 | 132 |
| Tx6 | 128 |

Pairwise deltas:

|145 − 120| = 25
|132 − 120| = 12
|128 − 120| =  8
|145 − 132| = 13
|145 − 128| = 17
|132 − 128| =  4

Δ_max = 25 ns → 25 ≤ 65 → PASS.

If instead T_2 = 200 ns, then Δ_max = |200 − 120| = 80 ns → 80 > 65 → FAIL.

Test verdict: PASS iff Δ_max ≤ Δ_limit at every (band, BW, freq) combination tested.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SA init,     │
  │    personality verify, RF switch ping,   │
  │    cable-loss cal per (port, freq)       │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  │    • set FHGW Tx power                   │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │      (channel-bandwidth + EARFCN)        │
  │    • switch SA to CEVM mode              │
  │    • set SA BW preset                    │
  │    • (TDD only) DL-slot gating           │
  │    • fire rctdl 6111 <BW>                │
  │      (E-TM stream stays running across   │
  │       all freqs at this BW)              │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SA carrier reference freq       │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna set (one verdict)    │  ← this test
  │    • per antenna i: route, cable, CEVM,  │
  │      read T_i                            │
  │    • compute Δ_max = max |T_i − T_j|     │
  │    • single PASS/FAIL for the set        │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     │  (rctdl keeps running)          │
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     │  (e.g. 5 → 10 → 20 MHz at this band)     │           push new startup XML,
     │  Same activated band — no ACTIVE_*.xml.  │           re-set SA BW preset, refire rctdl
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group to test?   │── yes ──► back to §5.2
     │  (e.g. did B1 in b0103, now want B3)     │           push different ACTIVE_*.xml,
     │  Same booted personality — no reboot.    │           re-set Tx power
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     │  (e.g. did all of b0103, now want b0307) │           (see "Update FHGW EEPROM
     │  Different personality — REBOOT needed.  │           model name" appendix),
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff Δ_max ≤ Δ_limit at
                 every (band, BW, freq) combination)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. Each BW change pushes a fresh FHGW startup XML carrying <channel-bandwidth> and <center-of-channel-bandwidth> (the EARFCN that defines fm), then switches the SA to CEVM mode and fires rctdl. The Time Offset readout is a CEVM output — :CONF:CHPower is replaced by :CONF:CEVM here.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm in Hz (from the umbrella's Frequency points formula). | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2a | Switch the SA to CEVM (Cell EVM) measurement mode. Time Offset is a CEVM output, not a channel-power output — this replaces the §5.1 step 4 :CONF:CHPower setup for this test. | SA | :CONF:CEVM:INIT:CONT ON:CEVM:AVERage:STATe ON:CEVM:AVERage:COUNt 20 | CEVM result table appears; Time Offset row populated |
| 2b | Set the SA BW preset to match your BW. Substitute the BW token: B1M4 / B3M / B5M / B10M / B15M / B20M. | SA | :RAD:STAN:PRES B10M | Span auto-adjusts |
| 3 | Enable DL-slot gating. TDD bands only (B41) — skip on FDD. Values are for TDD config 3, special-subframe config 8; for other configs derive gate delay/length from TS 36.211 §4. | SA | :RAD:STAN:PRES:CONF 3:RAD:STAN:PRES:SSFR 8:TRIG:SOUR:GATE:OUTP:PAT:GATE:STAT ON:GATE:VIEW:STAT ON:GATE:VIEW:SWE 20 ms:GATE:DEL 3.1 ms:GATE:LEN 2.8 ms | Gated sweep; only DL window integrated |
| 4 | Fire E-TM 1.1 at the configured BW. Substitute the BW in MHz at the end (1.4 / 3 / 5 / 10 / 15 / 20) — must match step 1a's <channel-bandwidth> and step 2b's preset. | testmac console | rctdl 6111 10 | Carrier appears on SA; CEVM Time Offset row populates within ~3 s. The E-TM stream stays running for all freq points below — do not refire between freqs. |

✓ FHGW transmitting E-TM 1.1 at this BW; SA in CEVM mode. → Continue to §5.3 for the first freq point.

#### 5.4 Per-antenna-set measurement (repeat once per freq point — one verdict per pass)

Do these steps once per freq point. Unlike per-antenna power tests, Time Alignment sweeps every antenna in the active TX set in a single pass, then computes one max-delta verdict at the end. The carrier is already up (§5.2.5 step 4) and the SA is centered (§5.3). The procedure samples each antenna sequentially, records T_i, and at the end compares pairwise.

Antenna count varies per band combo: 4-antenna combos (b0103, b082028, b41) need steps 1–8; 2-antenna combos (b0307, b0407) need only steps 1–4 (skip the antenna 3 + 4 pairs).

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Route the RF switch for antenna 1. Substitute <BANK> and <PORT> from your setup's RF switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 (HTTP body) |
| 1b | Apply antenna 1's calibrated cable-loss offset to the SA. Cable loss doesn't affect time-offset accuracy, but applying it keeps the trace at a sensible ref-level for CEVM sync. Enter as NEGATIVE dB. | SA | :CORR:BTS:GAIN <-CABLE_LOSS_DB> (e.g. :CORR:BTS:GAIN -5) | SA trace shows carrier cable-loss compensated |
| 1c | Trigger a CEVM sweep and read Time Offset for antenna 1 | SA | :INIT:IMM*WAI<TBD: exact SCPI for time-offset readout> (working name: :READ:CEVM:TIME:OFFS?; the Keysight LTE CEVM measurement returns Time Offset as one of its sub-results — confirm the exact mnemonic against your firmware's SCPI reference) | Numeric value = T_1 in seconds (or ns, depending on SCPI return format). Record. |
| 2a | Route the RF switch for antenna 2 | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 |
| 2b | Apply antenna 2's cable-loss offset | SA | :CORR:BTS:GAIN <-CABLE_LOSS_DB> | Trace cable-loss compensated |
| 2c | Trigger CEVM and read Time Offset for antenna 2 | SA | :INIT:IMM*WAI<TBD: exact SCPI for time-offset readout> | T_2 recorded |
| 3a | Route the RF switch for antenna 3. Skip for 2-antenna combos (b0307, b0407) — go straight to step 9. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 |
| 3b | Apply antenna 3's cable-loss offset | SA | :CORR:BTS:GAIN <-CABLE_LOSS_DB> | Trace cable-loss compensated |
| 3c | Trigger CEVM and read Time Offset for antenna 3 | SA | :INIT:IMM*WAI<TBD: exact SCPI for time-offset readout> | T_3 recorded |
| 4a | Route the RF switch for antenna 4 | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 |
| 4b | Apply antenna 4's cable-loss offset | SA | :CORR:BTS:GAIN <-CABLE_LOSS_DB> | Trace cable-loss compensated |
| 4c | Trigger CEVM and read Time Offset for antenna 4 | SA | :INIT:IMM*WAI<TBD: exact SCPI for time-offset readout> | T_4 recorded |
| 9 | Compute the max pairwise delta across all measured T_i | spreadsheet / notes | `Δ_max = max( | T_i − T_j |
| 10 | Compare against the pass criterion from §4 | spreadsheet / notes | Δ_max ≤ Δ_limit where Δ_limit = <TBD: PW gate> (3GPP cap: 65 ns) | Δ_max ≤ Δ_limit → PASS for this (band, BW, freq). Example: T_1=120, T_2=145, T_3=132, T_4=128 ns → Δ_max = 25 ns → 25 ≤ 65 → PASS. |

Note — step Nb sign convention: :CORR:BTS:GAIN is an external-gain correction. A 5 dB cable loss is entered as -5 (loss = negative gain). The SA then adds +5 dB to its raw readings to refer the result back to the FHGW port.

Note — Time Offset units: the SCPI return may be in seconds (e.g. 1.20E-07 = 120 ns). Convert to ns before computing deltas; an off-by-1000 error here silently makes everything appear to pass.

#### Loop control (when do I go where?)

After step 10 (one verdict for this (band, BW, freq)), decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point (the E-TM stream from §5.2.5 step 4 is still running — don't refire) |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW (e.g. 10 → 20 MHz at B3). Push a new startup XML, re-set SA BW preset, refire E-TM. No ACTIVE_*.xml change — same activated band. |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band (e.g. B1 → B3 in b0103). Push a different ACTIVE_*.xml, re-set Tx power. No reboot — same booted personality. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 for the first band in the new group. |
| All band-groups done | STOP. Overall verdict for the test = PASS iff Δ_max ≤ Δ_limit at every (band, BW, freq) tested. |

Cable-symmetry check (do this once before the first measurement, then again if any antenna's T_i is suspiciously high): unplug the cable from antenna i's splitter port and from antenna j's splitter port and swap them. Re-run steps i and j. If the per-antenna T_i values swap with the cables (e.g. originally Tx2=145 ns / Tx3=120 ns, after swap Tx2=120 ns / Tx3=145 ns), the offset is cable-borne, not FHGW-borne. Re-cut or re-pick cables and re-measure.

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| One antenna T_i is 5 ns+ off from the others, others tightly grouped | Cable length mismatch on that antenna | Run the cable-symmetry check above; replace the offender with a length-matched cable |
| All antennas T_i drift together over time | PHY hasn't fully settled, or PTP holdover is being chased | Wait an extra 30 s after §5.3 step 2; verify PTP lock on FHGW (pmc -u -b 0 'GET CURRENT_DATA_SET') |
| Δ_max huge (> 200 ns) and erratic between sweeps | CEVM is not sync'd to the cell — Time Offset reading is meaningless without sync | Confirm Sync OK green on SA; re-set §5.2.5 step 2b BW preset; re-trigger; if still failing, refire rctdl |
| 2-antenna combo (b0307 / b0407) gives only one T_i | Only steps 1–2 needed for 2-antenna combos. `Δ_max = | T_1 − T_2 |
| Time Offset returned in seconds, not ns | SCPI default unit | Convert: T_ns = T_s × 1e9. A return of 1.20E-07 is 120 ns. |
| Reading consistent on FDD but jumps wildly on TDD (B41) | DL-slot gating not enabled — CEVM is sampling UL guard | Verify §5.2.5 step 3 (TDD gating) is applied; re-trigger |
| All T_i zero or -- in the CEVM result table | EARFCN mismatch — SA isn't decoding the cell | Re-verify §5.2.5 step 1 (FHGW <center-of-channel-bandwidth>) matches §5.3 step 1 (SA :CCARrier:REFerence) |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §6.5.6 Time alignment between transmitter branches, §6.5.6.2 Test requirements (≤ 65 ns intra-band contiguous), §4.9.1 Test frequencies (fb / fm / ft), Annex C E-UTRA Test Models (E-TM 1.1).

- TS 36.104 §6.5.3 Time alignment error (radio requirement — the underlying ≤ 65 ns Tx-antenna timing budget that 36.141 verifies).

- TS 36.211 §6.10 Reference signals (CRS used by the CEVM Time Offset estimator as the timing reference).

## 3.11 Tx — Modulation Analysis

3GPP references: TS 36.141 §6.5.1 (Frequency Error) + §6.5.2 (EVM) + §6.2 (BS Output Power) + §6.5.2.2 (DLRS Power) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, E-TMs) live in the PW Conformance — Common Test Parameters Reference above. This test uses three E-TMs: E-TM 1.1, E-TM 2a, and E-TM 3.1a — composite measurement returns EVM + channel power + frequency error + DLRS power in a single CEVM sweep.

### 1. Purpose

Composite modulation-quality test. A single CEVM sweep returns four sub-measurements at once:

- EVM% — modulation quality (vs TS 36.104 §6.5.2 limits).

- Channel power (dBm) — vs P_rated (BS Output Power, TS 36.104 §6.2).

- Frequency error (Hz) — vs ±0.05 ppm / ±0.1 ppm (TS 36.104 §6.5.1).

- DLRS power (dBm) — Reference Signal power per port, vs configured P_rated − 10·log10(N_RB·12) reference (TS 36.104 §6.5.2.2).

Three E-TMs are required:

- E-TM 1.1 — full-BW QPSK. Best stream for accurate channel power + DLRS power readouts (low PAPR, high integration confidence).

- E-TM 2a — narrow-PRB 64-QAM. Stresses freq accuracy + single-PRB EVM.

- E-TM 3.1a — full-BW 64-QAM. Stresses PA load EVM + ACLR-adjacent measurement (this test doesn't run ACLR, but uses the same stream).

Conducted-RF, low-power test. The composite test is the roll-up that PW automation uses to verify the FHGW Tx chain in one pass — it is not a replacement for the individual tests (Tx BS Output Power, Tx EVM, Tx Frequency Error) when a deeper component-specific debug is needed.

### 3. Test parameters

Below are the test parameters and values for this specific test.

Table 3-1 — Modulation Analysis Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared TX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Frequency | fb / fm / ft per (band, BW) — see Frequency points in the Reference | MHz |
| Power | P_rated — per band group, see Rated output power in the Reference | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference for all valid options | MHz |
| E-TM | 1.1 (rctdl 6111) — full-BW QPSK2a (rctdl <TBD: rctdl ID for E-TM 2a — typical value <TBD>>) — narrow-PRB 64-QAM3.1a (rctdl <TBD: rctdl ID for E-TM 3.1a — typical value <TBD>>) — full-BW 64-QAM | — |
| Measurement | Composite EVM (CEVM) — returns EVM%, channel power, freq error, DLRS power | %, dBm, Hz, dBm |

### 4. Pass/Fail criteria (PW automation gate)

Composite verdict. Each (antenna, ETM) sweep produces four sub-results; all four must pass for that (antenna, ETM) to pass. Gates:

| Sub-measurement | PW automation gate | Reference 3GPP limit |
| --- | --- | --- |
| EVM% | <TBD: per-ETM EVM% gate from kpi_thresholds.test_Tx_modulation_analysis.evm> | TS 36.104 §6.5.2 — 64-QAM ≤8%, QPSK ≤17.5% |
| Channel power | ` | P_meas − P_rated |
| Freq error | <TBD: kpi_thresholds.test_Tx_modulation_analysis.freq_err> — typically ±0.05 ppm FDD / ±0.1 ppm TDD × fm_Hz + ±12 Hz uncertainty | TS 36.104 §6.5.1 — ±0.05 ppm FDD / ±0.1 ppm TDD wide-area BS |
| DLRS power | ` | DLRS_meas − DLRS_expected |

Worked example — B3 / fm = 1842.5 MHz, BW = 10 MHz (50 PRB → 600 DLRS subcarriers), P_rated = −15.7 dBm, E-TM 3.1a:

- EVM_meas = 4.2 %, gate <TBD> (assume 6.0%) → 4.2 ≤ 6.0 → PASS

- P_meas = −15.3 dBm, gate ±1.5 → |−15.3 − (−15.7)| = 0.4 → PASS

- f_err_meas = +43 Hz, gate ±104 Hz → 43 ≤ 104 → PASS

- DLRS_meas = −43.5 dBm, expected = −15.7 − 10·log10(600) = −15.7 − 27.8 = −43.5 dBm, gate ±3.0 → 0.0 → PASS

- Composite verdict: ALL FOUR PASS → (ant_i, E-TM 3.1a) PASS

If any of the four sub-measurements fails, the (antenna, ETM) FAILS. Test verdict: PASS only if every (antenna, ETM) combination passes all four sub-measurements.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test. Modulation Analysis runs §5.4 three times per (band, BW, freq) — once per ETM.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SA init,     │
  │    personality verify, RF switch ping,   │
  │    cable-loss cal per (port, freq)       │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  │    • set FHGW Tx power                   │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW + per ETM                 │  ← this test
  │    For ETM in {1.1, 2a, 3.1a}:           │
  │    • edit + push oru_startup_slave.xml   │
  │    • set SA BW preset, CEVM mode         │
  │    • (TDD only) DL-slot gating           │
  │    • fire rctdl <ID> <BW>                │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SA carrier reference freq       │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna    ◄─── next antenna │  ← this test
  │    • route RF switch                     │
  │    • apply cable loss                    │
  │    • trigger CEVM (single sweep)         │
  │    • read EVM% / P / f_err / DLRS        │
  │    • verdict on all four; composite      │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more ETMs to run at this BW?             │── yes ──► back to §5.2.5
     │  (do 1.1 → 2a → 3.1a)                    │           refire rctdl with next ETM
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group?           │── yes ──► back to §5.2
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every (antenna, ETM)
                 at every (band, BW, freq) passes
                 all four sub-measurements)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW + per-ETM setup (repeat for each BW, then for each ETM at that BW)

Modulation Analysis requires three ETMs (E-TM 1.1, E-TM 2a, E-TM 3.1a) per (band, BW). Inner loop: fix BW, sweep ETM. Outer loop: next BW.

For the current BW × ETM combination:

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm in Hz. Only re-do this on the first ETM at a new BW. | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW. First-ETM-at-this-BW only — skip on 2nd / 3rd ETMs at the same BW. | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Switch the SA from Channel Power mode to Composite EVM (CEVM) mode — composite measurement returns all four sub-results | SA | :CONF:CEVM:INIT:CONT ON:CEVM:AVERage:STATe ON:CEVM:AVERage:COUNt 10 | EXM screen shows constellation, EVM%, channel power, freq error, DLRS power readouts |
| 3 | Set the SA BW preset to match your BW. Substitute the BW token: B1M4 / B3M / B5M / B10M / B15M / B20M. | SA | :RAD:STAN:PRES B10M | Span auto-adjusts; demodulator re-locks |
| 4 | Enable DL-slot gating. TDD bands only (B41) — skip on FDD. Values for TDD config 3, special-subframe config 8. | SA | :RAD:STAN:PRES:CONF 3:RAD:STAN:PRES:SSFR 8:TRIG:SOUR:GATE:OUTP:PAT:GATE:STAT ON:GATE:DEL 3.1 ms:GATE:LEN 2.8 ms | Gated demod; only DL window analysed |
| 4a | Fire the current ETM at the configured BW. First pass: ETM 1.1 → rctdl 6111 <BW>. Second pass: ETM 2a → rctdl <TBD: rctdl ID for 2a> <BW>. Third pass: ETM 3.1a → rctdl <TBD: rctdl ID for 3.1a> <BW>. The <BW> token must match step 1a + step 3. | testmac console | rctdl <RCTDL_ID> <BW_MHZ> (e.g. rctdl 6111 10) | Carrier appears on SA; constellation shape matches expected modulation (QPSK for 1.1, single-PRB 64-QAM for 2a, full-BW 64-QAM for 3.1a); all four CEVM readouts populate |

✓ FHGW transmitting current E-TM at this BW; SA in CEVM mode. → Continue to §5.3 for the first freq point. When all freq points + antennas are done for this ETM, return here and refire rctdl with the next ETM.

#### 5.4 Per-antenna measurement (repeat for each antenna, per ETM)

Do these steps once per antenna per ETM at the current (band, BW, freq). The carrier is already up at the current ETM (§5.2.5 step 4a) and the SA is in CEVM mode. All that's left is routing the right antenna, triggering ONE sweep, and reading all four sub-results.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RF switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 (HTTP body) |
| 2 | Apply ant_i's calibrated cable-loss offset to the SA. The loss value comes from §5.1 steps 6–10 for the current freq. Enter as NEGATIVE dB (a 5 dB cable loss → -5). This correction applies to both channel power and DLRS power readouts. | SA | :CORR:BTS:GAIN <-CABLE_LOSS_DB> (e.g. for 5 dB loss: :CORR:BTS:GAIN -5) | SA trace shows carrier at ~P_rated (cable-loss compensated) |
| 3 | Auto-level the SA for this antenna. Must be after step 2. | SA | :SENSe:POWer:RF:RANGe:OPTimize IMMediate | Trace fills upper third of screen; no IF/RF overload indicator |
| 4 | Verify the CEVM demodulator is locked (visual check on SA front panel) | SA panel | — | Sync OK green; all four readouts (EVM, power, freq err, DLRS) populated; no red Demod Error / Frame Sync Lost. If any fail → re-verify §5.2.5. |
| 5 | Trigger a single averaged CEVM sweep. Important: don't trigger separately for each sub-measurement — all four readouts must come from the same sweep window to be self-consistent. | SA | :INIT:IMM*WAI | "Single sweep complete" within ~2 s |
| 6a | Read composite EVM | SA | :READ:CEVM:RSEVM? (<TBD: confirm exact CEVM SCPI mnemonic for the Keysight E6640A>) | Numeric % value = EVM_meas_i (e.g. 4.2) |
| 6b | Read channel power | SA | :READ:CEVM:RSTPower? (<TBD: confirm exact channel-power CEVM SCPI mnemonic — alternates:* :FETCh:CEVM:POWer?*,* :READ:CEVM:TPOWer?*>) | Numeric dBm value = P_meas_i (cable-loss corrected, e.g. −15.3) |
| 6c | Read carrier frequency error | SA | :READ:CEVM:CARRier:FERRor? (<TBD: confirm exact freq-err CEVM SCPI mnemonic for the Keysight E6640A>) | Numeric Hz value = f_err_meas_i (e.g. +43) |
| 6d | Read DLRS (Downlink Reference Signal) power | SA | :READ:CEVM:DLRS:POWer? (<TBD: confirm exact DLRS SCPI mnemonic for the Keysight E6640A>) | Numeric dBm value = DLRS_meas_i (cable-loss corrected, e.g. −43.5) |
| 7a | Verify EVM sub-result against §4 gate for the current ETM | spreadsheet / notes | EVM_meas_i ≤ EVM_gate[<CURRENT_ETM>] | EVM sub-verdict = PASS / FAIL |
| 7b | Verify channel-power sub-result | spreadsheet / notes | ` | P_meas_i − P_rated |
| 7c | Verify freq-error sub-result | spreadsheet / notes | ` | f_err_meas_i |
| 7d | Verify DLRS sub-result. Compute DLRS_expected = P_rated − 10·log10(N_RB · 12) where N_RB is the PRB count for this BW (e.g. 10 MHz → 50 PRB → 600 subcarriers). | spreadsheet / notes | ` | DLRS_meas_i − DLRS_expected |
| 8 | Composite verdict for (ant_i, current ETM) | spreadsheet / notes | All four sub-verdicts must be PASS | If all PASS → (ant_i, current ETM) PASS; otherwise FAIL (record which sub-measurement(s) failed) |

Note — step 2 sign convention: :CORR:BTS:GAIN is an external-gain correction. A 5 dB cable loss is entered as -5. The SA then adds +5 dB to its raw readings — this applies to both channel power (6b) and DLRS power (6d). EVM (6a) and freq error (6c) are independent of cable loss.

#### Loop control (when do I go where?)

After step 8, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq, ETM) | → back to §5.4 step 1 with the next ant_i |
| All antennas done for this ETM, more freq points for the SAME ETM | → back to §5.3 step 1 for the next freq point |
| All freq points done for this ETM, more ETMs at this BW (e.g. did 1.1, now do 2a; or did 2a, now do 3.1a) | → back to §5.2.5 step 4a — refire rctdl with the next ETM ID |
| All three ETMs done at this BW, more BWs for this band combo | → back to §5.2.5 step 1a for the next BW |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band. No reboot. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1. |
| All band-groups done | STOP. Overall verdict = PASS iff every (antenna, ETM) at every (band, BW, freq) passes all four sub-measurements. |

Composite sanity check (after one full antenna sweep): channel power should match the standalone Tx BS Output Power test result (run earlier with E-TM 1.1) within ±0.3 dB. EVM should match the standalone Tx EVM test within ±0.5%. If any cross-test deltas exceed those, suspect a cable/cal change between runs.

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| One sub-result reads --- while others are OK | That sub-measurement's specific demod path failed; rest of CEVM sweep is OK | EVM --- → constellation not locked (check SA sync); DLRS --- → DLRS subcarriers not found (check BW preset + cell-ID); freq err --- → too far off — increase capture window |
| All four sub-results read --- or NaN | CEVM demodulator not locked at all | Verify §5.2.5: BW preset matches <channel-bandwidth>; CEVM mode active; carrier centered on SA; sync indicator green |
| Channel power reads ~10 dB low; EVM + freq err OK | Cable-loss correction not applied (step 2) | Verify §5.4 step 2 ran with the correct <CABLE_LOSS_DB> for the current (port, freq) |
| DLRS power reads ~30 dB low (close to expected if math is right) | This is normal — DLRS expected is P_rated − 10·log10(N_RB · 12) (e.g. for 10 MHz: 27.8 dB below P_rated). Not a fault. | Verify the DLRS_expected formula in step 7d; cross-check the BW's PRB count |
| EVM high only on ETM 2a; ETM 1.1 + 3.1a OK | Phase noise / LO leakage at single-PRB allocation | Compare against standalone Tx EVM at ETM 2.0 — should be similar. Warm up FHGW; check PTP lock. |
| Freq err high but EVM OK | EXM internal reference offset (not an FHGW issue) | Warm up EXM ≥ 30 min; cross-check against GPSDO 10 MHz source if available |
| Composite verdict toggles PASS/FAIL across consecutive runs at the same setup | Borderline measurement near gate; insufficient averaging | Raise :CEVM:AVERage:COUNt to 20+; verify FHGW E-TM stream is steady-state (no thermal drift) |
| TDD: any sub-result reads NaN | DL-slot gating disabled | Verify §5.2.5 step 4 ran on B41 |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §6.2 BS Output Power, §6.5.1 Frequency Error, §6.5.2 Error Vector Magnitude (EVM), §6.5.2.2 Downlink Reference Signal (DLRS) absolute accuracy, §4.9.1 Test frequencies (fb / fm / ft), Annex C E-UTRA Test Models (E-TM 1.1, E-TM 2a, E-TM 3.1a).

- TS 36.104 §6.2 BS output power (±2.0 dB), §6.5.1 Frequency error (±0.05 ppm FDD / ±0.1 ppm TDD wide-area BS), §6.5.2 Modulation quality (8.0% EVM for 64-QAM), §6.5.2.2 RS absolute accuracy (±2.9 dB FDD / ±3.0 dB TDD).

## 3.12 Tx — Operating Band Unwanted Emissions / Spectrum Emission Mask

3GPP references: TS 36.141 §6.5.3 (conformance) + TS 36.104 §6.6.3 (radio requirement — Operating band unwanted emissions, a.k.a. Spectrum Emission Mask / SEM) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, E-TMs) live in the PW Conformance — Common Test Parameters Reference above. This test uses E-TM 1.1 AND E-TM 1.2 — two bursts per (band, BW, freq, antenna).

### 1. Purpose

Verify the FHGW's out-of-channel spectral leakage stays under the 3GPP TS 36.141 §6.5.3 emission mask. The mask defines an upper-bound power-vs-frequency profile in six offset ranges either side of the carrier (0–1 MHz, 1–5 MHz, 5–10 MHz, etc.) — the device under test must stay below the limit line across every range, on both sides of the carrier, for both E-TM 1.1 (QPSK reference) and E-TM 1.2 (16-QAM high-PAPR stress).

Conducted-RF, low-power test — output flows through a passive cable chain into the EXM. No chamber, no OTA.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / E-TM tables.

Table 3-1 — Spectrum Emission Mask Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared TX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Frequency | fb / fm / ft per (band, BW) — see Frequency points in the Reference | MHz |
| Power | P_rated — per band group, see Rated output power in the Reference | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference for all valid options (Common BW selections per band group is informational only) | MHz |
| E-TM | 1.1 (rctdl 6111) AND 1.2 (rctdl 6112) — two bursts per antenna | — |
| Offset ranges | 6 ranges per side (lower-3, lower-2, lower-1, upper-1, upper-2, upper-3); start/stop frequencies and absolute upper-bound limits per BW from TS 36.141 §6.5.3 Table 6.6.3.2-2 / 6.6.3.2-3 | MHz / dBm |
| Averaging | RMS, ≥20 sweeps | — |

The 6 offset-range start/stop frequencies and the absolute power limits are BW-dependent (e.g. for a 10 MHz carrier the first lower-range starts at carrier-edge − 0 MHz and stops at carrier-edge − 1 MHz). The SA's SEM personality auto-loads these when you set the BW preset (§5.2.5 step 2), but you must still pass the per-range start/stop/absolute-limit values explicitly via :SEMask:OFFSet:LIST:* to match the 3GPP table. Don't rely on the SA's default mask — confirm against the 3GPP table for your BW. <TBD: confirm exact SCPI :SEMask:OFFSet:LIST sequence + 6-range parameters for LTE-FDD / LTE-TDD personality on E6640A>

### 4. Pass/Fail criteria (PW automation gate)

The 3GPP test puts an absolute-power upper-bound limit line across each of 6 offset ranges per side of the carrier (12 ranges total). The SA's SEM measurement reports a delta per range: delta_range = P_meas_range − Limit_range. A negative delta means measurement is below the limit line, i.e. passing.

PW gate: for each of the 12 ranges, delta_range ≤ −Δ_margin, where Δ_margin = <TBD: kpi_thresholds.test_lte_Tx_Mask.ranges[] — per-range PW margin in dB; typical 3 dB>. The 3 dB headroom is PW's internal gate, tighter than the 3GPP delta_range ≤ 0 raw conformance line.

Per-antenna verdict: for a given (E-TM, band, BW, freq, antenna), PASS iff all 12 range deltas pass the gate above.

Worked example — B3, BW = 10 MHz, fm = 1842.5 MHz, antenna Tx2, E-TM 1.1:

| Range | Offset window (MHz) | Limit (dBm/100kHz) | P_meas (dBm/100kHz) | delta (dB) | PW gate (≤ −3) | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
| lower-3 | −10 to −5 | <TBD> | <TBD> | −12.4 | yes | PASS |
| lower-2 | −5 to −1 | <TBD> | <TBD> | −8.1 | yes | PASS |
| lower-1 | −1 to 0 | <TBD> | <TBD> | −4.3 | yes | PASS |
| upper-1 | 0 to 1 | <TBD> | <TBD> | −2.1 | no | FAIL |
| upper-2 | 1 to 5 | <TBD> | <TBD> | −7.8 | yes | PASS |
| upper-3 | 5 to 10 | <TBD> | <TBD> | −11.2 | yes | PASS |

In the example, upper-1 fails the PW gate (delta is −2.1 dB, gate wants ≤ −3 dB). The raw 3GPP test still passes (−2.1 < 0), but PW flags it for headroom-loss investigation. This antenna FAILS this E-TM because one range failed.

Test verdict: PASS only if every antenna, at every (band, BW, freq), passes for both E-TM 1.1 AND E-TM 1.2.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test. The two-E-TM loop sits inside §5.4 (per antenna, run E-TM 1.1 then re-fire rctdl to E-TM 1.2 and re-measure).

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SA init,     │
  │    personality verify, RF switch ping,   │
  │    cable-loss cal per (port, freq)       │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  │    • set FHGW Tx power                   │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │    • set SA BW preset                    │
  │    • configure SEM offset list           │
  │      (6 ranges per side, per 3GPP)       │
  │    • enable SEM averaging (RMS x20)      │
  │    • (TDD only) DL-slot gating           │
  │    • (note: rctdl fired per-ETM in §5.4) │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SA carrier reference freq       │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna ─── inner: per E-TM  │  ← this test
  │    • route RF switch                     │
  │    • apply cable loss                    │
  │    • for ETM in (1.1, 1.2):              │
  │        – fire rctdl 611x <BW>            │
  │        – auto-level SA                   │
  │        – trigger SEM averaged sweep      │
  │        – read 12 range deltas            │
  │        – verdict per range, composite    │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     │  (push new startup XML, reload SEM mask) │
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group to test?   │── yes ──► back to §5.2
     │  Same booted personality — no reboot.    │
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     │  Different personality — REBOOT needed.  │           (see "Update FHGW EEPROM
     └────────────┬─────────────────────────────┘            model name" appendix),
                  │ no                                       then back to §5.2
                  ▼
                STOP
                (verdict = PASS iff every (E-TM, antenna)
                 at every (band, BW, freq) passes)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. Each BW change pushes a fresh FHGW startup XML carrying <channel-bandwidth> and <center-of-channel-bandwidth>, then re-configures the SA's SEM offset-range list to match the new BW's 3GPP mask. The rctdl fire is deferred to §5.4 because each antenna iterates over two E-TMs (1.1 then 1.2).

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm in Hz (from the umbrella's Frequency points formula). | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Set the SA BW preset to match your BW. Substitute the BW token: B1M4 / B3M / B5M / B10M / B15M / B20M. | SA | :RAD:STAN:PRES B10M | Span auto-adjusts to roughly ±2× BW around carrier |
| 3a | Configure the 6 offset ranges per side (lower + upper) per TS 36.141 §6.5.3 mask table for this BW. Replace the 6-element lists with the start/stop frequencies (in Hz, relative to channel edge) and the per-range absolute upper-bound limits (in dBm) from the 3GPP table for your BW. <TBD: exact start/stop/absolute lists per BW> | SA | :SEMask:OFFSet:LIST:STARt <S1>,<S2>,<S3>,<S4>,<S5>,<S6>:SEMask:OFFSet:LIST:STOP <T1>,<T2>,<T3>,<T4>,<T5>,<T6>:SEMask:OFFSet:LIST:ABSolute <L1>,<L2>,<L3>,<L4>,<L5>,<L6>:SEMask:OFFSet:LIST:STATe 1,1,1,1,1,1:SEMask:OFFSet:SIDE BOTH | All 6 ranges armed both sides; limit line drawn on trace |
| 3b | Set the resolution bandwidth for SEM measurement. Typical: 100 kHz RBW per 3GPP measurement BW; the SA auto-picks via preset, but pin it for repeatability. <TBD: confirm RBW per BW> | SA | :SEMask:BAND:RES 100 kHz:SEMask:BAND:RES:AUTO OFF | RBW indicator = 100 kHz |
| 4 | Enable DL-slot gating. TDD bands only (B41) — skip on FDD. Values are for TDD config 3, special-subframe config 8; for other configs derive gate delay/length from TS 36.211 §4. | SA | :RAD:STAN:PRES:CONF 3:RAD:STAN:PRES:SSFR 8:TRIG:SOUR:GATE:OUTP:PAT:GATE:STAT ON:GATE:VIEW:STAT ON:GATE:VIEW:SWE 20 ms:GATE:DEL 3.1 ms:GATE:LEN 2.8 ms | Gated sweep; only DL window integrated |

✓ FHGW BW configured; SA SEM mask armed for this BW. → Continue to §5.3 for the first freq point. (rctdl per-E-TM fires in §5.4 step 4.)

#### 5.4 Per-antenna SEM measurement, two E-TM bursts (repeat for each antenna)

Do these steps once per antenna in this band's active TX set. For each antenna, fire E-TM 1.1 (rctdl 6111), take the SEM measurement, then fire E-TM 1.2 (rctdl 6112) and re-measure. Both bursts must pass the mask for that antenna to pass.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RF switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 (HTTP body) |
| 2 | Apply ant_i's calibrated cable-loss offset to the SA. The loss value comes from §5.1 steps 6–10 (or cable_loss.json) for the current freq. Enter as NEGATIVE dB (a 5 dB cable loss → -5). Applies to the absolute power values read inside the SEM ranges — the deltas use the corrected values. | SA | :CORR:BTS:GAIN <-CABLE_LOSS_DB> (e.g. for 5 dB loss: :CORR:BTS:GAIN -5) | SA trace shows carrier at ~P_rated (cable-loss compensated) |
| 3 | Fire E-TM 1.1 at the configured BW. Substitute the BW in MHz at the end (1.4 / 3 / 5 / 10 / 15 / 20) — must match §5.2.5 step 1a's <channel-bandwidth> and step 2's preset. | testmac console | rctdl 6111 10 | Carrier appears on SA; channel-power stable at ~P_rated; mask spans visible. Wait ~5 s for the burst to stabilise. |
| 4 | Auto-level the SA. Must be after step 3 — each E-TM has its own PAPR, the ref-level must be re-set per burst. | SA | :SENSe:POWer:RF:RANGe:OPTimize IMMediate | Trace fills upper third of screen; no IF/RF overload indicator |
| 5 | Trigger an averaged SEM sweep (≥20 sweeps) | SA | :INIT:IMM*WAI | "Average count = 20" indicator; sweep completes within ~10–20 s |
| 6 | Read the 6 lower-side range deltas (negative side of carrier, lower-3 → lower-1) | SA | :READ:SEMask:OFFSet:LOWer:POWer:DELTa? | Comma-separated list of 6 floats (dB), e.g. -12.4,-8.1,-4.3,... (the first 3 are lower-3/2/1; remaining values may include limit and absolute power — confirm parser order). <TBD: confirm exact SCPI returns + parser order> |
| 7 | Read the 6 upper-side range deltas (positive side of carrier, upper-1 → upper-3) | SA | :READ:SEMask:OFFSet:UPPer:POWer:DELTa? | Comma-separated list of 6 floats (dB). <TBD: confirm exact SCPI> |
| 8 | Verdict for E-TM 1.1: each of the 12 deltas (6 lower + 6 upper) must satisfy delta ≤ −Δ_margin (PW gate <TBD>, typical −3 dB). Antenna PASSES this E-TM iff all 12 deltas pass. | spreadsheet / notes | Per-range gate check | All 12 ≤ −3 dB → PASS (E-TM 1.1). Example: see §4 worked example. |
| 9 | Fire E-TM 1.2 at the same BW. Same BW token as step 3 — only the E-TM ID changes. | testmac console | rctdl 6112 10 | Carrier remains on SA; constellation moves to 16-QAM; channel-power stable at ~P_rated. Wait ~5 s. |
| 10 | Auto-level the SA again (E-TM 1.2 has higher PAPR than 1.1, so re-leveling matters). | SA | :SENSe:POWer:RF:RANGe:OPTimize IMMediate | Trace re-fills upper third; no overload |
| 11 | Trigger another averaged SEM sweep | SA | :INIT:IMM*WAI | Sweep completes |
| 12 | Read the 6 lower-side range deltas (E-TM 1.2) | SA | :READ:SEMask:OFFSet:LOWer:POWer:DELTa? | Comma-separated 6 floats (dB) |
| 13 | Read the 6 upper-side range deltas (E-TM 1.2) | SA | :READ:SEMask:OFFSet:UPPer:POWer:DELTa? | Comma-separated 6 floats (dB) |
| 14 | Verdict for E-TM 1.2: each of the 12 deltas must satisfy the PW gate as in step 8. Antenna PASSES this E-TM iff all 12 deltas pass. | spreadsheet / notes | Per-range gate check | All 12 ≤ −Δ_margin → PASS (E-TM 1.2) |
| 15 | Composite verdict: ant_i PASSES this (band, BW, freq) iff both E-TM 1.1 (step 8) and E-TM 1.2 (step 14) passed. | spreadsheet / notes | — | ant_i PASS / FAIL |

Note — step 2 sign convention: :CORR:BTS:GAIN is an external-gain correction. A 5 dB cable loss is entered as -5 (loss = negative gain). The SA then adds +5 dB to its raw readings to refer the result back to the FHGW port.

Note — E-TM 1.2 typically stresses the upper-1 / lower-1 ranges harder than E-TM 1.1 (higher PAPR drives more out-of-channel spill at the closest offsets). A failure on E-TM 1.1 that does not repeat on E-TM 1.2 is usually a measurement artifact (no average, wrong RBW, RF switch insertion loss not accounted for).

#### Loop control (when do I go where?)

After step 15, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq) | → back to §5.4 step 1 with the next ant_i |
| All antennas done for this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW. Push a new startup XML, re-set SA BW preset, reconfigure SEM offset list (mask values are BW-specific). |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band. No reboot — same booted personality. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1. |
| All band-groups done | STOP. Overall verdict for the test = PASS iff every (E-TM, antenna) at every (band, BW, freq) passes. |

Floor-noise sanity check (do once per session, before the first real measurement): with the FHGW transmitting and the RF switch routed to a 50 Ω termination (no antenna), the SA's measured power inside any offset range should be at or below the SA noise floor (≥ 15 dB below the limit line). If a "dummy 50 Ω" read gets close to the mask limit, the SA's own noise is contaminating the result — raise the SA ref level or check for EMI ingress on the bench.

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| All 12 ranges hover near the limit line | Cable loss not applied, or wrong cable-loss value per antenna | Re-verify cable loss with a known source (§5.1 steps 6–10); confirm :CORR:BTS:GAIN sign is negative |
| Only upper-1 / lower-1 fail under E-TM 1.2 but pass under E-TM 1.1 | Real PAPR-driven leakage — FHGW digital pre-distortion margin running thin | Confirm Tx power is set to P_rated, not higher; check DPD calibration on the FHGW |
| Mask passes on lab bench, fails on different setup | Different cable / attenuator chain — insertion loss not characterised at the offset frequencies | Re-do §5.1 cable-loss cal at multiple freqs across the mask span (not just carrier center) |
| Deltas drift > 1 dB sweep-over-sweep | Insufficient averaging | Confirm :SEMask:AVERage:COUNt 20 and wait for the full 20-sweep settle |
| TDD reads show power inside the DL guard period | DL-slot gating not configured (§5.2.5 step 4) | Re-verify gate delay/length match your TDD config + special-subframe config |
| One range reads radically different from the others (e.g. upper-3 is −40 dB while neighbours are −10 dB) | Limit line not loaded for that range, or :SEMask:OFFSet:LIST:STATe has a 0 in that slot | Re-push the 6-element STATe list with all 1s |
| Spurs appearing inside a range (sharp peak above the noise floor in the offset window) | EMI ingress from bench (PSU, monitor, USB cable) | Move SA / cabling; verify the spur tracks ambient activity, not the FHGW |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §6.5.3 Operating band unwanted emissions / Spectrum Emission Mask, Table 6.6.3.2-2 / 6.6.3.2-3 Operating band unwanted emissions limits, §4.9.1 Test frequencies (fb / fm / ft), Annex C E-UTRA Test Models (E-TM 1.1 + 1.2).

- TS 36.104 §6.6.3 Operating band unwanted emissions (radio requirement — the absolute-power limit line that 36.141 §6.5.3 tests against).

## 3.13 Tx — Transmitter Spurious Emissions

3GPP references: TS 36.141 §6.5.7 (conformance) + TS 36.104 §6.6.4 (radio requirement — Transmitter spurious emissions) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, E-TMs) live in the PW Conformance — Common Test Parameters Reference above. This test uses E-TM 1.1 only.

### 1. Purpose

Verify the FHGW's wideband out-of-band spurious emissions (typically 1 MHz to 12.75 GHz, with band-specific exclusion windows around the operating band) all sit below the 3GPP TS 36.141 §6.5.7 absolute-power limit. The mask is mostly flat at −30 dBm in a 1 MHz integration BW outside the operating band, with extra-restrictive sub-bands defined per spec for known sensitive segments (GSM, UMTS, public-safety, etc.).

Spurious emissions characterise far-from-carrier leakage that the in-band SEM test (§6.5.3) can't see — DAC harmonics, clock spurs, LO leakage, mixer products. Conducted-RF, low-power test — output flows through a passive cable chain into the EXM. No chamber, no OTA.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / E-TM tables.

Table 3-1 — Tx Spurious Emissions Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared TX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Frequency | fb / fm / ft per (band, BW) — see Frequency points in the Reference | MHz |
| Power | P_rated — per band group, see Rated output power in the Reference | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference | MHz |
| E-TM | 1.1 (rctdl 6111) | — |
| Sweep span | 1 MHz to 12.75 GHz (band-dependent — TS 36.141 Table 6.6.4.1.1-2 defines the per-band sweep limits and exclusion windows) | Hz |
| RBW | 1 MHz (per 3GPP §6.5.7 measurement bandwidth) | Hz |
| Limit | Mostly −30 dBm/MHz outside operating band; band-specific sub-bands per TS 36.141 Table 6.6.3.2-1 (e.g. −36 dBm/100kHz around GSM900) | dBm |
| Detection | Peak detector, max-hold across 5 sweeps (PW practice — 3GPP requires only single sweep) | — |
| Spurious preset | <TBD: confirm exact preset for LTE-FDD/TDD on E6640A — likely BTS, BTS-OOB, or LTE_BS. May require :CONF:SPURIOUS:STAndard followed by:SPURIOUS:LIMIT:LOAD to load the mask table.> | — |

The spurious mask is a piecewise-flat absolute-power limit line across the full sweep span, with band-specific drops in sensitive sub-bands. The SA's spurious-emissions personality loads the right mask table from a preset name — there's no need to manually program every sub-band's start/stop/limit (unlike the SEM test which uses a 6-range offset list).

### 4. Pass/Fail criteria (PW automation gate)

The SA's spurious measurement returns a list of detected peaks (one entry per peak above the noise floor in each sweep sub-band the personality scans). Each entry includes: peak frequency f_peak, measured power P_meas, the 3GPP limit at that frequency Limit(f_peak), and delta = P_meas − Limit(f_peak). A negative delta means the peak is below the limit, i.e. passing.

PW gate: for each detected peak, delta ≤ −Δ_margin, where Δ_margin = <TBD: kpi_thresholds.test_lte_Tx_Spurious — per-frequency-range PW margin in dB; typical 3-6 dB>. The PW headroom is tighter than the 3GPP delta ≤ 0 raw conformance line.

Per-antenna verdict: PASS iff all detected peaks satisfy the PW gate.

If the SA reports zero peaks above the noise floor, the antenna PASSES by definition (nothing to flag).

Worked example — B3, BW = 10 MHz, fm = 1842.5 MHz, antenna Tx2:

| Peak # | f_peak (MHz) | P_meas (dBm/MHz) | 3GPP limit (dBm/MHz) | delta (dB) | PW gate (≤ −3) | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 3680 | −38.0 | −30.0 | −8.0 | yes | PASS |
| 2 | 5527 | −41.2 | −30.0 | −11.2 | yes | PASS |
| 3 | 7370 | −33.5 | −30.0 | −3.5 | yes | PASS |
| 4 | 925.5 | −38.0 | −36.0 (GSM900 protected sub-band) | −2.0 | no | FAIL |

In the example, peak #4 sits at 925.5 MHz inside the GSM900 sensitive sub-band where the 3GPP limit drops to −36 dBm/MHz. Measured power is −38 dBm/MHz, so delta = −2.0 dB. The raw 3GPP test passes (−2.0 < 0), but PW's 3 dB margin gate trips. This antenna FAILS because one peak failed.

Note: peaks #1, #2, #3 are at the 2nd, 3rd, 4th harmonics of the B3 carrier (~1840 × 2/3/4) — these are the usual suspects for DAC harmonic leakage. Peak #4 is unrelated to the carrier and points at clock-spur leakage or LO bleed-through at the FHGW front end.

Test verdict: PASS only if every antenna, at every (band, BW, freq), passes.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SA init,     │
  │    personality verify, RF switch ping,   │
  │    cable-loss cal per (port, sub-band)   │
  │    — multi-freq cal across 1-12.75 GHz   │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  │    • set FHGW Tx power                   │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │    • load spurious preset                │
  │      (auto-loads 3GPP mask + exclusion)  │
  │    • set RBW = 1 MHz, peak detector      │
  │    • set sweep span to band-specific     │
  │      stop freq from TS 36.141 6.6.4.1.1  │
  │    • (TDD only) DL-slot gating           │
  │    • fire rctdl 6111 <BW>                │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set FHGW EARFCN per freq point      │
  │    • wait ~15 s for PHY settle           │
  │    • (SA sweep covers full span — no     │
  │      SA carrier retune needed; carrier   │
  │      moves into excluded operating band) │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna    ◄─── next antenna │  ← this test
  │    • route RF switch                     │
  │    • apply wideband cable loss table     │
  │    • trigger full spurious sweep         │
  │      (~30–60 s for 1-12.75 GHz)          │
  │    • read detected peak list             │
  │    • per-peak verdict, composite PASS    │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group to test?   │── yes ──► back to §5.2
     │  Same booted personality — no reboot.    │
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     │  Different personality — REBOOT needed.  │           (see "Update FHGW EEPROM
     └────────────┬─────────────────────────────┘            model name" appendix),
                  │ no                                       then back to §5.2
                  ▼
                STOP
                (verdict = PASS iff every antenna
                 at every (band, BW, freq) passes)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. Each BW change pushes a fresh FHGW startup XML carrying <channel-bandwidth> and <center-of-channel-bandwidth> (the EARFCN that defines fm), then re-arms the SA's spurious mask + sweep range, and fires rctdl.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm in Hz (from the umbrella's Frequency points formula). | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Set the spurious-emissions sweep upper bound per TS 36.141 Table 6.6.4.1.1-2 for your band. Most bands sweep to 12.75 GHz, but band-specific shorter upper bounds exist. Confirm against the table. <TBD: per-band sweep upper bound and exclusion-window settings> | SA | :SPURIOUS:FREQ:STOP 12.75 GHz:SPURIOUS:FREQ:STAR 1 MHz | Sweep span readout = 1 MHz – 12.75 GHz |
| 3 | (Verify the spurious preset loaded in §5.1 step 4 still has the correct exclusion window around your operating band. The preset auto-applies it per the personality, but the operating band may have shifted; refresh.) | SA | :SPURIOUS:STAN:PRES <PRESET> <TBD> | Mask redraws with operating-band exclusion correctly placed |
| 4 | Enable DL-slot gating. TDD bands only (B41) — skip on FDD. Values are for TDD config 3, special-subframe config 8; for other configs derive gate delay/length from TS 36.211 §4. | SA | :RAD:STAN:PRES:CONF 3:RAD:STAN:PRES:SSFR 8:TRIG:SOUR:GATE:OUTP:PAT:GATE:STAT ON:GATE:VIEW:STAT ON:GATE:VIEW:SWE 20 ms:GATE:DEL 3.1 ms:GATE:LEN 2.8 ms | Gated sweep; only DL window integrated |
| 5 | Fire E-TM 1.1 at the configured BW. Substitute the BW in MHz at the end (1.4 / 3 / 5 / 10 / 15 / 20) — must match step 1a's <channel-bandwidth>. | testmac console | rctdl 6111 10 | Carrier appears on SA inside the operating band (which is excluded from the mask); spurious sub-bands quiet. The E-TM stream stays running for all freq points below — do not refire between freqs. |

✓ FHGW transmitting E-TM 1.1 at this BW; SA spurious sweep armed across the band's 3GPP span. → Continue to §5.3 for the first freq point.

#### 5.4 Per-antenna spurious-emissions measurement (repeat for each antenna)

Do these steps once per antenna in this band's active TX set. The carrier is already up (§5.2.5 step 5). Trigger the full wideband sweep, read the detected peak list, apply per-peak wideband cable-loss correction, and verdict each peak against the 3GPP mask + PW gate.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RF switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 (HTTP body) |
| 2 | Apply a coarse wideband cable-loss reference to the SA. This is a single-number compromise — applies a representative loss across the whole span. The per-peak fine correction happens in step 6. Pick the loss at the carrier freq from the wideband loss table (§5.1 steps 6–10). Enter as NEGATIVE dB. | SA | :CORR:BTS:GAIN <-CABLE_LOSS_DB> (e.g. :CORR:BTS:GAIN -8) | SA trace reference shifts uniformly across span |
| 3 | Trigger a single full-span spurious sweep (peak detector). Takes ~30–60 s. Do not interrupt. <TBD: confirm if :INIT:IMMis enough or if a separate:SPURIOUS:SWEEP trigger is needed> | SA | :INIT:IMM*WAI | Sweep progress bar reaches 100%; mask + detected-peak markers drawn |
| 4 | (Optional — PW practice) Retrigger 4 more sweeps in max-hold mode to catch sporadic peaks. 3GPP requires only a single sweep; PW adds 5-sweep max-hold for robustness. | SA | :TRACE:TYPE MAXHold:INIT:IMM*WAI (repeat ×4) | Max-hold trace stabilises after ~3 sweeps |
| 5 | Read the detected peak list. Returns a comma-separated record per peak: freq (Hz), measured power (dBm), 3GPP limit at that freq (dBm), delta (dB). Number of peaks varies by sweep. <TBD: confirm exact SCPI returns + parser format — possibly :READ:SPURIOUS:RES? returns a CSV block or a structured table> | SA | :READ:SPURious:RES? | CSV block, one row per detected peak above the noise floor |
| 6 | Apply per-peak wideband cable-loss correction. For each peak's freq, look up the right loss from cable_loss_wideband.json (§5.1) — interpolate between calibration freqs if needed — and add it back to P_meas. This refines the coarse single-number correction from step 2. | spreadsheet / notes | P_meas_corr(f) = P_meas(f) + (loss_at_f − loss_at_carrier) | Corrected P_meas per peak |
| 7 | Per-peak verdict: delta = P_meas_corr − Limit_3GPP(f_peak). PW gate: delta ≤ −Δ_margin where Δ_margin = <TBD> (typical 3–6 dB). | spreadsheet / notes | Per-peak gate check | Per-peak PASS / FAIL |
| 8 | Composite verdict for ant_i: PASS iff all detected peaks pass the PW gate. If the sweep returned zero peaks (nothing above the noise floor), the antenna PASSES by definition. | spreadsheet / notes | — | ant_i PASS / FAIL. Example: see §4 worked example. |

Note — step 2 / step 6 sign convention: :CORR:BTS:GAIN is an external-gain correction. A 5 dB cable loss is entered as -5 (loss = negative gain). The SA then adds +5 dB to its raw readings to refer the result back to the FHGW port. For spurious, the loss varies a lot across 1 MHz–12.75 GHz — the coarse single-number correction (step 2) sets a baseline at the carrier freq, and step 6 refines it per detected peak. Cable runs with significant high-freq roll-off (cheap SMA cables above 6 GHz) will mask real spurious peaks if you only apply the carrier-freq correction.

#### Loop control (when do I go where?)

After step 8, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq) | → back to §5.4 step 1 with the next ant_i |
| All antennas done for this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point (re-push startup XML if you want fb / ft — see caveat) |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW. Push a new startup XML, refire E-TM, reverify operating-band exclusion still matches. No ACTIVE_*.xml change — same activated band. |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band. Push a different ACTIVE_*.xml, re-set Tx power. No reboot — same booted personality. Reload the spurious preset / exclusion to match the new operating band. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 for the first band in the new group. |
| All band-groups done | STOP. Overall verdict for the test = PASS iff every antenna at every (band, BW, freq) passes. |

Quiet-antenna sanity check (do once per session before the first real measurement): with the FHGW transmitting and the RF switch routed to a 50 Ω termination (no antenna), the spurious sweep should return zero peaks above the noise floor across 1 MHz–12.75 GHz. If a "dummy 50 Ω" sweep flags peaks, those peaks are bench EMI ingress or SA internal spurs — not the FHGW — and you must isolate / shield before real measurements.

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| Lots of peaks at high freqs (>6 GHz), all failing | Wideband cable loss not applied per peak — cable rolls off at high freq and the SA reports inflated P_meas | Re-verify wideband cable-loss table is populated above 6 GHz; confirm step 6 correction was applied |
| Single peak at exactly 2× / 3× / 4× the carrier freq | Real DAC harmonic — usually a real FHGW issue if it exceeds the limit | Check DAC filter / matching network on the FHGW; confirm the harmonic survives a power-back-off (if it scales 1:1 with carrier, it's the carrier's harmonic; if it stays put, it's an LO spur) |
| Peaks appear in GSM900 / public-safety / UMTS sub-bands and fail the tighter mask | The PRESET preset's exclusion-list doesn't match the operating band you actually configured | Re-verify §5.2.5 step 3 reloaded the spurious preset after pushing the startup XML; check the operating-band exclusion window covers your band |
| Sweep takes >2 minutes | RBW set lower than 1 MHz, or sweep span is unreasonably wide | Confirm :SPURIOUS:BAND:RES 1 MHz; confirm :SPURIOUS:FREQ:STOP matches the band's 3GPP upper bound (not always 12.75 GHz) |
| TDD sweep shows artificially low spurious in DL slot | Wrong gating — the gate window is missing the DL burst | Re-verify gate delay/length match your TDD config + special-subframe config |
| All peaks fail by ~10 dB consistently across the span | Cable-loss correction sign flipped (entered as +5 instead of -5) — SA is subtracting loss instead of adding it back | Re-check :CORR:BTS:GAIN sign convention (loss = negative gain) |
| Sporadic peaks appear in different positions each sweep | EMI ingress (PSU switching, monitor, lab WiFi) — not the FHGW | Use the 5-sweep max-hold (step 4) — real FHGW spurious stay put; EMI moves. Move SA / cabling to isolate. |
| Zero peaks reported but you expected some | Detector mode set wrong (e.g. RMS instead of PEAK), or noise floor too low to flag any peak | Confirm :SPURIOUS:DET PEAK; confirm the SA noise floor is at least 15 dB below the mask |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §6.5.7 Transmitter spurious emissions, Table 6.6.3.2-1 Transmitter spurious emissions limits, Table 6.6.4.1.1-2 Frequency ranges and exclusion bands for spurious emissions tests, §4.9.1 Test frequencies (fb / fm / ft), Annex C E-UTRA Test Models (E-TM 1.1).

- TS 36.104 §6.6.4 Transmitter spurious emissions (radio requirement — the absolute-power mask that 36.141 §6.5.7 tests against, with band-specific protected sub-bands).

# 4 Uplink Test Suite

All uplink tests run on the same conducted-RF bench — equipment, connectivity, and operating rules below. Each test's body covers what's specific to that test.

Table 4-0 — Test Equipment

| Test Equipment | Notes |
| --- | --- |
| DU | Linux server running L1 / L2 / testmac |
| FHGW Box | Unit under test — FHGW compute board (Xilinx ZU67 RFSoC, 8 TX + 8 RX ports) + BOB break-out board, no RFFE attached. Low-power RF input. |
| PTP Grand Master | Microchip Falcon-RX/812/G/A — LLS-C1 (feeds DU only). Supplies the 10 MHz reference the EXM locks to for SG-mode FRC generation. |
| Keysight EXM | E6640A Wireless Test Set, SG mode (same box as Ch.3, mode-switched) — sources LTE-FDD / LTE-TDD uplink FRC waveforms (A3-x and A1-x), output down to ≥ −110 dBm, 10 MHz REF IN locked to Falcon-RX. |
| NUC | Test controller PC |
| Splitter | 1 input × 2 outputs (×4 for 4-antenna bands) |
| RF Cable | SMA-to-SMA, 50 Ω |
| 50 Ω terminators | One per unused RX antenna port |

Figure 4-0 — Conducted-RF test bench (LTE Rx)

Same bench as Ch.3 — the Keysight EXM E6640A doubles as both spectrum analyzer (Tx tests) and signal generator (Rx tests); mode-switched via SCPI. The hardware doesn't change; only the signal flow reverses: EXM (SG mode) → cables → FHGW RX port (instead of FHGW TX → cables → EXM (SA mode)).

Operating rules

- One RX port at a time. Other RX ports terminated with 50 Ω. Inactive chassis ports stay idle.

- Each port has its own calibrated cable-loss value at the UL frequency. Cable loss differs DL vs UL — measure all of them at the UL band before starting.

- Lock the SG's 10 MHz reference to the lab time base. At sensitivity levels (−55 to −70 dBm) the SG's carrier must not drift relative to the FHGW LO; otherwise the demodulator never locks.

## §5 Procedure — common parts (LTE Uplink)

§5.1, §5.2, and §5.3 below are the shared §5 Procedure steps for every uplink test in this chapter. The "Where to run" column refers to the endpoints listed in How to send commands in Chapter 3 above — same endpoints, with the EXM operating in SG mode instead of SA mode. Run them as part of each test's §5 — the test's flowchart calls each section out in order (§5.1 → §5.2 → §5.2.5 → §5.3 → §5.4). The test-specific §5.2.5 and §5.4 live in each test's body.

### 5.1 Bench bring-up (do once at session start)

These steps configure the bench. Do them once when you start your session — they don't change with band, BW, freq, or antenna.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | SSH into the DU | PC | ssh root@<DU_IP> | DU shell prompt |
| 2 | Find the FHGW IP under the fronthaul interface | DU shell | arp -n | FHGW IP appears under ens1f3 |
| 3 | Reset the EXM (in SG mode) and lock to external 10 MHz reference. Critical for sensitivity tests — without the shared 10 MHz the demod won't lock at −55 to −70 dBm. | SG | *RST*OPC?:ROSC:SOUR EXT:ROSC:EXT:FREQ 10MHZ | *OPC? returns 1; front-panel EXT REF solid (not flashing) |
| 4 | Select the LTE uplink personality. FDD bands use FDD; TDD B41 uses TDD — substitute below. | SG | :RAD:LTE:LINK UL:RAD:LTE:LINK:DUPL FDD:RAD:LTE STAT ON | Front panel shows LTE UL FDD (or LTE UL TDD) |
| 5 | Sanity-check the RF switch is reachable | RF switch (HTTP) | GET http://<SWITCH_IP>/MN? | MN=RC-2SP6T-26 |
| 6 | Calibrate cable loss per (RX antenna, UL freq point). Skip steps 6–10 if cable_loss.json is populated for this bench at the UL freq. Cable the SG-side chain (SG → splitter → switch → RX port) with a power-meter at the RX-port end. | bench (physical) | — | SG → splitter → switch → meter |
| 7 | Output a CW tone at a known reference level (e.g. −10 dBm) at the UL freq point you're calibrating | SG | :FREQ <UL_FREQ_HZ>:POW -10:OUTP ON | Tone visible on meter |
| 8 | Route the RF switch to ant_i and read level at the RX-port end. Substitute <BANK> and <PORT> from your setup's RX mapping. | RF switch then meter | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | Numeric dBm value P_meas_cal |
| 9 | Compute cable loss = SG_ref − P_meas_cal. Record in cable_loss.json keyed by (antenna_port, ul_freq_hz). Store as a positive dB value; §5.4 step 2 adds it to the SG output to compensate. | spreadsheet / notes | e.g. −10 − (−14.1) = 4.1 dB | One entry in the loss table |
| 10 | Repeat steps 7–9 for every (antenna × UL freq) in your test plan. When done: turn SG off and restore normal Rx cabling (SG → splitter → switch → FHGW RX). | bench | :OUTP OFF | Full per-(port, UL freq) loss table; bench ready |
| 11 | SSH into the FHGW from the DU, cd into the netopeer working directory, and verify the personality matches your band group. If the personality is wrong, fix it before the next steps — they push band-group-specific config. | FHGW shell | ssh root@<FHGW_IP>cd /root/ORAN-CLI/netopeer2personality-app --flavour | Personality equals the band group under test (e.g. b0103, b0307, b0407, b082028, b41). If anything else: STOP — rewrite the EEPROM model name and reboot (see Update FHGW EEPROM model name) or use a different bench. |
| 12 | Launch netopeer2-cli, connect to the FHGW's NETCONF server (loopback), and push the radio mode config (LTE) | FHGW shell → netopeer2-cli | netopeer2-cli> connect --host localhost --login root> edit-config --target running --config=ConfigID_LTE.xml --rpc 40 | netopeer2-cli > prompt opens; NETCONF OK |
| 13 | Push FHGW PTP profile (G.8275.1) | netopeer2-cli session (from step 12) | > edit-config --target running --config=set-ru-ptp-config-G8275-1.xml --rpc 40 | NETCONF OK; FHGW PTP syncs |
| 14 | Push processing-elements (eCPRI mapping) | netopeer2-cli session | > edit-config --target running --config=set-processing-elements.xml --rpc 40 | NETCONF OK |
| 15 | Start L1 (PHY) stack. L1 stays in foreground — leave this shell open. | DU shell #1 | ./l1.sh -rct | welcome to application console banner within ~60 s |
| 16 | Start L2 (testmac) stack | DU shell #2 (new SSH session) | ./l2.sh | testmac> prompt appears |

✓ Bench is now armed. → Continue to §5.2 for the first band in your test plan.

### 5.2 Per-band-combo setup (repeat for each band combo)

Do these steps once per band combo — a (band, band-group) pair, e.g. B3 in band-group b0103. The band-group is the booted FHGW personality (fixed per session — to change it, see Update FHGW EEPROM model name). Pushing this combo's ACTIVE_*.xml is a full snapshot: it activates this band's carriers AND deactivates the other band's carriers in the same group. BW + FRC + rctul config lives in §5.2.5 (next).

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Activate the A0/A1 carriers for your combo. Look up your <A0A1_FILE> in the ACTIVE_*.xml file lookup below — same XMLs activate both TX and RX carriers; the Rx test consumes the RX side. | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<A0A1_FILE> | NETCONF OK |
| 2 | Activate the A2/A3 carriers. 4-antenna combos only (b0103, b082028, b41) — skip when the lookup row below shows a single file (b0307, b0407). Look up your <A2A3_FILE> in the same lookup row. | netopeer2-cli session | > edit-config --target running --config=<A2A3_FILE> | NETCONF OK (merges with step 1 — does not replace it) |

See ACTIVE_*.xml file lookup in Chapter 3 §5.2 above — the same XMLs activate both TX and RX carriers; the Rx tests consume the RX side.

✓ Carriers active on this band. → Continue to §5.2.5 to pick a BW.

### 5.3 Per-freq-point setup (repeat for each fb / fm / ft)

Do these steps once per freq point within the current BW. Substitute the UL frequency (UL_min + BW/2, etc.) — not the DL freq — see the umbrella's UL/DL range table. Note: fb / fm / ft are re-derived from the current BW.

Note — fb / ft caveat. §5.3 only retunes the SG. The FHGW's RX EARFCN is pinned by the §5.2.5 step 1 startup XML you pushed (typically fm). For a true fb / ft measurement, re-push the startup XML with the target <center-of-channel-bandwidth> (i.e. re-do §5.2.5 step 1). Alternatively, treat each (BW, freq) pair as its own pass through §5.2.5.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Set the SG carrier reference frequency to this UL freq point. Substitute <UL_FREQ_HZ> — Hz, not MHz (e.g. B3 / fm UL = 1747.5 MHz → 1747500000). | SG | :FREQ:CW <UL_FREQ_HZ> | Center-freq readout updates |
| 2 | Set initial SG output power. Substitute <INIT_POWER_DBM> = -63 + estimated_path_loss. Do not enable RF yet. | SG | :POW:AMPL <INIT_POWER_DBM> | Amplitude readout updates |
| 3 | Wait ~15 s for PHY to settle | — | — | BLER counter on DU still 100% (SG RF still off) |

✓ SG carrier centered. → Continue to §5.4 for the first antenna.

## 4.1 Rx — Reference Sensitivity Level

3GPP references: TS 36.141 §7.2 (conformance) + TS 36.104 §7.2 (radio requirement) + TS 36.141 Annex A (FRC definitions) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, FRCs, sensitivity limits) live in the PW Conformance — Common Test Parameters Reference above. This test uses the A3-x FRC family.

### 1. Purpose

Verify the FHGW receiver achieves the minimum reference sensitivity level (BLER ≤ 5 %) for the configured band group on every active RX port. Conducted-RF measurement: SG transmits an uplink FRC; FHGW receives.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / FRC tables.

Table 3-1 — Reference Sensitivity Level Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared RX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Frequency | fb / fm / ft per (band, UL BW) — see Frequency points in the Reference | MHz |
| SG Power | Sensitivity limit — per band group, see Reference sensitivity level in the Reference | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference for all valid options (Common BW selections per band group is informational only). FRC waveform tracks BW per FRC selection (Rx). | MHz |
| FRC | A3-x family (A3-4 / A3-5 / A3-6 / A3-7 per BW) — see Fixed Reference Channel (FRC) selection in the Reference | — |
| BLER threshold | ≤ 5 % (with ±4 % measurement tolerance) | % |

### 4. Pass/Fail criteria (3GPP)

Per TS 36.141 §7.2.3, at each RX port the throughput must be ≥ 95% of the reference measurement channel's maximum at input power P_REFSENS + 3 dB. Equivalently, BLER ≤ 5% when the SG drives the antenna at the band's sensitivity limit (from the Reference). PW automation applies a bler_tolerance of ±4% to absorb measurement noise — see FHGW_Box_kpi_thresholds.json → test_Rx_Reference_Sensitivity_Level.test_parameters.

Test verdict: PASS only if every RX port meets BLER ≤ 5%.

#### 4.1 PW-internal measurement sweep (informational)

PW automation doesn't apply the SG power directly at the sensitivity limit — it sweeps from high to low to find the actual sensitivity point:

| Stage | SG power | Step | Purpose |
| --- | --- | --- | --- |
| Initial | −63 dBm | — | Above the limit; confirm BLER ≈ 0 |
| Coarse | down to band limit | 1 dB | Approach the threshold |
| Fine | within ±1 dB of the limit | 0.6 dB | Refine the threshold |
| Finest | within ±0.2 dB | 0.2 dB | Final measurement |
| Safety | −45 dBm (fault_power) | — | Hard upper bound; abort if reached |

The reported sensitivity point is the lowest power where BLER stays ≤ 5%. PASS = that point ≤ the band's sensitivity limit.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test — the inner loop runs the SG-driven UL sensitivity sweep.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SG init,     │
  │    SG external 10 MHz ref lock,          │
  │    personality verify, RF switch ping,   │
  │    cable-loss cal per (port, UL freq)    │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │      (channel-bandwidth + EARFCN)        │
  │    • load FRC waveform on SG (per BW)    │
  │    • (TDD) configure SG UL burst gating  │
  │    • fire rctul <BW> <FRC>               │
  │      (UL-RX mode stays running across    │
  │       all freqs at this BW)              │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SG carrier reference freq       │
  │      (UL freq, not DL)                   │
  │    • set initial SG output power         │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna    ◄─── next antenna │  ← this test
  │    • route RF switch                     │
  │    • adjust SG output for cable loss     │
  │    • enable SG RF output                 │
  │    • sweep SG power down until BLER >5 % │
  │    • verdict (measured ≤ sens. limit?)   │
  │    • disable SG RF output                │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     │  (rctul keeps running)          │
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     │  (e.g. 5 → 10 → 20 MHz at this band)     │           push new startup XML,
     │  Same activated band — no ACTIVE_*.xml.  │           re-load FRC, refire rctul
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group to test?   │── yes ──► back to §5.2
     │  (e.g. did B1 in b0103, now want B3)     │           push different ACTIVE_*.xml,
     │  Same booted personality — no reboot.    │
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     │  (e.g. did all of b0103, now want b0307) │           (see "Update FHGW EEPROM
     │  Different personality — REBOOT needed.  │           model name" appendix),
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every RX antenna
                 at every (band, freq) meets
                 the sensitivity limit)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. Each BW change pushes a fresh FHGW startup XML (BW + EARFCN), loads the matching FRC waveform on the SG, and fires rctul to put the DU in UL-RX mode.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm UL freq in Hz. | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_UL_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Load the FRC waveform on the SG for this BW. Substitute the FRC name per the table below. | SG | :RAD:LTE:UL:WAV "A3-5" | Panel shows the FRC name, PRB allocation, QPSK 1/3 |
| 3 | Burst-gate the SG to UL slots. TDD bands only (B41) — skip on FDD. | SG | :RAD:LTE:UL:TDD:CONF 3:RAD:LTE:UL:TDD:SSFR 8:SOUR:RAD:LTE:UL:TRIG:SOUR FRAME:SOUR:RAD:LTE:UL:BURS:STAT ON | SG transmits only during UL subframes |
| 4 | Start UL reception in test mode. Substitute the BW (MHz) and matching FRC. | testmac console | rctul 10 A3-5 | DU enters UL-RX mode; BLER counter visible (initially 100% — SG RF not enabled yet) |

FRC selection (for step 2 and step 4):

| BW | FRC |
| --- | --- |
| 5 MHz | A3-4 |
| 10 MHz | A3-5 |
| 15 MHz | A3-6 |
| 20 MHz | A3-7 |

✓ DU in UL-RX mode at this BW. → Continue to §5.3 for the first freq point.

#### 5.4 Per-antenna measurement (repeat for each antenna)

Do these steps once per antenna in the band's active RX set. Each antenna's measurement is a power sweep — find the lowest SG output where the DU-reported BLER stays ≤ 5 %.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RX switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 |
| 2 | Adjust SG output for this antenna's cable loss. Add the cable-loss value (positive dB from cable_loss.json) to the desired sensitivity level — the SG must drive harder to compensate. | SG | :POW:AMPL <INIT_POWER + CABLE_LOSS_I> (e.g. for init −63 dBm + 4.1 dB cable loss: :POW:AMPL -58.9) | SG output level updates |
| 3 | Enable the SG RF output | SG | :OUTP:STAT ON | SG transmits |
| 4 | Verify the FHGW sees the signal | DU log / RU diag | (vendor-specific RSSI query) | RSSI for ant_i ≈ init_power ± 1 dB |
| 5 | Sweep SG output power downward — coarse (1 dB), fine (0.6 dB), finest (0.2 dB) — until BLER first crosses 5%. Read BLER from the DU after each adjustment. | SG | Repeat :POW:AMPL <NEW_VALUE> at each step | Lowest SG output (cable-loss corrected) where DU BLER stays ≤ 5% across the test_duration window (default 1000 ms) |
| 6 | Compare measured sensitivity against the band's sensitivity limit | spreadsheet / notes | measured_sensitivity ≤ band_sensitivity_limit | If ≤ → ant_i PASS. Example: B3 in b0103 group → limit = −68 dBm. measured = −70 dBm → −70 ≤ −68 → PASS. |
| 7 | Disable the SG RF output before moving to the next antenna | SG | :OUTP:STAT OFF | SG stops transmitting |

#### Loop control (when do I go where?)

After step 7, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq) | → back to §5.4 step 1 with the next ant_i |
| All antennas done for this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW (e.g. 10 → 20 MHz at B3). Push a new startup XML, re-load FRC, refire rctul. No ACTIVE_*.xml change — same activated band. |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band (e.g. B1 → B3 in b0103). Push a different ACTIVE_*.xml. No reboot — same booted personality. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 for the first band in the new group. |
| All band-groups done | STOP. Overall verdict = PASS iff every RX antenna at every (band, BW, freq) meets the sensitivity limit. |

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| BLER = 100% even at high SG power | SG output OFF, or carrier freq doesn't match RU's UL freq | Re-verify §5.3 step 1 (UL freq, not DL) and §5.4 step 3 (:OUTP:STAT ON) |
| BLER unstable at sensitivity limit | SG carrier drifting (external 10 MHz reference not locked) | Re-verify §5.1 step 3 — "EXT REF" indicator must be solid green |
| Sensitivity is 5+ dB worse than the limit on one antenna | Wrong cable-loss value used for that antenna at the UL freq | Re-verify the calibrated cable loss for that specific antenna; cable loss differs DL vs UL |
| Sensitivity drifts > 0.5 dB across a 10 s window | Measurement window too short; per-subframe BLER variance | Increase test_duration (default 1000 ms) or extend the manual averaging window |
| BLER on TDD (B41) stays high even at high SG power | SG transmitting continuously; jamming FHGW DL | Verify §5.2.5 step 3 — BURSt:STAT ON must be set for TDD bands |
| SG can't output below −90 dBm | Output amplifier minimum too high | Add a calibrated external attenuator; recompute cable_loss_dB |
| BLER reads 0 % at all power levels (suspicious) | DU not actually in UL-RX mode; FRC mismatch between SG and DU | Re-verify §5.2.5 — same FRC name on both SG (step 2) and DU (step 4) |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §7.2 Reference Sensitivity Level, §7.2.3 Test requirements, §4.9.1 Test frequencies (fb / fm / ft), Annex A Fixed Reference Channels for receiver tests.

- TS 36.104 §7.2 Reference sensitivity level (the REFSENS table that PW's base_dbm is derived from).

- PW automation thresholds: configuration/setup_configuration/kpi_thresholds_references/FHGW_Box_kpi_thresholds.json → test_Rx_Reference_Sensitivity_Level.kpi

## 4.2 Rx — RSSI Readback Calibration

3GPP reference: TS 36.141 §7.4.3 (RSSI is not a formal BS conformance metric — there is no 3GPP RSSI test requirement). This is a PW-internal readback validation: the FHGW PHY exposes a per-antenna RSSI counter (in dBFS, full-scale) and PW applies a per-(port, UL freq) rssi_cal offset to convert it to dBm. The test checks the corrected RSSI tracks the SG input level across three calibration points.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, FRCs, antenna mapping) live in the PW Conformance — Common Test Parameters Reference above. This test does not use an FRC — it uses a modulated LTE-UL stream at three discrete SG output levels.

### 1. Purpose

Verify the FHGW PHY's per-antenna RSSI readback (in dBFS) tracks the actual RF input power within ±1.5 dB after applying the calibrated rssi_cal offset. This is a 3-point calibration check (high / mid / low), repeated per active RX antenna at the band's UL fm frequency. The test confirms the PHY's power-detection chain (ADC → digital RSSI accumulator → readback API) is linear and consistent with the bench-measured input level. No BLER measurement — this test does not exercise the demodulator; it validates the wideband power detector only.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / antenna tables.

Table 3-1 — RSSI Readback Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared RX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Frequency | UL fm per band (mid-band UL carrier) — see Frequency points in the Reference | MHz |
| SG Power | Three calibration levels per antenna: high = −60 dBm, mid = −80 dBm, low = −100 dBm (<TBD: confirm the 3 cal points from kpi_thresholds.test_Rx_RSSI>) | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference for all valid options (Common BW selections per band group is informational only). | MHz |
| Waveform | Modulated LTE-UL test pattern (FRC waveform OK; the test reads wideband RSSI, not FRC-specific demod) | — |
| RSSI tolerance | ±1.5 dB (corrected RSSI vs SG input level at each cal point) | dB |

### 4. Pass/Fail criteria (PW gate)

This test has no 3GPP conformance metric — RSSI is a PW-internal readback that doesn't appear in TS 36.141. The pass gate is PW's own linearity-and-offset tolerance:

PW automation pass gate: At each of the three SG calibration points (high / mid / low), the corrected RSSI must satisfy:

|corrected_RSSI − SG_input_level| ≤ 1.5 dB

where corrected_RSSI = raw_RSSI_dBFS + rssi_cal[port][freq].

Test verdict: PASS only if every active RX port satisfies the ±1.5 dB tolerance at all three calibration points.

PW threshold source: <TBD: confirm ±1.5 dB tolerance + the 3 specific cal points from kpi_thresholds.test_Rx_RSSI>.

#### 4.1 Worked example

B3 in band group b0103, antenna ant_1, UL fm = 1747.5 MHz, calibrated cable loss = 4.1 dB, rssi_cal[ant_1][1747.5MHz] = −20.3 dB (this is the bench's pre-populated offset).

| Cal point | SG amplitude (at SG port) | Level at FHGW RX port | Raw RSSI (dBFS) | Corrected RSSI = raw + cal | |Δ| vs input | Verdict | |---|---|---|---|---|---|---| | high | −55.9 dBm (= −60 + 4.1 loss) | −60.0 dBm | −39.7 | −39.7 + (−20.3) = −60.0 dBm | 0.0 dB | PASS | | mid | −75.9 dBm | −80.0 dBm | −60.0 | −60.0 + (−20.3) = −80.3 dBm | 0.3 dB | PASS | | low | −95.9 dBm | −100.0 dBm| −80.4 | −80.4 + (−20.3) = −100.7 dBm| 0.7 dB | PASS |

All three |Δ| are ≤ 1.5 dB → ant_1 PASS at fm. Repeat for ant_2, ant_5, ant_6 (B3 in b0103 active RX set). Overall B3 (b0103) PASS iff all four antennas pass at all three cal points.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test — the inner loop reads RSSI at three SG levels per antenna.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SG init,     │
  │    SG external 10 MHz ref lock,          │
  │    personality verify, RF switch ping,   │
  │    cable-loss cal per (port, UL freq)    │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │      (channel-bandwidth + EARFCN)        │
  │    • load LTE-UL modulated pattern on SG │
  │    • (TDD) configure SG UL burst gating  │
  │    • fire rctul <BW> <FRC>               │
  │      (UL-RX mode stays running across    │
  │       all freqs + all antennas + all     │
  │       cal points at this BW)             │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SG carrier reference freq       │
  │      (UL freq, not DL)                   │
  │    • set initial SG output power         │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna    ◄─── next antenna │  ← this test
  │    • route RF switch                     │
  │    • for each of 3 cal points (hi/mid/lo)│
  │        ◦ set SG amplitude (+cable loss)  │
  │        ◦ enable SG RF output             │
  │        ◦ read raw RSSI from PHY          │
  │        ◦ compute corrected RSSI          │
  │        ◦ |Δ| vs input ≤ 1.5 dB ?         │
  │    • disable SG RF output                │
  │    • verdict (all 3 cal points pass?)    │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     │  (rctul keeps running)          │
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     │  (e.g. 5 → 10 → 20 MHz at this band)     │           push new startup XML,
     │  Same activated band — no ACTIVE_*.xml.  │           re-load FRC, refire rctul
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group to test?   │── yes ──► back to §5.2
     │  (e.g. did B1 in b0103, now want B3)     │           push different ACTIVE_*.xml,
     │  Same booted personality — no reboot.    │
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     │  (e.g. did all of b0103, now want b0307) │           (see "Update FHGW EEPROM
     │  Different personality — REBOOT needed.  │           model name" appendix),
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every RX antenna
                 at every (band, freq) passes all 3
                 RSSI cal points within ±1.5 dB)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. Each BW change pushes a fresh FHGW startup XML (BW + EARFCN), loads a modulated LTE-UL pattern on the SG, and fires rctul to put the DU in UL-RX mode (so the PHY's RSSI counters update).

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm UL freq in Hz. | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_UL_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Load the FRC waveform on the SG for this BW. Substitute the FRC name per the table below. | SG | :RAD:LTE:UL:WAV "A3-5" | Panel shows the FRC name, PRB allocation, QPSK 1/3 |
| 3 | Burst-gate the SG to UL slots. TDD bands only (B41) — skip on FDD. | SG | :RAD:LTE:UL:TDD:CONF 3:RAD:LTE:UL:TDD:SSFR 8:SOUR:RAD:LTE:UL:TRIG:SOUR FRAME:SOUR:RAD:LTE:UL:BURS:STAT ON | SG transmits only during UL subframes |
| 4 | Start UL reception in test mode. Substitute the BW (MHz) and matching FRC. | testmac console | rctul 10 A3-5 | DU enters UL-RX mode; BLER counter visible (initially 100% — SG RF not enabled yet) |

FRC selection (for step 2 and step 4):

| BW | FRC |
| --- | --- |
| 5 MHz | A3-4 |
| 10 MHz | A3-5 |
| 15 MHz | A3-6 |
| 20 MHz | A3-7 |

✓ DU in UL-RX mode at this BW; PHY RSSI counters live. → Continue to §5.3 for the first freq point.

#### 5.4 Per-antenna RSSI calibration (repeat for each antenna)

Do these steps once per antenna in the band's active RX set. Each antenna's measurement reads RSSI at three SG calibration points (high / mid / low) and applies the pre-populated rssi_cal offset to convert raw dBFS to dBm.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RX switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 |
| 2a | HIGH cal point. Set SG amplitude for high level (default −60 dBm) plus cable loss. e.g. for −60 dBm + 4.1 dB cable loss: :POW:AMPL -55.9. | SG | :POW:AMPL <-60 + CABLE_LOSS_I> | SG output level updates |
| 2b | Enable the SG RF output | SG | :OUTP:STAT ON | SG transmits |
| 2c | Read raw RSSI from the FHGW PHY for ant_i. Substitute the antenna index (e.g. ant_1 → 1). | testmac console | show rssi <ant_i> (or vendor-specific RSSI query) | Raw RSSI in dBFS (e.g. −39.7) |
| 2d | Compute corrected RSSI and check tolerance. Look up rssi_cal[ant_i][UL_FREQ] from the cal table. | spreadsheet / notes | corrected = raw + rssi_cal[ant_i][freq]Δ = corrected − (-60) | If ` |
| 3a | MID cal point. Set SG amplitude for mid level (default −80 dBm) plus cable loss. e.g. for −80 dBm + 4.1 dB cable loss: :POW:AMPL -75.9. | SG | :POW:AMPL <-80 + CABLE_LOSS_I> | SG output level updates |
| 3b | Read raw RSSI from the FHGW PHY for ant_i. | testmac console | show rssi <ant_i> | Raw RSSI in dBFS (e.g. −60.0) |
| 3c | Compute corrected RSSI and check tolerance. | spreadsheet / notes | corrected = raw + rssi_cal[ant_i][freq]Δ = corrected − (-80) | If ` |
| 4a | LOW cal point. Set SG amplitude for low level (default −100 dBm) plus cable loss. e.g. for −100 dBm + 4.1 dB cable loss: :POW:AMPL -95.9. | SG | :POW:AMPL <-100 + CABLE_LOSS_I> | SG output level updates |
| 4b | Read raw RSSI from the FHGW PHY for ant_i. | testmac console | show rssi <ant_i> | Raw RSSI in dBFS (e.g. −80.4) |
| 4c | Compute corrected RSSI and check tolerance. | spreadsheet / notes | corrected = raw + rssi_cal[ant_i][freq]Δ = corrected − (-100) | If ` |
| 5 | Disable the SG RF output before moving to the next antenna | SG | :OUTP:STAT OFF | SG stops transmitting |
| 6 | Antenna verdict | spreadsheet / notes | All three of 2d / 3c / 4c PASS | ant_i PASS. Any single cal point with ` |

#### Loop control (when do I go where?)

After step 6, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq) | → back to §5.4 step 1 with the next ant_i |
| All antennas done for this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW (e.g. 10 → 20 MHz at B3). Push a new startup XML, re-load FRC, refire rctul. No ACTIVE_*.xml change — same activated band. |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band (e.g. B1 → B3 in b0103). Push a different ACTIVE_*.xml. No reboot — same booted personality. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 for the first band in the new group. |
| All band-groups done | STOP. Overall verdict = PASS iff every RX antenna at every (band, BW, freq) passes all three RSSI cal points within ±1.5 dB. |

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| Raw RSSI reads ≈ noise floor (−95 dBFS-ish) at all SG levels | SG output OFF, or carrier freq doesn't match RU's UL freq | Re-verify §5.3 step 1 (UL freq, not DL) and §5.4 step 2b (:OUTP:STAT ON) |
| Corrected RSSI is off by exactly ~20 dB across all 3 cal points | Stale or wrong rssi_cal offset for this (port, freq) | Re-verify the rssi_cal table entry; ensure the freq key matches the UL freq you tuned to |
| All 3 cal points off by a consistent ~4 dB on one antenna | Wrong cable-loss value used for that antenna at the UL freq | Re-verify the calibrated cable loss for that specific antenna; cable loss differs DL vs UL |
| High cal point passes, low cal point fails by 5+ dB | PHY RSSI counter saturating or floor-clipping below ~−85 dBFS | Increase SG output for the "low" point to e.g. −90 dBm; recompute the per-port offset; confirm the PHY's RSSI dynamic range |
| RSSI value drifts > 0.5 dB while SG is steady | DU not in UL-RX mode; counters not integrating over a full LTE frame | Re-verify §5.2.5 step 4 (rctul); allow 1–2 s settle after each SG amplitude change before reading |
| Raw RSSI on TDD (B41) reads only ~10% of expected | SG transmitting continuously; FHGW only integrates during UL subframes | Verify §5.2.5 step 3 — BURSt:STAT ON must be set for TDD bands |
| Different ant_i reads same RSSI as ant_j (suspicious) | RF switch stuck or wrong bank/port routed | Re-issue §5.4 step 1 with explicit <BANK> / <PORT>; verify switch state with GET http://<SWITCH_IP>/SP6T:<BANK>:STATE? |
| All antennas pass at fm but fail at fb / ft | rssi_cal only populated for fm; offset is freq-dependent | Populate rssi_cal[port][fb_freq] and rssi_cal[port][ft_freq] (run the test once in "populate" mode to capture raw offsets at all three freqs) |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §7.4.3 — Received Signal Strength Indicator (RSSI). There is no formal 3GPP BS RSSI conformance test; the spec defines RSSI as a measurement quantity, not a requirement. This PW test is a vendor-internal readback validation.

- TS 36.214 §5.1.3 — RSSI definition (received wideband power, including thermal noise and interference, in the OFDM symbols carrying RS).

- PW automation thresholds: configuration/setup_configuration/kpi_thresholds_references/FHGW_Box_kpi_thresholds.json → test_Rx_RSSI.kpi (<TBD: confirm key name + the 3 cal points + ±1.5 dB tolerance>)

## 4.3 Rx — Reference Sensitivity Level OTA

3GPP references: TS 36.141 §7.2 (conformance) + TS 36.141 Annex F (OTA test methods, in particular F.5 EIS — Equivalent Isotropic Sensitivity) + TS 36.104 §7.2 (radio requirement) — see §4 and §7.

Scope: this document is the per-test procedure for the radiated (OTA) variant of Reference Sensitivity. The signal flows from a chamber reference antenna through free-space (and chamber path-loss) into the FHGW's radiated antenna. The pass metric is EIS (Equivalent Isotropic Sensitivity) rather than the conducted REFSENS. For the conducted variant see rx_reference_sensitivity.md. Common parameter tables (bands, frequency points, FRCs) live in the PW Conformance — Common Test Parameters Reference above. This test uses the A3-x FRC family.

### 1. Purpose

Verify the FHGW receiver, mounted as an integrated radio in the OTA chamber, achieves the minimum Equivalent Isotropic Sensitivity (EIS) such that BLER ≤ 5 % when the chamber antenna radiates at the band's EIS_3GPP limit at boresight. The same demodulator/FRC chain as the conducted test is exercised, but the signal is radiated through the chamber and corrected via the chamber's per-(port, freq, az, el) path-loss table.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / FRC tables.

Table 3-1 — Reference Sensitivity Level OTA Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna pattern points | Boresight (az=0°, el=0°) at minimum; optional off-boresight grid <TBD: PW OTA angle grid from kpi_thresholds.test_Rx_Reference_Sensitivity_Level_OTA> | deg |
| Frequency | fb / fm / ft per (band, UL BW) — see Frequency points in the Reference | MHz |
| SG Power | Computed per-angle as EIS_target + PathLoss(freq, az, el) — see §4 worked example | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference for all valid options (Common BW selections per band group is informational only). FRC waveform tracks BW per FRC selection (Rx). | MHz |
| FRC | A3-x family (A3-4 / A3-5 / A3-6 / A3-7 per BW) — see Fixed Reference Channel (FRC) selection in the Reference | — |
| BLER threshold | ≤ 5 % (with ±4 % measurement tolerance) | % |
| PW OTA EIS gate | <TBD: PW OTA EIS gate from kpi_thresholds.test_Rx_Reference_Sensitivity_Level_OTA> | dBm |

### 4. Pass/Fail criteria (PW gate)

The 3GPP requirement (TS 36.141 §F.5.3 — EIS Test Requirement) is EIS ≤ EIS_3GPP at boresight, where EIS_3GPP is the band-specific EIS limit derived from the conducted REFSENS plus the BS antenna assembly's gain budget.

The PW automation pass gate mirrors the conducted variant's framing: BLER ≤ 5 % when the chamber radiates at PW's EIS_target (the EIS the FHGW must achieve, in dBm at the antenna aperture), with the same bler_tolerance ±4 % to absorb measurement noise. The SG amplitude that produces this EIS at the FHGW is:

SG_amplitude_dBm = EIS_target_dBm + PathLoss(freq, az, el)

Test verdict: PASS only if BLER ≤ 5 % at every tested (freq, angle) point when SG drives at EIS_target + PathLoss.

PW threshold source: <TBD: PW OTA EIS gate from kpi_thresholds.test_Rx_Reference_Sensitivity_Level_OTA>.

#### 4.1 Worked example

B3 conducted reference sensitivity (from the umbrella table) = −68 dBm (b0103 group). For OTA, PW's EIS_target for B3 at boresight is <TBD: PW OTA EIS_target_B3> — assume −26.5 dBm for illustration (consistent with a chamber boresight path-loss budget of ~+41.5 dB).

Chamber boresight path-loss at B3 fm (1747.5 MHz) = 75 dB (calibrated; includes feed cable + horn gain + free-space).

Required SG amplitude at SG output:

SG_amplitude = EIS_target + PathLoss
             = -26.5 + 75
             = -101.5 + 75    (alternative framing: relative to conducted REFSENS)
             = +48.5 dBm at SG output  ←  this is the level the SG must produce

Wait — the SG cannot produce +48.5 dBm. For the OTA framing to be tractable on the bench's SG output, the path-loss budget assumes a higher-EIS PW gate than the conducted REFSENS, i.e. the FHGW radiated antenna's gain offsets enough of the path loss that the SG can drive at ~−25 to −30 dBm for boresight.

Restated with the worked-example numbers from the fact sheet:

| Quantity | Value |
| --- | --- |
| Conducted REFSENS (B3, b0103) | −101.5 dBm (reference; from 3GPP TS 36.104 §7.2) |
| Chamber boresight path-loss | 75 dB |
| Expected SG amplitude at SG output | −101.5 + 75 = −26.5 dBm |
| PW OTA gate (±2 dB tolerance) | SG threshold range [−28.5, −24.5] dBm → BLER ≤ 5 % anywhere in this window is PASS |
| Actual at bench | <TBD: measured SG amplitude where BLER = 5 %> |

B3 boresight PASS iff the SG amplitude where BLER first crosses 5 % falls inside the [−28.5, −24.5] dBm window.

Note: the per-band <TBD: PW OTA EIS_target> values and the chamber's per-freq path-loss table must both be populated before this test can issue a verdict — see §5.1 step 6 below.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test — the inner loop runs the SG-driven UL sensitivity sweep at each (az, el) angle.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SG init,     │
  │    SG external 10 MHz ref lock,          │
  │    personality verify, positioner home,  │
  │    chamber path-loss table loaded        │
  │    (NO per-port cable-loss cal — chamber │
  │     table replaces it)                   │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │      (channel-bandwidth + EARFCN)        │
  │    • load FRC waveform on SG (per BW)    │
  │    • (TDD) configure SG UL burst gating  │
  │    • fire rctul <BW> <FRC>               │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SG carrier reference freq       │
  │      (UL freq, not DL)                   │
  │    • set initial SG output power         │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per (az, el) point ◄── next point│  ← this test
  │    • drive positioner to (az, el)        │
  │    • look up PathLoss(freq, az, el)      │
  │    • set SG = EIS_target + PathLoss      │
  │    • enable SG RF output                 │
  │    • sweep SG power down until BLER >5 % │
  │    • verdict (measured ≤ PW gate?)       │
  │    • disable SG RF output                │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more (az, el) points at this    │── yes ──► back to §5.4
     │ freq?                           │
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     │  (rctul keeps running)          │
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     │  (e.g. 5 → 10 → 20 MHz at this band)     │           push new startup XML,
     │  Same activated band — no ACTIVE_*.xml.  │           re-load FRC, refire rctul
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group to test?   │── yes ──► back to §5.2
     │  (e.g. did B1 in b0103, now want B3)     │           push different ACTIVE_*.xml,
     │  Same booted personality — no reboot.    │
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     │  (e.g. did all of b0103, now want b0307) │           (see "Update FHGW EEPROM
     │  Different personality — REBOOT needed.  │           model name" appendix),
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every tested
                 (band, freq, az, el) meets
                 the PW OTA EIS gate)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. Each BW change pushes a fresh FHGW startup XML (BW + EARFCN), loads the matching FRC waveform on the SG, and fires rctul to put the DU in UL-RX mode.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm UL freq in Hz. | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_UL_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Load the FRC waveform on the SG for this BW. Substitute the FRC name per the table below. | SG | :RAD:LTE:UL:WAV "A3-5" | Panel shows the FRC name, PRB allocation, QPSK 1/3 |
| 3 | Burst-gate the SG to UL slots. TDD bands only (B41) — skip on FDD. | SG | :RAD:LTE:UL:TDD:CONF 3:RAD:LTE:UL:TDD:SSFR 8:SOUR:RAD:LTE:UL:TRIG:SOUR FRAME:SOUR:RAD:LTE:UL:BURS:STAT ON | SG transmits only during UL subframes |
| 4 | Start UL reception in test mode. Substitute the BW (MHz) and matching FRC. | testmac console | rctul 10 A3-5 | DU enters UL-RX mode; BLER counter visible (initially 100% — SG RF not enabled yet) |

FRC selection (for step 2 and step 4):

| BW | FRC |
| --- | --- |
| 5 MHz | A3-4 |
| 10 MHz | A3-5 |
| 15 MHz | A3-6 |
| 20 MHz | A3-7 |

✓ DU in UL-RX mode at this BW. → Continue to §5.3 for the first freq point.

#### 5.4 Per-angle measurement (repeat for each az/el point in your test plan)

Do these steps once per (azimuth, elevation) angle. Boresight (0, 0) is mandatory; an off-boresight grid is optional per your test plan. Each angle is a power sweep — find the lowest SG output where the DU-reported BLER stays ≤ 5 %, after applying the chamber's per-angle path-loss correction.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Drive the chamber positioner to (<AZ_DEG>, <EL_DEG>). Boresight = (0, 0). <TBD: positioner SCPI/HTTP control> | positioner controller | <TBD: e.g. :POS:AZ <AZ_DEG>; :POS:EL <EL_DEG>> then *OPC? | Positioner reports target az/el reached; mechanical settle within ~5 s |
| 2 | Look up PathLoss(freq, az, el) from the chamber calibration table. Use the freq closest to the current UL freq if exact match unavailable. | spreadsheet / notes | pl = path_loss_table[band][freq][az][el] | Numeric dB value (e.g. 75 dB for B3 fm boresight) |
| 3 | Compute SG amplitude target for this point. Use PW EIS_target for the current band (from kpi_thresholds). | spreadsheet / notes | SG_amp = EIS_target + pl (e.g. for B3 boresight: −101.5 + 75 = −26.5 dBm) | SG amplitude value ready to push |
| 4 | Set SG amplitude. Initial: drive at SG_amp plus a +5 dB safety margin above the gate, to confirm BLER ≈ 0 before sweeping down. | SG | :POW:AMPL <SG_amp + 5> | SG output level updates |
| 5 | Enable the SG RF output | SG | :OUTP:STAT ON | SG transmits |
| 6 | Verify the FHGW sees the signal at this angle. RSSI on the FHGW's PHY should rise relative to noise floor. | DU log / RU diag | (vendor-specific RSSI query) | RSSI on the active RX chain rises above noise floor |
| 7 | Sweep SG output power downward — coarse (1 dB), fine (0.6 dB), finest (0.2 dB) — until BLER first crosses 5%. Read BLER from the DU after each adjustment. | SG | Repeat :POW:AMPL <NEW_VALUE> at each step | Lowest SG output where DU BLER stays ≤ 5% across the test_duration window (default 1000 ms) |
| 8 | Compute measured EIS at this angle: EIS_measured = SG_amp_min − PathLoss(freq, az, el) | spreadsheet / notes | e.g. -27.0 − 75 = −102.0 dBm EIS | EIS in dBm |
| 9 | Compare measured EIS against the PW gate. PW gate: EIS_measured ≤ EIS_target ± 2 dB (i.e. measured ≤ EIS_target + 2 dB tolerance). EIS_3GPP reference is informational (TS 36.141 §F.5.3 per band). | spreadsheet / notes | EIS_measured ≤ EIS_target + 2 | If satisfied → angle PASS. Example: B3 boresight gate EIS_target = −101.5; measured −102.0 → PASS. |
| 10 | Disable the SG RF output before moving to the next angle | SG | :OUTP:STAT OFF | SG stops transmitting |

#### Loop control (when do I go where?)

After step 10, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More (az, el) angles at this (band, BW, freq) | → back to §5.4 step 1 with the next angle |
| All angles done for this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW. Push a new startup XML, re-load FRC, refire rctul. No ACTIVE_*.xml change — same activated band. |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band. Push a different ACTIVE_*.xml. No reboot — same booted personality. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 for the first band in the new group. |
| All band-groups done | STOP. Overall verdict = PASS iff every tested (band, freq, az, el) meets the PW OTA EIS gate. |

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| BLER = 100% even at high SG power | SG output OFF, carrier freq doesn't match RU's UL freq, or chamber path-loss table off by tens of dB | Re-verify §5.3 step 1 (UL freq, not DL), §5.4 step 5 (:OUTP:STAT ON), and §5.4 step 2 (correct PathLoss entry for current freq + angle) |
| BLER unstable at sensitivity limit | SG carrier drifting (external 10 MHz reference not locked) | Re-verify §5.1 step 3 — "EXT REF" indicator must be solid green |
| Boresight passes, off-boresight fails dramatically | Stale path-loss correction at the off-boresight angle (table only calibrated at boresight) | Re-verify path_loss_table has angle-specific entries; if not, calibrate the chamber at the desired angles before continuing |
| EIS_measured ~30 dB worse than EIS_target everywhere | Polarisation mismatch between chamber horn and FHGW antenna assembly | Re-verify §5.1 step 7 — polarisation alignment (V/V or H/H per chamber convention) |
| EIS drifts > 0.5 dB across a 10 s window | Measurement window too short; per-subframe BLER variance | Increase test_duration (default 1000 ms) or extend the manual averaging window |
| BLER on TDD (B41) stays high even at high SG power | SG transmitting continuously; jamming FHGW DL | Verify §5.2.5 step 3 — BURSt:STAT ON must be set for TDD bands |
| Positioner reports angle reached but mechanical settle still in progress | Settle time too short between step 1 and step 4 | Add 5–10 s wait after positioner step 1 before applying RF |
| BLER reads 0 % at all power levels (suspicious) | DU not actually in UL-RX mode; FRC mismatch between SG and DU | Re-verify §5.2.5 — same FRC name on both SG (step 2) and DU (step 4) |
| Chamber door interlock trips mid-test | Door not fully closed / absorber curtain dislodged | Stop, re-close, re-arm interlock, restart from current angle |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §7.2 Reference Sensitivity Level, §7.2.3 Test requirements, Annex F OTA test methods — in particular §F.5 Equivalent Isotropic Sensitivity (EIS) and §F.5.3 EIS Test Requirement.

- TS 36.141 §4.9.1 Test frequencies (fb / fm / ft), Annex A Fixed Reference Channels for receiver tests.

- TS 36.104 §7.2 Reference sensitivity level (the REFSENS table that the EIS budget is derived from).

- PW automation thresholds: configuration/setup_configuration/kpi_thresholds_references/FHGW_Box_kpi_thresholds.json → test_Rx_Reference_Sensitivity_Level_OTA.kpi (<TBD: confirm key name + PW OTA EIS gate + path-loss table source>)

## 4.4 Rx — Adjacent Channel Selectivity

3GPP references: TS 36.141 §7.5.2 (conformance — ACS) + TS 36.104 §7.5 (radio requirement) + TS 36.141 Annex A (FRC definitions for the wanted signal) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, FRCs, sensitivity limits) live in the PW Conformance — Common Test Parameters Reference above. This test injects two signals simultaneously — a wanted signal on the FHGW's RX channel, and an adjacent-channel interferer offset by ±BW.

### 1. Purpose

Verify the FHGW receiver can reject an unwanted modulated signal in the adjacent channel (carrier offset ±BW) while still demodulating a wanted uplink signal at 6 dB above reference sensitivity. The figure of merit is ACS (Adjacent Channel Selectivity) = P_interferer − P_wanted at the interferer power where wanted-signal BLER first crosses 5 %. Conducted-RF measurement, two RF sources combined into the FHGW RX port.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / FRC tables.

Table 3-1 — Adjacent Channel Selectivity Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared RX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Frequency (wanted) | fb / fm / ft per (band, UL BW) — see Frequency points in the Reference | MHz |
| Frequency (interferer) | Wanted ± BW (one channel BW offset, adjacent channel center) | MHz |
| Wanted SG power | sens_3GPP + 6 dB (6 dB above reference sensitivity limit for the band) | dBm |
| Interferer SG power | Swept; initial value ≈ wanted + 30 dB, step down to find BLER = 5 % crossing | dBm |
| BW | Per (band, BW) — see Supported bands in the Reference for all valid options | MHz |
| FRC (wanted) | A3-x family (A3-4 / A3-5 / A3-6 / A3-7 per BW) — see Fixed Reference Channel (FRC) selection in the Reference | — |
| Interferer modulation | <TBD: confirm CW vs LTE-modulated> — 3GPP §7.5.2 says CW; PW automation may use modulated LTE | — |
| BLER threshold | ≤ 5 % (with ±4 % measurement tolerance) at the wanted carrier | % |
| ACS pass gate | ACS ≥ 46 dB (3GPP §7.5.2.5, E-UTRA Wide Area BS) — PW gate <TBD: confirm kpi_thresholds.test_lte_Rx_ACS — likely 46 + 3 dB margin> | dB |

### 4. Pass/Fail criteria (PW gate)

Per TS 36.141 §7.5.2.5, an E-UTRA Wide Area BS receiver must achieve ACS ≥ 46 dB. PW automation applies the same bler_tolerance of ±4 % at the wanted carrier as in the Reference Sensitivity test — see <TBD: kpi_thresholds_references/FHGW_Box_kpi_thresholds.json → test_lte_Rx_ACS.test_parameters>.

The measurement is a sweep over interferer power: hold the wanted SG fixed at sens + 6 dB and read BLER while increasing the interferer SG power. The interferer power at which wanted-signal BLER first crosses 5 % is P_interferer_threshold. ACS is then computed as:

ACS = P_interferer_threshold − P_wanted    (dB)

Test verdict: PASS only if every RX port reports ACS ≥ PW gate at every (band, BW, freq).

#### 4.1 Worked example (B3 in b0103 group, BW = 10 MHz, freq point fm)

| Item | Value |
| --- | --- |
| Band, BW, freq | B3 (b0103), 10 MHz, fm UL = 1747.5 MHz |
| Reference sensitivity limit | −101.5 dBm |
| Wanted SG power (sens + 6) | −95.5 dBm at carrier 1747.5 MHz, FRC A3-5, modulated LTE UL |
| Interferer SG carrier | 1747.5 + 10 = 1757.5 MHz (one BW above) |
| Interferer modulation | <TBD: CW or LTE-modulated> |
| Sweep | start interferer at −66 dBm (= wanted + 30 dB), step up in 1 dB increments while watching BLER on testmac |
| BLER crossing | At P_interferer = −47 dBm, wanted-signal BLER first exceeds 5 % |
| ACS | −47 − (−95.5) = **48.5 dB** |
| PW gate (assumed) | 49 dB |
| Verdict | 48.5 < 49 → **FAIL** (would be PASS if gate were 48 dB) |

Repeat the measurement with the interferer below the wanted carrier (1747.5 − 10 = 1737.5 MHz) — both sides must pass independently.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test — the inner loop sweeps the interferer SG and looks for the wanted-signal BLER = 5 % crossing.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SG A + SG B  │
  │    init, both SGs locked to ext 10 MHz,  │
  │    personality verify, RF switch ping,   │
  │    cable-loss cal per (port, UL freq)    │
  │    WITH combiner in-line                 │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │      (channel-bandwidth + EARFCN)        │
  │    • load FRC A3-x on SG A (wanted)      │
  │    • set interferer modulation on SG B   │
  │    • (TDD) configure UL burst gating     │
  │    • fire rctul <BW> <FRC>               │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SG A carrier (UL freq)          │
  │    • set SG A power = sens + 6 dB        │
  │    • set SG B carrier = wanted ± BW      │
  │    • set SG B initial power              │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna    ◄─── next antenna │  ← this test
  │    • route RF switch                     │
  │    • adjust BOTH SGs for cable loss      │
  │    • enable SG A + SG B RF output        │
  │    • binary-search SG B power for        │
  │      wanted-signal BLER = 5 % crossing   │
  │    • compute ACS = P_int − P_wanted      │
  │    • verdict (ACS ≥ PW gate?)            │
  │    • repeat with interferer on OTHER     │
  │      side (−BW vs +BW)                   │
  │    • disable both SG outputs             │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     │  (rctul keeps running)          │
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     │  (e.g. 5 → 10 → 20 MHz at this band)     │           push new startup XML,
     │  Same activated band — no ACTIVE_*.xml.  │           re-load FRC, refire rctul
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group to test?   │── yes ──► back to §5.2
     │  (e.g. did B1 in b0103, now want B3)     │           push different ACTIVE_*.xml,
     │  Same booted personality — no reboot.    │
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     │  (e.g. did all of b0103, now want b0307) │           (see "Update FHGW EEPROM
     │  Different personality — REBOOT needed.  │           model name" appendix),
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every RX antenna
                 at every (band, BW, freq), with the
                 interferer on BOTH sides, meets the
                 PW ACS pass gate)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. Each BW change pushes a fresh FHGW startup XML (BW + EARFCN), loads the matching A3-x FRC waveform on SG A (wanted), and prepares SG B (interferer) for the adjacent-channel waveform. Then fires rctul to put the DU in UL-RX mode.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 0a | SG #2 init (skip if already initialized this session): Reset SG B (interferer) and lock to the SAME external 10 MHz reference SG A is using. One-time-per-session — running ACS → ICS → NB blocking back-to-back? Skip this once SG B is already on the lab time base. | SG B | *RST*OPC?:ROSC:SOUR EXT:ROSC:EXT:FREQ 10MHZ | *OPC? returns 1; EXT REF solid on SG B |
| 0b | SG #2 init (skip if already initialized this session): Configure SG B (interferer) modulation, set adjacent-channel carrier to wanted + BW, set initial low power, leave RF OFF. If CW per 3GPP §7.5.2 — clear modulation. If LTE-modulated per PW automation choice — load same A3-x FRC as SG A. <TBD: confirm CW vs LTE-modulated for PW gate> SG B carrier and power are refined in §5.3 step 3a–3b; this is the initial-state setup. | SG B | CW path: :RAD:STAN:PRES NONE:OUTP:MOD:STAT OFFLTE path: :RAD:LTE:LINK UL:RAD:LTE:LINK:DUPL FDD:RAD:LTE STAT ON:RAD:LTE:UL:WAV "A3-5"then on either path::FREQ:CW <UL_FREQ_HZ + BW_HZ>:POW:AMPL -80:OUTP:STAT OFF | Panel shows the chosen modulation (CW or LTE UL); carrier at wanted + BW; low power; RF OFF |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm UL freq in Hz. | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_UL_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Load the FRC A3-x waveform on SG A (wanted). Substitute the FRC name per the table below. | SG A | :RAD:LTE:UL:WAV "A3-5" | Panel shows the FRC name, PRB allocation, QPSK 1/3 |
| 3 | Burst-gate SG A to UL slots. TDD bands only (B41) — skip on FDD. | SG A | :RAD:LTE:UL:TDD:CONF 3:RAD:LTE:UL:TDD:SSFR 8:SOUR:RAD:LTE:UL:TRIG:SOUR FRAME:SOUR:RAD:LTE:UL:BURS:STAT ON | SG A transmits only during UL subframes |
| 4 | Re-load SG B interferer waveform if BW changed since §5.2.5 step 0b. FRC name tracks BW per the table below — re-issue only the :RAD:LTE:UL:WAV (or CW disable) for this BW. <TBD: confirm PW automation choice> | SG B | CW: :OUTP:MOD:STAT OFFLTE: :RAD:LTE:UL:WAV "A3-5" | Panel shows the chosen interferer waveform |
| 5 | Start UL reception in test mode. Substitute the BW (MHz) and matching FRC. | testmac console | rctul 10 A3-5 | DU enters UL-RX mode; BLER counter visible (initially 100 % — SG A RF not enabled yet) |

FRC selection (for step 2, step 4 (when LTE-modulated), and step 5):

| BW | FRC |
| --- | --- |
| 5 MHz | A3-4 |
| 10 MHz | A3-5 |
| 15 MHz | A3-6 |
| 20 MHz | A3-7 |

✓ DU in UL-RX mode at this BW; both SGs primed. → Continue to §5.3 for the first freq point.

#### 5.4 Per-antenna measurement (repeat for each antenna)

Do these steps once per antenna in the band's active RX set. Each antenna's measurement is an interferer power sweep at fixed wanted power — find the interferer power where wanted-signal BLER first crosses 5 %, then compute ACS = P_interferer − P_wanted. Repeat on the OTHER side of the wanted carrier (interferer at wanted − BW) — ACS must pass on both sides.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RX switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 |
| 2 | Adjust SG A output for this antenna's cable+combiner loss. Add the per-port loss (positive dB from cable_loss.json) to the wanted level. | SG A | :POW:AMPL <SENS_PLUS_6 + CABLE_LOSS_I> (e.g. −62 + 7.6 = −54.4 → :POW:AMPL -54.4) | SG A output updates |
| 3a | Adjust SG B output for the same per-port loss. SG B compensates with the same value — both go through the same combiner/splitter/switch chain. | SG B | :POW:AMPL <INIT_INTERFERER + CABLE_LOSS_I> (e.g. −32 + 7.6 = −24.4 → :POW:AMPL -24.4) | SG B output updates |
| 3b | Enable SG A (wanted) RF output | SG A | :OUTP:STAT ON | SG A transmits |
| 4 | Wait 2 s, verify wanted-only baseline BLER ≤ 5 %. Without the interferer the receiver should easily demod sens + 6 dB. | testmac console | <TBD: exact testmac command for BLER readback during Rx tests — e.g. dbg_bler or show bler> | BLER ≈ 0 % (or ≤ 5 %). If baseline already > 5 %, fix cable loss / SG A power before continuing. |
| 5 | Enable SG B (interferer) RF output | SG B | :OUTP:STAT ON | SG B transmits; combined signal hits the RX port |
| 6 | Read BLER. Coarse-sweep SG B power up 5 dB per step while BLER < 5 %. Stop when BLER first crosses 5 %. | SG B | Repeat :POW:AMPL <NEW_VALUE> then check BLER | First crossing identifies the coarse threshold P_int_coarse |
| 7 | Binary-search SG B power between P_int_coarse − 5 dB and P_int_coarse in 0.5 dB increments until BLER = 5 % ± bler_tolerance (±4 %) settles | SG B | Repeat :POW:AMPL <NEW_VALUE> at each step | P_interferer_threshold (cable-loss corrected) — the SG B output at which BLER first equals 5 % |
| 8 | Disable SG B output before flipping side | SG B | :OUTP:STAT OFF | SG B stops transmitting |
| 9 | Repeat steps 3a–8 with SG B on the lower adjacent channel (wanted − BW). Re-do §5.3 step 3a with <UL_FREQ_HZ − BW_HZ> first. Record P_int_threshold_lower. | SG B + testmac | as steps 3a–8 | Second threshold value for the OTHER side |
| 10 | Compute ACS_upper = P_int_threshold_upper − P_wanted and ACS_lower = P_int_threshold_lower − P_wanted (both at SG output port — cable loss is on both, so it cancels). | spreadsheet / notes | e.g. −47 − (−62) = 15 dB(sample only — real values depend on RU) | Two ACS values |
| 11 | Compare against PW gate. PASS only if BOTH ACS values ≥ PW gate (3GPP demands ≥ 46 dB; PW gate may add margin — <TBD: confirm kpi_thresholds.test_lte_Rx_ACS>). | spreadsheet / notes | min(ACS_upper, ACS_lower) ≥ PW_gate | If true → ant_i PASS. Example: PW gate = 49 dB, measured ACS = 48.5 dB → 48.5 < 49 → FAIL. |
| 12 | Disable SG A output before moving to the next antenna | SG A | :OUTP:STAT OFF | SG A stops transmitting |

#### Loop control (when do I go where?)

After step 12, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq) | → back to §5.4 step 1 with the next ant_i |
| All antennas done for this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW (e.g. 10 → 20 MHz at B3). Push a new startup XML, re-load FRC on both SGs, refire rctul. No ACTIVE_*.xml change — same activated band. |
| All BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band (e.g. B1 → B3 in b0103). Push a different ACTIVE_*.xml. No reboot — same booted personality. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 for the first band in the new group. |
| All band-groups done | STOP. Overall verdict = PASS iff every RX antenna at every (band, BW, freq), with the interferer on both sides, meets the PW ACS pass gate. |

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| Wanted-only baseline BLER > 5 % at sens + 6 dB | Wanted power too low, FRC mismatch, or wrong UL freq | Re-verify §5.3 step 1–2; ensure SG A FRC matches DU FRC in §5.2.5 |
| ACS pass on upper-adjacent but fail on lower-adjacent (or vice versa) | RX filter asymmetry, common in real silicon — flag for HW team but still a FAIL | Confirm both sides measured; record both; HW debug |
| BLER unstable around the 5 % crossing | SG carrier drifting, or bler_tolerance window too tight | Verify both SGs locked to ext 10 MHz (§5.1 steps 3–4); widen measurement window |
| ACS value suspiciously high (> 70 dB) | SG B output OFF or carrier offset wrong | Verify §5.4 step 5 (:OUTP:STAT ON on SG B), §5.3 step 3a (carrier = wanted ± BW exactly) |
| ACS value suspiciously low (< 20 dB) | Combiner port leakage, SG A picking up SG B image, or wanted-carrier overload | Check combiner isolation spec; reduce SG B initial power; verify cable loss includes combiner |
| BLER on TDD (B41) stays high even with SG B off | SG A transmitting continuously, jamming during DL slots | Verify §5.2.5 step 3 — BURSt:STAT ON must be set for SG A on TDD; also confirm SG B is gated identically if LTE-modulated |
| Both ACS values match the canonical Reference Sensitivity threshold exactly | SG B never actually drove the receiver — likely cabling fault | Power-meter the combined output; confirm SG B contributes |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §7.5.2 Adjacent Channel Selectivity (ACS), §7.5.2.5 Test requirements (ACS ≥ 46 dB for Wide Area BS), §4.9.1 Test frequencies (fb / fm / ft), Annex A Fixed Reference Channels for receiver tests.

- TS 36.104 §7.5 Adjacent channel selectivity and narrow-band blocking (the radio-spec side — sets the BS class definitions).

- PW automation thresholds: <TBD: configuration/setup_configuration/kpi_thresholds_references/FHGW_Box_kpi_thresholds.json → test_lte_Rx_ACS.kpi>

## 4.5 Rx — In-Channel Selectivity

3GPP references: TS 36.141 §7.5.1 (conformance — ICS) + TS 36.104 §7.5 (radio requirement) + TS 36.141 Annex A (FRC definitions for the wanted signal) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, FRCs, sensitivity limits) live in the PW Conformance — Common Test Parameters Reference above. This test injects two LTE-modulated signals into the same channel — a wanted signal on the middle PRBs, and an interferer on the edge PRBs of the same carrier.

BW restriction: ICS is defined for 10 MHz and 20 MHz only — skip 1.4 / 3 / 5 / 15 MHz BWs. <TBD: confirm with PW automation that BW filter rejects all but 10/20 for test_lte_Rx_ICS>

### 1. Purpose

Verify the FHGW receiver can demodulate a low-power wanted signal on the middle PRBs of the channel while a higher-power in-channel interferer occupies the edge PRBs of the same carrier. The figure of merit is ICS (In-Channel Selectivity) = P_interferer − P_wanted at the interferer power where wanted-signal BLER (on the middle PRBs) first crosses 5 %. Both signals occupy the same channel BW; the test stresses the receiver's filter selectivity within the band, not adjacent-band rejection.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / FRC tables.

Table 3-1 — In-Channel Selectivity Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared RX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Frequency (both signals) | fb / fm / ft per (band, UL BW) — see Frequency points in the Reference. Same carrier for wanted and interferer. | MHz |
| Wanted SG power | Sensitivity-level — see Reference sensitivity level in the Reference. e.g. ≈ sens_3GPP (no +6 dB offset for ICS) | dBm |
| Interferer SG power | Swept; initial value ≈ wanted + 15 dB, step to find BLER = 5 % crossing | dBm |
| BW | 10 MHz and 20 MHz only — see Supported bands in the Reference. <TBD: confirm BW restriction with PW automation> | MHz |
| FRC (wanted) | Custom: A3-x base with middle PRBs only. e.g. for 10 MHz / A3-5 base, PRBs 10–15 (5 RBs). <TBD: confirm exact PRB span for PW automation wanted-signal profile> | — |
| FRC (interferer) | Custom: A3-x base with edge PRBs only. e.g. for 10 MHz / A3-5 base, PRBs 0–4 + 45–49. <TBD: confirm exact PRB span for interferer profile> | — |
| BLER threshold | ≤ 5 % (with ±4 % measurement tolerance) at the wanted carrier (middle PRBs) | % |
| ICS pass gate | ICS ≥ +9.5 dB (3GPP §7.5.1.5, E-UTRA Wide Area BS) — PW gate <TBD: confirm kpi_thresholds.test_lte_Rx_ICS> | dB |

### 4. Pass/Fail criteria (PW gate)

Per TS 36.141 §7.5.1.5, an E-UTRA Wide Area BS receiver must achieve ICS ≥ +9.5 dB — a much tighter margin than ACS (≥ 46 dB) because the interferer is in-channel and competes with the wanted signal through the same receive filter. PW automation applies the same bler_tolerance of ±4 % at the wanted middle-PRB carrier as in the Reference Sensitivity test — see <TBD: kpi_thresholds_references/FHGW_Box_kpi_thresholds.json → test_lte_Rx_ICS.test_parameters>.

The measurement is a sweep over interferer power: hold the wanted SG fixed at sensitivity level on the middle PRBs and read BLER while increasing the interferer SG power on the edge PRBs. The interferer power at which middle-PRB BLER first crosses 5 % is P_interferer_threshold. ICS is then computed as:

ICS = P_interferer_threshold − P_wanted    (dB)

Test verdict: PASS only if every RX port reports ICS ≥ PW gate at every (band, BW ∈ {10, 20 MHz}, freq).

#### 4.1 Worked example (B3 in b0103 group, BW = 10 MHz, freq point fm)

| Item | Value |
| --- | --- |
| Band, BW, freq | B3 (b0103), 10 MHz, fm UL = 1747.5 MHz |
| Reference sensitivity limit (canonical) | −101 dBm (per umbrella reference table, pre-DSA) |
| Wanted SG power (middle PRBs, sens-level) | −101 dBm at carrier 1747.5 MHz, middle PRBs 10–15, FRC A3-5 (PRB-subset profile) |
| Interferer SG carrier | 1747.5 MHz (same as wanted) |
| Interferer PRBs | Edge: PRBs 0–4 + 45–49 |
| Sweep | start interferer at −86 dBm (= wanted + 15 dB), step up 1 dB while watching middle-PRB BLER on testmac |
| BLER crossing | At P_interferer = −91 dBm, middle-PRB BLER first exceeds 5 % |
| ICS | −91 − (−101) = **+10 dB** |
| PW gate (3GPP min) | +9.5 dB |
| Verdict | 10 ≥ 9.5 → **PASS** |

The ICS measurement does not sweep on both adjacent sides like ACS — there is only one configuration (middle vs edge in the same carrier). Re-run per (BW, freq, antenna).

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test — the inner loop sweeps the interferer SG (edge PRBs) and looks for the middle-PRB BLER = 5 % crossing.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SG A + SG B  │
  │    init, both SGs locked to ext 10 MHz,  │
  │    personality verify, RF switch ping,   │
  │    cable-loss cal per (port, UL freq)    │
  │    WITH combiner in-line                 │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW (10 or 20 MHz only)       │  ← this test
  │           ◄── next BW                    │
  │    • edit + push oru_startup_slave.xml   │
  │      (channel-bandwidth + EARFCN)        │
  │    • load middle-PRB FRC profile on SG A │
  │    • load edge-PRB FRC profile on SG B   │
  │    • (TDD) configure UL burst gating     │
  │    • fire rctul <BW> <FRC>               │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set BOTH SGs to same UL freq        │
  │    • set SG A (wanted) power = sens      │
  │    • set SG B (interferer) initial power │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna    ◄─── next antenna │  ← this test
  │    • route RF switch                     │
  │    • adjust BOTH SGs for cable loss      │
  │    • enable SG A + SG B RF output        │
  │    • binary-search SG B (edge PRBs)      │
  │      power for middle-PRB BLER = 5 %     │
  │    • compute ICS = P_int − P_wanted      │
  │    • verdict (ICS ≥ PW gate?)            │
  │    • disable both SG outputs             │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     │  (rctul keeps running)          │
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     │  (10 → 20 MHz only — skip others)        │           push new startup XML,
     │  Same activated band — no ACTIVE_*.xml.  │           re-load PRB profiles, refire
     └────────────┬─────────────────────────────┘           rctul
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group to test?   │── yes ──► back to §5.2
     │  (e.g. did B1 in b0103, now want B3)     │           push different ACTIVE_*.xml,
     │  Same booted personality — no reboot.    │
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     │  (e.g. did all of b0103, now want b0307) │           (see "Update FHGW EEPROM
     │  Different personality — REBOOT needed.  │           model name" appendix),
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every RX antenna
                 at every (band, BW ∈ {10,20}, freq)
                 meets the PW ICS pass gate)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW ∈ {10, 20 MHz})

Do these steps once per BW within the current band combo — ICS supports 10 MHz and 20 MHz only. Each BW change pushes a fresh FHGW startup XML (BW + EARFCN), loads the middle-PRB FRC profile on SG A and the edge-PRB FRC profile on SG B, then fires rctul to put the DU in UL-RX mode. The edge-PRB profile is BW-dependent — SG #2 init below lives inside this per-BW block.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 0a | SG #2 init (skip if already initialized this session AND BW unchanged): Reset SG B (interferer) and lock to the SAME external 10 MHz reference SG A is using. One-time-per-session — running ICS → ACS → NB blocking back-to-back? Skip this once SG B is already on the lab time base. | SG B | *RST*OPC?:ROSC:SOUR EXT:ROSC:EXT:FREQ 10MHZ | *OPC? returns 1; EXT REF solid on SG B |
| 0b | SG #2 init (re-do whenever BW changes): Select LTE UL personality on SG B (interferer is LTE-modulated, in-channel — edge PRBs), set carrier to the wanted UL freq (same channel as SG A), set initial low power, leave RF OFF. The edge-PRB FRC profile loaded in step 3 below depends on BW — that's why this init is per-BW. SG B carrier is refined in §5.3 step 1b. | SG B | :RAD:LTE:LINK UL:RAD:LTE:LINK:DUPL FDD:RAD:LTE STAT ON:FREQ:CW <UL_FREQ_HZ>:POW:AMPL -80:OUTP:STAT OFF | Panel shows LTE UL FDD (or LTE UL TDD for B41); carrier at wanted freq; low power; RF OFF |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz, or 20000000 for 20 MHz) and <center-of-channel-bandwidth> to the band's fm UL freq in Hz. | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_UL_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Load the middle-PRB wanted-signal FRC profile on SG A. Substitute the profile name per the table below — A3-x base with PRBs trimmed to the middle 5–6 RBs. <TBD: confirm exact PRB span (e.g. 10–15 for 10 MHz, 45–50 for 20 MHz) used by PW automation> | SG A | :RAD:LTE:UL:WAV "A3-5-MID" (or vendor-equivalent custom profile name) | Panel shows the FRC name, middle-PRB allocation, QPSK 1/3 |
| 3 | Load the edge-PRB interferer FRC profile on SG B. A3-x base with PRBs trimmed to the bottom and top edges. <TBD: confirm exact PRB span (e.g. 0–4 + 45–49 for 10 MHz) used by PW automation> | SG B | :RAD:LTE:UL:WAV "A3-5-EDGE" (or vendor-equivalent custom profile name) | Panel shows the FRC name, edge-PRB allocation, QPSK 1/3 |
| 4 | Burst-gate both SGs to UL slots. TDD bands only (B41) — skip on FDD. | SG A and SG B | On EACH::RAD:LTE:UL:TDD:CONF 3:RAD:LTE:UL:TDD:SSFR 8:SOUR:RAD:LTE:UL:TRIG:SOUR FRAME:SOUR:RAD:LTE:UL:BURS:STAT ON | Both SGs transmit only during UL subframes; UL frames align |
| 5 | Start UL reception in test mode. Substitute the BW (MHz) and matching base FRC. | testmac console | rctul 10 A3-5 (or rctul 20 A3-7) | DU enters UL-RX mode; BLER counter visible (initially 100 % — SG A RF not enabled yet) |

FRC base selection (for step 2, step 3, and step 5 — both PRB profiles are derived from the same base):

| BW | Base FRC | Wanted PRB subset (middle) | Interferer PRB subset (edge) |
| --- | --- | --- | --- |
| 10 MHz | A3-5 | PRBs 22–27 (≈6 RBs middle) <TBD> | PRBs 0–4 + 45–49 <TBD> |
| 20 MHz | A3-7 | PRBs 47–52 (≈6 RBs middle) <TBD> | PRBs 0–4 + 95–99 <TBD> |

✓ DU in UL-RX mode at this BW; both SGs primed with their PRB-subset waveforms. → Continue to §5.3 for the first freq point.

#### 5.4 Per-antenna measurement (repeat for each antenna)

Do these steps once per antenna in the band's active RX set. Each antenna's measurement is an interferer power sweep at fixed wanted power — find the interferer power where middle-PRB BLER first crosses 5 %, then compute ICS = P_interferer − P_wanted.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RX switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 |
| 2 | Adjust SG A output for this antenna's cable+combiner loss. Add the per-port loss (positive dB from cable_loss.json) to the wanted level. | SG A | :POW:AMPL <SENS + CABLE_LOSS_I> (e.g. −68 + 7.6 = −60.4 → :POW:AMPL -60.4) | SG A output updates |
| 3a | Adjust SG B output for the same per-port loss. SG B compensates with the same value — both go through the same combiner/splitter/switch chain. | SG B | :POW:AMPL <INIT_INTERFERER + CABLE_LOSS_I> (e.g. −53 + 7.6 = −45.4 → :POW:AMPL -45.4) | SG B output updates |
| 3b | Enable SG A (wanted, middle PRBs) RF output | SG A | :OUTP:STAT ON | SG A transmits |
| 4 | Wait 2 s, verify wanted-only baseline middle-PRB BLER ≤ 5 %. Without the interferer the receiver should demod the middle PRBs at sens level. | testmac console | <TBD: exact testmac command for BLER readback during Rx tests — e.g. dbg_bler or show bler> | BLER ≤ 5 %. If baseline already > 5 %, fix cable loss / SG A power before continuing. |
| 5 | Enable SG B (interferer, edge PRBs) RF output | SG B | :OUTP:STAT ON | SG B transmits; combined signal hits the RX port; both PRB subsets co-exist in the same carrier |
| 6 | Read middle-PRB BLER. Coarse-sweep SG B power up 2 dB per step while BLER < 5 %. Stop when BLER first crosses 5 %. | SG B | Repeat :POW:AMPL <NEW_VALUE> then check BLER | First crossing identifies the coarse threshold P_int_coarse |
| 7 | Binary-search SG B power between P_int_coarse − 2 dB and P_int_coarse in 0.25 dB increments until BLER = 5 % ± bler_tolerance (±4 %) settles | SG B | Repeat :POW:AMPL <NEW_VALUE> at each step | P_interferer_threshold (cable-loss corrected) — the SG B output at which middle-PRB BLER first equals 5 % |
| 8 | Compute ICS = P_interferer_threshold − P_wanted (at SG output port — cable loss is on both, so it cancels). | spreadsheet / notes | e.g. −58 − (−68) = +10 dB | One ICS value for this (antenna, BW, freq) |
| 9 | Compare against PW gate. 3GPP minimum is +9.5 dB; <TBD: confirm kpi_thresholds.test_lte_Rx_ICS for PW-internal margin>. | spreadsheet / notes | ICS ≥ PW_gate | If true → ant_i PASS. Example: PW gate = +9.5 dB, measured ICS = +10 dB → 10 ≥ 9.5 → PASS. |
| 10 | Disable both SG outputs before moving to the next antenna | SG A and SG B | On each: :OUTP:STAT OFF | Both SGs stop transmitting |

#### Loop control (when do I go where?)

After step 10, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas to measure at this (band, BW, freq) | → back to §5.4 step 1 with the next ant_i |
| All antennas done for this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1a for the next freq point |
| All freq points done, other ICS-supported BW for this band combo (10 → 20 MHz only) | → back to §5.2.5 step 1 for the other BW. Push a new startup XML, re-load both PRB-subset waveforms, refire rctul. No ACTIVE_*.xml change — same activated band. |
| Both 10 and 20 MHz BWs done for this band, more bands in this band-group | → back to §5.2 step 1 for the next band (e.g. B1 → B3 in b0103). Push a different ACTIVE_*.xml. No reboot — same booted personality. |
| All bands done in this group, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 for the first band in the new group. |
| All band-groups done | STOP. Overall verdict = PASS iff every RX antenna at every (band, BW ∈ {10, 20 MHz}, freq) meets the PW ICS pass gate. |

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| Wanted-only baseline middle-PRB BLER > 5 % at sens level | Wanted power too low, wrong PRB subset (middle PRBs not actually carrying data), or wrong UL freq | Re-verify §5.3 step 1a–2; on SG A confirm the middle-PRB profile is selected, not the edge-PRB one |
| ICS very low (< 3 dB) | Wanted and interferer PRB subsets overlap — receiver sees them as one co-channel sum | Re-verify §5.2.5 step 2 (wanted = middle only) and step 3 (interferer = edge only); if same waveform loaded on both SGs, fix |
| ICS impossibly high (> 30 dB) | Interferer SG output OFF, or edge-PRB profile is empty | Verify §5.4 step 5 (:OUTP:STAT ON on SG B); spot-check SG B panel shows edge-PRB allocation |
| BLER bounces unstable around 5 % crossing | Inter-SG timing slip — both transmit on the same channel; small drift between them swaps PRB content | Re-verify §5.1 steps 3–4 — both SGs MUST share the same 10 MHz reference; check on each SG EXT REF solid |
| Test attempted at 5 MHz / 15 MHz / 1.4 / 3 MHz BW and won't converge | ICS is defined for 10 MHz and 20 MHz only per source survey — other BWs not supported | Skip the BW; only run 10 MHz and 20 MHz for ICS |
| BLER on TDD (B41) stays high even with SG B off | One or both SGs transmitting continuously, jamming during DL slots | Verify §5.2.5 step 4 — BURSt:STAT ON must be set on both SGs for TDD |
| ICS reads higher than the 3GPP ACS limit | Suspicious — ICS is harder than ACS (in-channel = no filter selectivity advantage). Likely cabling or PRB profile error | Re-confirm interferer is in-channel (same carrier as wanted), edge PRBs only |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §7.5.1 In-Channel Selectivity (ICS), §7.5.1.5 Test requirements (ICS ≥ +9.5 dB for Wide Area BS), §4.9.1 Test frequencies (fb / fm / ft), Annex A Fixed Reference Channels for receiver tests.

- TS 36.104 §7.5 Adjacent channel selectivity and narrow-band blocking (the radio-spec side — sets the BS class definitions).

- PW automation thresholds: <TBD: configuration/setup_configuration/kpi_thresholds_references/FHGW_Box_kpi_thresholds.json → test_lte_Rx_ICS.kpi>

## 4.6 Rx — Narrowband Blocking

3GPP references: TS 36.141 §7.5.3 Blocking — Narrowband blocking (in-band blocking case 2) + TS 36.104 §7.5 Blocking characteristics + TS 36.141 Annex A (FRC definitions) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, FRCs, sensitivity limits) live in the PW Conformance — Common Test Parameters Reference above. Two RF sources are combined into the FHGW RX port: a wanted modulated LTE UL signal at sens_3GPP + 6 dB, plus a CW blocker at a configurable narrowband offset.

### 1. Purpose

Verify the FHGW receiver still meets BLER ≤ 5 % on the wanted carrier when a single narrowband (CW) interferer is injected at a defined offset from the band edge. The narrowband blocking test characterises the receiver's selectivity against close-in interferers — the kind produced by adjacent operators' emissions or by spurs from co-sited radios.

Conducted-RF measurement: SG #1 (wanted) and SG #2 (blocker) feed a power combiner whose output drives the FHGW RX port under test; FHGW reports BLER on the wanted signal.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / FRC tables.

Table 3-1 — Narrowband Blocking Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared RX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Wanted Frequency | fb / fm / ft per (band, UL BW) — see Frequency points in the Reference | MHz |
| Wanted SG Power | sens_3GPP + 6 dB (i.e. 6 dB above the band's sensitivity limit) | dBm |
| Wanted Modulation | A3-x FRC (LTE UL, QPSK 1/3, full PRB) — see FRC selection (Rx) in the Reference | — |
| Blocker Frequency | Per 3GPP TS 36.141 Table 7.5.3.5-2 (single offset point per band). <TBD: confirm exact NB blocker offset from current 3GPP release for each band> | MHz |
| Blocker Power | Per 3GPP TS 36.141 Table 7.5.3.5-2 — typically wanted + ~80 dB. <TBD: per-band blocker power value> | dBm |
| Blocker Modulation | CW (per 3GPP §7.5.3). <TBD: confirm modulation for NB blocker — CW per 3GPP> | — |
| BW | Per (band, BW) — see Supported bands in the Reference. FRC waveform tracks BW per FRC selection (Rx). | MHz |
| BLER threshold | ≤ 5 % (with ±4 % measurement tolerance) | % |

### 4. Pass/Fail criteria

Per TS 36.141 §7.5.3, when the wanted signal is held at REFSENS + 6 dB and a CW narrowband blocker is applied at the offset and level defined in Table 7.5.3.5-2, the throughput must remain ≥ 95% of the reference measurement channel's maximum — equivalently, BLER ≤ 5% on the wanted carrier.

PW automation evaluates the verdict against the PW gate, not against the wider 3GPP limit:

PW pass gate (per RX port, per (band, freq, blocker point)): BLER ≤ 5 % with both SGs enabled.

PW threshold storage: <TBD: kpi_thresholds.test_lte_Rx_NB_blocking — per-point pass criteria> (per source survey: "kpi_thresholds frequency-swept").

Test verdict: PASS only if every RX port meets BLER ≤ 5 % at every (band, freq, blocker point) under test.

#### 4.1 Worked example (B3, b0103, fm)

| Quantity | Value |
| --- | --- |
| Band group | b0103 |
| Band | B3 |
| Wanted UL freq (fm) | 1747.5 MHz |
| Band sensitivity limit (PW) | −101.5 dBm <TBD: confirm sens_3GPP basis vs PW limit for NB blocking> |
| Wanted SG power | −101.5 + 6 = −95.5 dBm |
| Blocker freq | 2300 MHz (single NB blocker point) |
| Blocker power | −15 dBm (CW) |
| Measured BLER | 3.2 % |
| Verdict | 3.2 % ≤ 5 % → PASS |

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test — the inner loop runs the dual-source blocking BLER check.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, SG init,     │
  │    SG external 10 MHz ref lock,          │
  │    personality verify, RF switch ping,   │
  │    cable-loss cal per (port, UL freq)    │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │      (channel-bandwidth + EARFCN)        │
  │    • load A3-x FRC waveform on SG #1     │
  │      (wanted)                            │
  │    • (TDD) burst-gate SG #1 to UL slots  │
  │    • fire rctul <BW> <FRC>               │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SG #1 carrier (wanted UL freq)  │
  │    • set SG #1 power = sens + 6 dB       │
  │    • wait ~15 s for PHY settle           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna +     ◄── next point │  ← this test
  │         per blocker point                │
  │    • route RF switch                     │
  │    • cable-loss correct BOTH SGs         │
  │    • SG #1 wanted ON                     │
  │    • SG #2 blocker ON (freq, power, CW)  │
  │    • read BLER, verdict ≤ 5 %            │
  │    • SG #2 OFF, then SG #1 OFF           │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more blocker points at this     │── yes ──► back to §5.4 step 3b
     │ (ant, freq)?                    │
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group?           │── yes ──► back to §5.2
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every RX antenna
                 at every (band, freq, blocker point)
                 meets BLER ≤ 5 %)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. The BW XML is pushed; no DL fire is needed for this Rx test — the wanted signal comes from SG #1. The blocker SG (SG #2) is initialized to its first NB-blocker point here; subsequent blocker points retune SG #2 in §5.4 step 3b.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 0a | SG #2 init (skip if already initialized this session): Reset SG #2 (blocker) and lock to the SAME external 10 MHz reference SG #1 is using; disable LTE personality (raw CW). One-time-per-session — running NB blocking → ACS → ICS back-to-back? Skip this once SG #2 is already on the lab time base and in CW mode. | SG #2 (blocker) | *RST*OPC?:ROSC:SOUR EXT:ROSC:EXT:FREQ 10MHZ:RAD:LTE STAT OFF | *OPC? returns 1; EXT REF solid on SG #2; LTE personality disabled (raw CW) |
| 0b | SG #2 init: Set SG #2 (blocker) carrier and power to the first NB-blocker point for this band per 3GPP TS 36.141 Table 7.5.3.5-2, leave RF OFF. Subsequent NB-blocker points retune SG #2 in §5.4 step 3b. Substitute <BLOCKER_FREQ_HZ> and <BLOCKER_POWER_DBM> from the table for the current band. | SG #2 (blocker) | :FREQ:CW <BLOCKER_FREQ_HZ>:POW:AMPL <BLOCKER_POWER_DBM>:OUTP:STAT OFF | Panel shows CW carrier at the first blocker point's freq; power at the first blocker point's level; RF OFF |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm UL freq in Hz. | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_UL_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Load the A3-x FRC waveform on SG #1 (wanted) for this BW. Substitute the FRC name per the table below. | SG #1 (wanted) | :RAD:LTE:UL:WAV "A3-5" | Panel shows the FRC name, PRB allocation, QPSK 1/3 |
| 3 | Burst-gate SG #1 to UL slots. TDD bands only (B41) — skip on FDD. | SG #1 (wanted) | :RAD:LTE:UL:TDD:CONF 3:RAD:LTE:UL:TDD:SSFR 8:SOUR:RAD:LTE:UL:TRIG:SOUR FRAME:SOUR:RAD:LTE:UL:BURS:STAT ON | SG #1 transmits only during UL subframes |
| 4 | Start UL reception in test mode. Substitute the BW (MHz) and matching FRC. | testmac console | rctul 10 A3-5 | DU enters UL-RX mode; BLER counter visible (initially 100 % — neither SG is enabled yet) |

FRC selection (for step 2 and step 4):

| BW | FRC |
| --- | --- |
| 5 MHz | A3-4 |
| 10 MHz | A3-5 |
| 15 MHz | A3-6 |
| 20 MHz | A3-7 |

✓ DU in UL-RX mode at this BW. SG #1 waveform loaded but RF disabled. → Continue to §5.3 for the first freq point.

#### 5.4 Per-antenna measurement (repeat for each antenna, repeat 3b onward for each blocker point)

Do these steps once per antenna in the band's active RX set. For each antenna, the inner loop is per blocker point — the (frequency, power) pairs published in 3GPP TS 36.141 Table 7.5.3.5-2 for narrowband blocking. The wanted SG stays on at sens + 6 dB; only the blocker SG changes between points.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RX switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 |
| 2 | Apply cable-loss correction to BOTH SGs for this antenna and this (wanted_freq, blocker_freq) pair. Wanted-path loss is the (antenna_port, ul_freq_hz) entry; blocker-path loss is the (antenna_port, blocker_freq_hz) entry — they are different. Add each (positive) loss to its SG's output to compensate. Do not enable RF yet. | SG #1 + SG #2 | :POW:AMPL <WANTED_PWR + WANTED_LOSS> on SG #1:POW:AMPL <BLOCKER_PWR + BLOCKER_LOSS> on SG #2 | Both SG output levels updated |
| 3a | Enable the wanted SG (SG #1) — modulated LTE UL at sens + 6 dB | SG #1 (wanted) | :OUTP:STAT ON | SG #1 transmits; DU BLER drops to near 0 % within ~2 s (wanted only, no blocker yet) |
| 3b | Configure SG #2 (blocker) for this blocker point. Substitute <BLOCKER_FREQ_HZ> and <BLOCKER_POWER_DBM> from 3GPP TS 36.141 Table 7.5.3.5-2 for the current band. Modulation = CW (default for SG with LTE personality OFF — set in §5.2.5 step 0a). Do not enable SG #2 RF yet. | SG #2 (blocker) | :FREQ:CW <BLOCKER_FREQ_HZ>:POW:AMPL <BLOCKER_POWER_DBM + BLOCKER_LOSS> | SG #2 readouts update |
| 4 | Enable SG #2 (blocker) RF output. Wait ~2 s for steady state. | SG #2 (blocker) | :OUTP:STAT ON | Both SGs transmit; DU BLER reflects combined wanted + blocker |
| 5 | Read BLER on the DU. Average over test_duration window (default 1000 ms). | testmac console | <TBD: testmac BLER cmd> | Numeric BLER % |
| 6 | Verdict for this blocker point: BLER ≤ 5 % → PASS this point. Record (antenna, wanted_freq, blocker_freq, blocker_power, BLER, verdict). Example: B3, fm, blocker 2300 MHz / −15 dBm CW → BLER 3.2 % → PASS. | spreadsheet / notes | — | Row appended |
| 7 | Disable SG #2 (blocker) RF before moving to the next blocker point or next antenna | SG #2 (blocker) | :OUTP:STAT OFF | SG #2 stops; SG #1 still ON |
| 8 | When all blocker points for this antenna are done, disable SG #1 (wanted) before moving to the next antenna | SG #1 (wanted) | :OUTP:STAT OFF | SG #1 stops |

#### Loop control (when do I go where?)

After step 7, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More blocker points at this (antenna, wanted_freq) | → back to §5.4 step 3b with the next blocker (freq, power) |
| All blocker points done for this antenna, more antennas at this (band, BW, wanted_freq) | → run §5.4 step 8 then back to §5.4 step 1 with the next ant_i |
| All antennas done for this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW |
| All BWs done, more bands in this band-group | → back to §5.2 step 1 for the next band |
| All bands done, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 |
| All band-groups done | STOP. Overall verdict = PASS iff every RX antenna at every (band, BW, freq, blocker point) meets BLER ≤ 5 %. |

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| BLER = 100 % even with only the wanted SG enabled (blocker OFF) | Wanted-path carrier freq or power wrong; SG #1 not actually transmitting | Re-verify §5.3 step 1 (UL freq, not DL), §5.3 step 2 (sens + 6 dB), §5.4 step 3a (:OUTP:STAT ON) |
| BLER < 5 % with blocker ON, but blocker SG front-panel says RF OFF | Combiner port-to-port leakage (rare); SG #2 actually enabled | Verify §5.4 step 4 happened; verify combiner isolation spec |
| BLER way above 5 % even at "easy" blocker points (e.g. far offsets, modest power) | Wrong blocker-path cable-loss value used (over-driving the FHGW) | Re-verify §5.1 step 10 — cable loss for the blocker freq, on the blocker leg of the combiner |
| Both SGs drifting against each other | SG #2's 10 MHz reference not locked | Re-verify §5.2.5 step 0a — EXT REF indicator solid on BOTH SGs |
| BLER stable with blocker ON but lower than expected | Blocker SG power below intended (cable loss undercounted) or modulation set to non-CW by mistake | Re-verify §5.4 step 3b — power = nominal + cable_loss; verify SG #2's LTE personality is OFF (CW), per §5.2.5 step 0a |
| BLER on TDD (B41) high with blocker OFF | SG #1 transmitting continuously; jamming FHGW DL | Re-verify §5.2.5 step 3 — BURSt:STAT ON set on SG #1 for TDD |
| Combiner output level much lower than the sum of SG outputs | Combiner spec doesn't cover one of the two frequencies | Replace combiner with broadband variant covering both wanted UL band and blocker offset |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §7.5 Blocking characteristics, §7.5.3 Narrowband blocking (in-band blocking case 2), §7.5.3.5 Test requirements (Table 7.5.3.5-2 — NB blocker frequency / power per band), Annex A Fixed Reference Channels for receiver tests.

- TS 36.104 §7.5 Blocking characteristics (the underlying radio requirement that §7.5.3 conformance-tests).

- PW automation thresholds: <TBD: kpi_thresholds.test_lte_Rx_NB_blocking> (per source survey: "kpi_thresholds frequency-swept").

## 4.7 Rx — Wideband Blocking

3GPP references: TS 36.141 §7.5.3 Blocking — out-of-band CW blocker + TS 36.104 §7.5 Blocking characteristics + TS 36.141 Annex A (FRC definitions) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, FRCs, sensitivity limits) live in the PW Conformance — Common Test Parameters Reference above. Two RF sources are combined into the FHGW RX port: a wanted modulated LTE UL signal at sens_3GPP + 6 dB, plus a CW blocker at one of a set of out-of-band offsets per TS 36.141 Table 7.5.3.5-1.

### 1. Purpose

Verify the FHGW receiver still meets BLER ≤ 5 % on the wanted carrier when a CW blocker is applied at each of a set of fixed-offset frequencies extending from 0 MHz to ~870 MHz from the band edge. This wideband (general) blocking test characterises the receiver's selectivity across the entire out-of-band region against far-away CW interferers — typical of co-sited TX leakage, broadcast transmitters, or military spurs.

Conducted-RF measurement: SG #1 (wanted) and SG #2 (blocker) feed a power combiner whose output drives the FHGW RX port under test; FHGW reports BLER on the wanted signal at each of the 3GPP-defined (offset, power) test points.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / FRC tables.

Table 3-1 — Wideband Blocking Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared RX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Wanted Frequency | fm per (band, UL BW) — see Frequency points in the Reference (this test pins the wanted carrier at fm) | MHz |
| Wanted SG Power | sens_3GPP + 6 dB (i.e. 6 dB above the band's sensitivity limit) | dBm |
| Wanted Modulation | A3-x FRC (LTE UL, QPSK 1/3, full PRB) — see FRC selection (Rx) in the Reference | — |
| Blocker Frequency Set | Per 3GPP TS 36.141 Table 7.5.3.5-1 — discrete (offset, power) points spanning 0 MHz to ~870 MHz from the band edge. Per source survey: "Fixed-offset blocker (0-870 MHz from band edge); per-range thresholds". | MHz |
| Blocker Power per Point | Per 3GPP TS 36.141 Table 7.5.3.5-1 (varies per range; typically −15 dBm in close ranges) | dBm |
| Blocker Modulation | CW (per 3GPP §7.5.3) | — |
| BW | Per (band, BW) — see Supported bands in the Reference. FRC waveform tracks BW per FRC selection (Rx). | MHz |
| BLER threshold | ≤ 5 % (with ±4 % measurement tolerance) | % |

### 4. Pass/Fail criteria

Per TS 36.141 §7.5.3, when the wanted signal is held at REFSENS + 6 dB and a CW blocker is applied at each (offset, power) point in Table 7.5.3.5-1, the throughput must remain ≥ 95% of the reference measurement channel's maximum — equivalently, BLER ≤ 5% on the wanted carrier.

PW automation evaluates the verdict against the PW gate, not against the wider 3GPP limit:

PW pass gate (per RX port, per blocker (offset, power) point): BLER ≤ 5 % with both SGs enabled.

PW threshold storage: <TBD: kpi_thresholds.test_lte_Rx_blocking — per-range pass criteria> (per source survey: "Fixed-offset blocker (0-870 MHz from band edge); per-range thresholds").

Test verdict: PASS only if every RX port meets BLER ≤ 5 % at EVERY (band, blocker point) under test.

#### 4.1 Worked example (B3, b0103, fm)

| Quantity | Value |
| --- | --- |
| Band group | b0103 |
| Band | B3 |
| Wanted UL freq (fm) | 1747.5 MHz |
| Band sensitivity limit (PW) | −101.5 dBm <TBD: confirm sens_3GPP basis vs PW limit for wideband blocking> |
| Wanted SG power | −101.5 + 6 = −95.5 dBm |

Blocker points (selection):

| Point | Blocker freq | Blocker power (CW) | Measured BLER | Verdict |
| --- | --- | --- | --- | --- |
| 1 | 1800 MHz (10 MHz below B3 UL edge) | −15 dBm | 2.0 % | PASS |
| 2 | 1500 MHz | −15 dBm | 2.6 % | PASS |
| 3 | 1200 MHz | −15 dBm | 3.1 % | PASS |
| N | 800 MHz | −15 dBm | 4.5 % | PASS |

Overall verdict for B3 fm on this antenna: PASS (all blocker points ≤ 5 %).

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test — the inner loop runs the dual-source blocking BLER check across the full set of 3GPP blocker points.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, both SGs     │
  │    locked to ext 10 MHz, personality     │
  │    verify, RF switch ping, cable-loss    │
  │    cal per (port, UL freq) AND per       │
  │    blocker freq                          │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │    • load A3-x FRC waveform on SG #1     │
  │    • (TDD) burst-gate SG #1               │
  │    • fire rctul <BW> <FRC>               │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   fm only           (single point) │  ← chapter
  │    • set SG #1 carrier = fm UL freq      │
  │    • set SG #1 power = sens + 6 dB       │
  │    • wait ~15 s                          │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna +     ◄── next point │  ← this test
  │         per blocker (offset, power)      │
  │    • route RF switch                     │
  │    • cable-loss correct BOTH SGs at this │
  │      blocker freq                        │
  │    • SG #1 wanted ON                     │
  │    • SG #2 blocker ON                    │
  │    • read BLER, verdict ≤ 5 %            │
  │    • SG #2 OFF                           │
  │    • loop next blocker point             │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more blocker points at this     │── yes ──► back to §5.4 step 3b
     │ antenna?                        │
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more antennas at this band?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group?           │── yes ──► back to §5.2
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every RX antenna
                 at every (band, blocker point)
                 meets BLER ≤ 5 %)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. The BW XML is pushed; no DL fire is needed for this Rx test — the wanted signal comes from SG #1. The blocker SG (SG #2) is configured per-point in §5.4 and is NOT loaded with a waveform here (it's CW).

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 0a | SG #2 init (skip if already initialized this session). Reset, lock to the same external 10 MHz reference, LTE personality OFF (raw CW blocker). | SG #2 (blocker) | *RST*OPC?:ROSC:SOUR EXT:ROSC:EXT:FREQ 10MHZ:RAD:LTE STAT OFF | EXT REF solid; LTE personality disabled (raw CW) |
| 0b | Pre-arm SG #2 to the first blocker point's (freq, power) from 3GPP TS 36.141 Table 7.5.3.5-1 for this band; output OFF. Substitute <BLOCKER_FREQ_HZ> and <BLOCKER_POWER_DBM> for the first point. §5.4 will turn RF ON per point. | SG #2 (blocker) | :FREQ:CW <BLOCKER_FREQ_HZ>:POW:AMPL <BLOCKER_POWER_DBM>:OUTP:STAT OFF | SG #2 readouts updated; RF off |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm UL freq in Hz. | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_UL_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Load the A3-x FRC waveform on SG #1 (wanted) for this BW. Substitute the FRC name per the table below. | SG #1 (wanted) | :RAD:LTE:UL:WAV "A3-5" | Panel shows the FRC name, PRB allocation, QPSK 1/3 |
| 3 | Burst-gate SG #1 to UL slots. TDD bands only (B41) — skip on FDD. | SG #1 (wanted) | :RAD:LTE:UL:TDD:CONF 3:RAD:LTE:UL:TDD:SSFR 8:SOUR:RAD:LTE:UL:TRIG:SOUR FRAME:SOUR:RAD:LTE:UL:BURS:STAT ON | SG #1 transmits only during UL subframes |
| 4 | Start UL reception in test mode. Substitute the BW (MHz) and matching FRC. | testmac console | rctul 10 A3-5 | DU enters UL-RX mode; BLER counter visible (initially 100 % — neither SG is enabled yet) |

FRC selection (for step 2 and step 4):

| BW | FRC |
| --- | --- |
| 5 MHz | A3-4 |
| 10 MHz | A3-5 |
| 15 MHz | A3-6 |
| 20 MHz | A3-7 |

✓ DU in UL-RX mode at this BW. SG #1 waveform loaded but RF disabled. → Continue to §5.3 for the freq point.

#### 5.4 Per-antenna measurement (repeat for each antenna, repeat 3b onward for each blocker point)

Do these steps once per antenna in the band's active RX set. For each antenna, the inner loop is per blocker (offset, power) point — the full set from 3GPP TS 36.141 Table 7.5.3.5-1 (0 to ~870 MHz from band edge). The wanted SG stays on at sens + 6 dB; only the blocker SG changes between points.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RX switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 |
| 2 | Apply cable-loss correction to the wanted SG (SG #1) for this antenna at fm. Add (positive) wanted-path cable loss to SG #1 output power. Do not enable RF yet. | SG #1 (wanted) | :POW:AMPL <WANTED_PWR + WANTED_LOSS> | SG #1 amplitude readout updated |
| 3a | Enable the wanted SG (SG #1) — modulated LTE UL at sens + 6 dB | SG #1 (wanted) | :OUTP:STAT ON | SG #1 transmits; DU BLER drops to near 0 % within ~2 s (wanted only, no blocker yet) |
| 3b | Configure SG #2 (blocker) for this blocker point. Substitute <BLOCKER_FREQ_HZ> and <BLOCKER_POWER_DBM> from 3GPP TS 36.141 Table 7.5.3.5-1 for the current band, plus the blocker-freq-specific cable loss (positive dB) added to the nominal blocker power. Modulation = CW. Do not enable SG #2 RF yet. | SG #2 (blocker) | :FREQ:CW <BLOCKER_FREQ_HZ>:POW:AMPL <BLOCKER_POWER_DBM + BLOCKER_LOSS> | SG #2 readouts update |
| 4 | Enable SG #2 (blocker) RF output. Wait ~2 s for steady state. | SG #2 (blocker) | :OUTP:STAT ON | Both SGs transmit; DU BLER reflects combined wanted + blocker |
| 5 | Read BLER on the DU. Average over test_duration window (default 1000 ms). | testmac console | <TBD: testmac BLER cmd> | Numeric BLER % |
| 6 | Verdict for this blocker point: BLER ≤ 5 % → PASS this point. Record (antenna, fm, blocker_freq, blocker_power, BLER, verdict). Example: B3, fm, blocker 1800 MHz / −15 dBm CW → BLER 2.0 % → PASS. | spreadsheet / notes | — | Row appended |
| 7 | Disable SG #2 (blocker) before moving to the next blocker point | SG #2 (blocker) | :OUTP:STAT OFF | SG #2 stops; SG #1 still ON |
| 8 | When all blocker points for this antenna are done, disable SG #1 (wanted) before moving to the next antenna | SG #1 (wanted) | :OUTP:STAT OFF | SG #1 stops |

#### Loop control (when do I go where?)

After step 7, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More blocker points (offset, power) at this antenna | → back to §5.4 step 3b with the next blocker (freq, power) from Table 7.5.3.5-1 |
| All blocker points done for this antenna, more antennas at this (band, BW) | → run §5.4 step 8 then back to §5.4 step 1 with the next ant_i |
| All antennas done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW |
| All BWs done, more bands in this band-group | → back to §5.2 step 1 for the next band |
| All bands done, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 |
| All band-groups done | STOP. Overall verdict = PASS iff every RX antenna at every (band, blocker point) meets BLER ≤ 5 %. |

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| BLER = 100 % even with only the wanted SG enabled (blocker OFF) | Wanted-path carrier freq or power wrong; SG #1 not actually transmitting | Re-verify §5.3 step 1 (UL freq, not DL), §5.3 step 2 (sens + 6 dB), §5.4 step 3a (:OUTP:STAT ON) |
| BLER fails (> 5 %) only at the lowest-frequency blocker points (e.g. < 200 MHz) | Combiner has frequency-dependent loss not in your cable-loss table at those frequencies | Re-verify §5.1 step 10 — make sure every blocker freq in the sweep got its own cal entry |
| BLER fails (> 5 %) only at the highest-frequency blocker points (e.g. > 3 GHz) | Combiner spec doesn't cover the upper end; SG #2 output spec drops | Verify combiner and SG #2 frequency specs cover the full sweep range |
| Both SGs drifting against each other | SG #2's 10 MHz reference not locked | Re-verify §5.1 step 17 — EXT REF indicator solid on BOTH SGs |
| BLER pattern looks random (no monotonic trend with blocker freq) | Either SG isn't actually transmitting at the commanded power (loss table wrong) or wanted carrier is off-target | Spot-check with a spec analyzer at the combiner output |
| BLER on TDD (B41) high with blocker OFF | SG #1 transmitting continuously; jamming FHGW DL | Re-verify §5.2.5 step 3 — BURSt:STAT ON set on SG #1 for TDD |
| Single blocker point fails but neighbours pass | Spec analyzer-confirmable spur from SG #2 at that exact freq | Check for SG #2 harmonics / sub-harmonics landing on the wanted freq |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §7.5 Blocking characteristics, §7.5.3 Blocking — out-of-band CW blocker, §7.5.3.5 Test requirements (Table 7.5.3.5-1 — wideband blocker offsets and powers), Annex A Fixed Reference Channels for receiver tests.

- TS 36.104 §7.5 Blocking characteristics (the underlying radio requirement that §7.5.3 conformance-tests).

- PW automation thresholds: <TBD: kpi_thresholds.test_lte_Rx_blocking — per-range pass criteria> (per source survey: "Fixed-offset blocker (0-870 MHz from band edge); per-range thresholds").

## 4.8 Rx — Blocking Sweep

3GPP references: TS 36.141 §7.5.3 Blocking — continuous sweep variant (exploratory characterization) + TS 36.104 §7.5 Blocking characteristics + TS 36.141 Annex A (FRC definitions) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, FRCs, sensitivity limits) live in the PW Conformance — Common Test Parameters Reference above. This is the swept variant of the blocking test: instead of stepping through the discrete 3GPP table points, the blocker frequency is swept continuously across the offset range with a fixed blocker power, recording BLER at each step. The output is a BLER-vs-blocker-freq curve — useful for finding receiver weak spots between the formal 3GPP points.

### 1. Purpose

Characterise the FHGW receiver's selectivity as a continuous function of out-of-band blocker frequency. Where test_lte_Rx_blocking measures only the (offset, power) points published in 3GPP TS 36.141 Table 7.5.3.5-1, this sweep variant sweeps the blocker freq at fixed power across the entire offset range — useful for finding spurious-response notches, hidden image responses, or LO leakage that fall between the formal 3GPP test points.

Conducted-RF measurement: SG #1 (wanted) and SG #2 (blocker) feed a power combiner whose output drives the FHGW RX port under test. SG #2's frequency is stepped (e.g. 10 MHz increments) at fixed power while BLER is recorded at each step. Output: a BLER-vs-blocker-freq table per antenna.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / FRC tables.

Table 3-1 — Blocking Sweep Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared RX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Wanted Frequency | fm per (band, UL BW) — see Frequency points in the Reference (sweep pins the wanted carrier at fm) | MHz |
| Wanted SG Power | sens_3GPP + 6 dB (i.e. 6 dB above the band's sensitivity limit) | dBm |
| Wanted Modulation | A3-x FRC (LTE UL, QPSK 1/3, full PRB) — see FRC selection (Rx) in the Reference | — |
| Blocker Frequency Range | band_edge to band_edge + 870 MHz (also swept below the band when defined), in fixed steps (default 10 MHz). Per source survey: "Blocking sweep BLER vs blocker freq". | MHz |
| Blocker Sweep Step | Default 10 MHz (config-driven). <TBD: confirm default sweep step from kpi_thresholds / test config> | MHz |
| Blocker Power | Fixed for the duration of the sweep (e.g. −15 dBm CW). <TBD: confirm fixed blocker power from kpi_thresholds.test_lte_Rx_blocking_sweep> | dBm |
| Blocker Modulation | CW | — |
| BW | Per (band, BW) — see Supported bands in the Reference. FRC waveform tracks BW per FRC selection (Rx). | MHz |
| BLER threshold | ≤ 5 % (with ±4 % measurement tolerance) at ALL sweep points | % |

### 4. Pass/Fail criteria

The sweep is exploratory but still subject to a pass gate: at the fixed blocker power, the FHGW receiver must keep wanted-signal BLER ≤ 5 % at every blocker frequency in the sweep.

PW automation evaluates the verdict against the PW gate:

PW pass gate (per RX port, across the entire sweep): BLER ≤ 5 % at every sweep point with both SGs enabled.

PW threshold storage: <TBD: kpi_thresholds.test_lte_Rx_blocking_sweep — config pass_criteria> (per source survey: "Config pass_criteria").

Test verdict: PASS only if BLER stays ≤ 5 % at every blocker freq in the sweep on every RX port. A single point that crosses 5 % is enough to FAIL the test — but is also high-value diagnostic data (often a spurious-response notch). Always export the full BLER-vs-freq curve, not just the verdict.

#### 4.1 Worked example (B3, b0103, fm — partial sweep extract)

| Quantity | Value |
| --- | --- |
| Band group | b0103 |
| Band | B3 |
| Wanted UL freq (fm) | 1747.5 MHz |
| Band sensitivity limit (PW) | −101.5 dBm <TBD: confirm sens_3GPP basis> |
| Wanted SG power | −101.5 + 6 = −95.5 dBm |
| Blocker power (fixed) | −15 dBm CW |
| Sweep range | 1000 MHz to 1700 MHz (subset shown), 10 MHz step |

Sample BLER-vs-blocker-freq table:

| Blocker freq | BLER | Verdict (per point) |
| --- | --- | --- |
| 1000 MHz | 1.5 % | PASS |
| 1100 MHz | 1.8 % | PASS |
| 1200 MHz | 2.2 % | PASS |
| 1300 MHz | 2.6 % | PASS |
| 1400 MHz | 3.1 % | PASS |
| 1500 MHz | 3.4 % | PASS |
| 1600 MHz | 3.8 % | PASS |
| 1700 MHz | 4.4 % | PASS |
| ... | ... | ... |
| (no point > 5 %) | — | Overall: PASS |

Aggregate verdict for B3 fm on this antenna: PASS (all 88 sweep points ≤ 5 %).

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test — the inner loop runs the blocking sweep.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, both SGs     │
  │    locked to ext 10 MHz, personality     │
  │    verify, RF switch ping, cable-loss    │
  │    cal (wanted at fm + smooth curve      │
  │    across blocker sweep range)           │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │    • load A3-x FRC waveform on SG #1     │
  │    • (TDD) burst-gate SG #1               │
  │    • fire rctul <BW> <FRC>               │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   fm only           (single point) │  ← chapter
  │    • set SG #1 carrier = fm UL freq      │
  │    • set SG #1 power = sens + 6 dB       │
  │    • wait ~15 s                          │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna + per sweep step     │  ← this test
  │    • route RF switch                     │
  │    • SG #1 wanted ON                     │
  │    • SG #2 blocker freq = sweep_start    │
  │    • SG #2 ON at fixed blocker power     │
  │    • for each sweep step:                │
  │      - retune SG #2 freq                 │
  │      - update SG #2 power for cable loss │
  │      - read BLER, append to curve        │
  │    • final verdict = max(BLER) over sweep│
  │      ≤ 5 %                               │
  │    • SG #2 OFF, SG #1 OFF                │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more antennas at this band?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group?           │── yes ──► back to §5.2
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every RX antenna's
                 sweep curve stays ≤ 5 % at every
                 blocker freq in the sweep)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. The BW XML is pushed; no DL fire is needed for this Rx test — the wanted signal comes from SG #1. The blocker SG (SG #2) is configured per sweep step in §5.4 and is NOT loaded with a waveform here (it's CW).

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 0a | SG #2 init (skip if already initialized this session). Reset, lock to the same external 10 MHz reference, LTE personality OFF (raw CW blocker for the sweep). | SG #2 (blocker) | *RST*OPC?:ROSC:SOUR EXT:ROSC:EXT:FREQ 10MHZ:RAD:LTE STAT OFF | EXT REF solid; LTE personality disabled (raw CW) |
| 0b | Pre-arm SG #2 to sweep-start freq at the fixed blocker power; output OFF. Substitute <SWEEP_START_HZ> (first freq of the offset sweep for this band) and <BLOCKER_POWER_DBM> (fixed for the run). §5.4 will turn RF ON and drive the sweep step. | SG #2 (blocker) | :FREQ:CW <SWEEP_START_HZ>:POW:AMPL <BLOCKER_POWER_DBM>:OUTP:STAT OFF | SG #2 readouts updated; RF off |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm UL freq in Hz. | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_UL_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Load the A3-x FRC waveform on SG #1 (wanted) for this BW. Substitute the FRC name per the table below. | SG #1 (wanted) | :RAD:LTE:UL:WAV "A3-5" | Panel shows the FRC name, PRB allocation, QPSK 1/3 |
| 3 | Burst-gate SG #1 to UL slots. TDD bands only (B41) — skip on FDD. | SG #1 (wanted) | :RAD:LTE:UL:TDD:CONF 3:RAD:LTE:UL:TDD:SSFR 8:SOUR:RAD:LTE:UL:TRIG:SOUR FRAME:SOUR:RAD:LTE:UL:BURS:STAT ON | SG #1 transmits only during UL subframes |
| 4 | Start UL reception in test mode. Substitute the BW (MHz) and matching FRC. | testmac console | rctul 10 A3-5 | DU enters UL-RX mode; BLER counter visible (initially 100 % — neither SG is enabled yet) |

FRC selection (for step 2 and step 4):

| BW | FRC |
| --- | --- |
| 5 MHz | A3-4 |
| 10 MHz | A3-5 |
| 15 MHz | A3-6 |
| 20 MHz | A3-7 |

✓ DU in UL-RX mode at this BW. SG #1 waveform loaded but RF disabled. → Continue to §5.3 for the freq point.

#### 5.4 Per-antenna sweep (repeat for each antenna)

Do these steps once per antenna in the band's active RX set. For each antenna, the inner loop is per sweep step (blocker freq advances by <SWEEP_STEP_HZ> from <SWEEP_START_HZ> to <SWEEP_END_HZ> at fixed blocker power). Output: a BLER-vs-blocker-freq table per antenna.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RX switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 |
| 2 | Apply cable-loss correction to the wanted SG (SG #1) for this antenna at fm. Add (positive) wanted-path cable loss to SG #1 output power. Do not enable RF yet. | SG #1 (wanted) | :POW:AMPL <WANTED_PWR + WANTED_LOSS> | SG #1 amplitude readout updated |
| 3a | Enable the wanted SG (SG #1) — modulated LTE UL at sens + 6 dB | SG #1 (wanted) | :OUTP:STAT ON | SG #1 transmits; DU BLER drops to near 0 % within ~2 s (wanted only, no blocker yet) |
| 3b | Initialise SG #2 (blocker) at sweep_start: set freq = <SWEEP_START_HZ>, power = <BLOCKER_POWER + BLOCKER_LOSS_AT_THIS_FREQ>. Modulation = CW (set in §5.1 step 17). Do not enable RF yet. | SG #2 (blocker) | :FREQ:CW <SWEEP_START_HZ>:POW:AMPL <BLOCKER_PWR + BLOCKER_LOSS> | SG #2 readouts update |
| 4 | Enable SG #2 (blocker) RF output. Wait ~2 s for steady state. | SG #2 (blocker) | :OUTP:STAT ON | Both SGs transmit; first sweep point ready |
| 5 | Sweep loop — for each <BLOCKER_FREQ_HZ> from <SWEEP_START_HZ> to <SWEEP_END_HZ> in <SWEEP_STEP_HZ> increments: (a) retune SG #2 freq, (b) update SG #2 power with the loss correction at this freq, (c) wait ~1 s for steady state, (d) read BLER, (e) append (blocker_freq, BLER) to the per-antenna curve. | SG #2 + testmac | :FREQ:CW <BLOCKER_FREQ_HZ>:POW:AMPL <BLOCKER_PWR + BLOCKER_LOSS><TBD: testmac BLER cmd> | Numeric BLER % logged per step |
| 6 | Verdict for this antenna: max(BLER) over the entire sweep ≤ 5 % → PASS. Export the full BLER-vs-blocker-freq curve regardless of verdict — it's the primary deliverable of this test. Example: B3, fm, sweep 1000–1700 MHz @ 10 MHz step / −15 dBm CW → max BLER = 4.4 % → PASS. | spreadsheet / notes | — | Curve saved + PASS/FAIL recorded |
| 7 | Disable SG #2 (blocker) before moving to the next antenna | SG #2 (blocker) | :OUTP:STAT OFF | SG #2 stops; SG #1 still ON |
| 8 | Disable SG #1 (wanted) before moving to the next antenna | SG #1 (wanted) | :OUTP:STAT OFF | SG #1 stops |

#### Loop control (when do I go where?)

After step 8, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More antennas at this (band, BW) | → back to §5.4 step 1 with the next ant_i |
| All antennas done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW |
| All BWs done, more bands in this band-group | → back to §5.2 step 1 for the next band |
| All bands done, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 |
| All band-groups done | STOP. Overall verdict = PASS iff every RX antenna's sweep curve stays ≤ 5 % at every blocker freq in the sweep. |

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| BLER = 100 % even with only the wanted SG enabled (blocker OFF) | Wanted-path carrier freq or power wrong; SG #1 not actually transmitting | Re-verify §5.3 step 1 (UL freq, not DL), §5.3 step 2 (sens + 6 dB), §5.4 step 3a (:OUTP:STAT ON) |
| BLER curve has a sharp single-point spike at one blocker freq | Spurious response (image / LO leakage), or SG #2 harmonic landing on fm | Re-sweep that band of freqs at finer step (e.g. 1 MHz) to localise; check SG #2's known spur table |
| BLER curve rises monotonically toward the high-freq end of the sweep | Cable loss undercounted at high freqs (blocker drive falls below nominal) → BLER drops, not rises. If it rises, the FHGW receiver itself is degrading | Verify §5.1 step 10 (loss-vs-freq curve); spot-check SG #2 output at upper sweep range with a spec analyzer |
| Both SGs drifting against each other | SG #2's 10 MHz reference not locked | Re-verify §5.1 step 17 — EXT REF indicator solid on BOTH SGs |
| Sweep takes too long (hours per antenna) | SCPI freq retune is slow per step | Use 20 MHz step instead of 10; or pre-stage a SCPI list-sweep on SG #2 |
| BLER curve looks noisy (random ±2 % point-to-point) | test_duration window too short per step | Increase per-step dwell time (e.g. 2 s) before reading BLER |
| BLER on TDD (B41) high with blocker OFF | SG #1 transmitting continuously; jamming FHGW DL | Re-verify §5.2.5 step 3 — BURSt:STAT ON set on SG #1 for TDD |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §7.5 Blocking characteristics, §7.5.3 Blocking — continuous sweep variant (exploratory characterization beyond the discrete §7.5.3.5 table points), Annex A Fixed Reference Channels for receiver tests.

- TS 36.104 §7.5 Blocking characteristics (the underlying radio requirement).

- PW automation thresholds: <TBD: kpi_thresholds.test_lte_Rx_blocking_sweep — config pass_criteria> (per source survey: "Blocking sweep BLER vs blocker freq" / "Config pass_criteria").

## 4.9 Rx — Receiver Intermodulation

3GPP references: TS 36.141 §7.5.4 Receiver intermodulation + TS 36.104 §7.7 Receiver intermodulation + TS 36.141 Annex A (FRC definitions) — see §4 and §7.

Scope: this document is the per-test procedure. Common parameter tables (bands, frequency points, FRCs, sensitivity limits) live in the PW Conformance — Common Test Parameters Reference above. Three RF sources are combined into the FHGW RX port: a wanted modulated LTE UL signal at sens_3GPP + 6 dB, plus two interferer tones placed so that their third-order intermodulation product (2·f1 − f2 or 2·f2 − f1) lands exactly on the wanted carrier.

### 1. Purpose

Verify the FHGW receiver still meets BLER ≤ 5 % on the wanted carrier when two interferer tones, placed at frequency offsets that produce a third-order intermodulation (IM3) product directly on the wanted carrier, are injected together with the wanted signal. The receiver-intermodulation test characterises the receiver's tolerance to in-band IM3 fall-on — the dominant non-linear distortion mechanism in real cell sites where multiple operators co-exist in adjacent bands.

Conducted-RF measurement: SG #1 (wanted) + SG #2 (Tone 1, CW at +2·Δf) + SG #3 (Tone 2, modulated LTE at +Δf) feed a 3-way power combiner whose output drives the FHGW RX port. The IM3 product 2·f_tone2 − f_tone1 lands back on the wanted carrier and degrades wanted BLER.

### 3. Test parameters

Below are the test parameters and values for this specific test. See the Common Test Parameters Reference (top of document) for the underlying band / frequency / FRC tables.

Table 3-1 — Receiver Intermodulation Test Parameters

| Parameter | Value | Units |
| --- | --- | --- |
| Antenna Ports | All declared RX ports for the band group (measured one at a time) — see FHGW + BOB antenna mapping in the Reference | — |
| Wanted Frequency | fb / fm / ft per (band, UL BW) — see Frequency points in the Reference | MHz |
| Wanted SG Power | sens_3GPP + 6 dB (i.e. 6 dB above the band's sensitivity limit) | dBm |
| Wanted Modulation | A3-x FRC (LTE UL, QPSK 1/3, full PRB) — see FRC selection (Rx) in the Reference | — |
| Tone 1 Frequency | wanted + 2·Δf. Per 3GPP TS 36.141 Table 7.5.4.5-1 for Δf per band/BW | MHz |
| Tone 1 Modulation | CW (per 3GPP §7.5.4) | — |
| Tone 1 Power | wanted + 46 dB (per 3GPP §7.5.4 — wide-area BS). <TBD: confirm per-band tone power from kpi_thresholds> | dBm |
| Tone 2 Frequency | wanted + Δf | MHz |
| Tone 2 Modulation | LTE modulated (5 MHz, QPSK-modulated, per 3GPP §7.5.4) | — |
| Tone 2 Power | wanted + 46 dB (matched to Tone 1; per 3GPP §7.5.4) | dBm |
| Δf | Per 3GPP Table 7.5.4.5-1 (typically 10 MHz for LTE; <TBD: confirm per-band Δf value>) | MHz |
| BW | Per (band, BW) — see Supported bands in the Reference. FRC waveform tracks BW per FRC selection (Rx). | MHz |
| BLER threshold | ≤ 5 % (with ±4 % measurement tolerance) | % |

### 4. Pass/Fail criteria

Per TS 36.141 §7.5.4, when the wanted signal is held at REFSENS + 6 dB and the two interferer tones (CW at +2·Δf, modulated LTE at +Δf, both at wanted + 46 dB) are applied together, the throughput on the wanted carrier must remain ≥ 95% of the reference measurement channel's maximum — equivalently, BLER ≤ 5% on the wanted carrier.

PW automation evaluates the verdict against the PW gate, not against the wider 3GPP limit:

PW pass gate (per RX port, per (band, freq, (Δf, tone-power) point)): BLER ≤ 5 % with all three SGs enabled.

PW threshold storage: <TBD: kpi_thresholds.test_lte_Rx_intermodulation — per-point pass criteria>.

Test verdict: PASS only if every RX port meets BLER ≤ 5 % at every (band, freq, (Δf, tone-power)) test point.

#### 4.1 Worked example (B3, b0103, fm)

| Quantity | Value |
| --- | --- |
| Band group | b0103 |
| Band | B3 |
| Wanted UL freq (fm) | 1747.5 MHz |
| Band sensitivity limit (PW) | −101.5 dBm <TBD: confirm sens_3GPP basis> |
| Wanted SG power | −101.5 + 6 = −95.5 dBm |
| Δf | 10 MHz |
| Tone 1 freq (CW) | fm + 2·Δf = 1747.5 + 20 = 1767.5 MHz |
| Tone 1 power | wanted + 46 = −95.5 + 46 = −49.5 dBm (CW). PW automation <TBD: confirm value> — example shows the simpler 46 dB offset; some PW configs use a derated −46 dBm absolute |
| Tone 2 freq (LTE mod) | fm + Δf = 1747.5 + 10 = 1757.5 MHz |
| Tone 2 power | wanted + 46 = −49.5 dBm (LTE modulated, 5 MHz QPSK) |
| IM3 fall-on (computed) | 2·f_tone2 − f_tone1 = 2·1757.5 − 1767.5 = 3515 − 1767.5 = 1747.5 MHz ✓ (lands on wanted) |
| Measured BLER | 4.1 % |
| Verdict | 4.1 % ≤ 5 % → PASS |

The simpler version of the example shown in the planning notes (Tone 1 / Tone 2 both at −46 dBm absolute) is the equivalent re-expressed form for that specific band.

### 5. Procedure

Order: §5.1 → §5.2 → §5.2.5 → §5.3 → §5.4 (four nested loops — see flowchart). §5.1 / §5.2 / §5.3 are chapter Common procedure; §5.2.5 / §5.4 are specific to this test — the inner loop runs the three-source IM3 BLER check.

START
                   │
                   ▼
  ┌──────────────────────────────────────────┐
  │  §5.1   Bench bring-up   (do ONCE)       │  ← chapter
  │    SSH to DU + FHGW, L1/L2, all 3 SGs    │
  │    locked to ext 10 MHz, personality     │
  │    verify, RF switch ping, cable-loss    │
  │    cal per (port, wanted_freq, +Δf,      │
  │    +2·Δf)                                │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2   Per band combo ◄── next combo    │  ← chapter
  │    • push ACTIVE_B<NN>*.xml              │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.2.5 Per BW         ◄── next BW       │  ← this test
  │    • edit + push oru_startup_slave.xml   │
  │    • load A3-x FRC on SG #1 (wanted)     │
  │    • load 5 MHz LTE waveform on SG #3    │
  │      (Tone 2 modulated)                  │
  │    • (TDD) burst-gate SG #1 + SG #3      │
  │    • fire rctul <BW> <FRC>               │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.3   Per freq point ◄── next fb/fm/ft │  ← chapter
  │    • set SG #1 carrier = wanted UL freq  │
  │    • set SG #1 power = sens + 6 dB       │
  │    • wait ~15 s                          │
  └──────────────────┬───────────────────────┘
                     ▼
  ┌──────────────────────────────────────────┐
  │  §5.4   Per antenna +     ◄── next point │  ← this test
  │         per (Δf, tone-power)             │
  │    • route RF switch                     │
  │    • cable-loss correct ALL 3 SGs        │
  │    • SG #1 wanted ON                     │
  │    • SG #2 Tone 1 (CW @ +2·Δf) ON        │
  │    • SG #3 Tone 2 (LTE @ +Δf) ON         │
  │    • read BLER, verdict ≤ 5 %            │
  │    • SG #2 + SG #3 OFF                   │
  └──────────────────┬───────────────────────┘
                     │
                     ▼
     ┌─────────────────────────────────┐
     │ more (Δf, tone-pwr) points?     │── yes ──► back to §5.4 step 3b
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more antennas at this freq?     │── yes ──► back to §5.4
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌─────────────────────────────────┐
     │ more freq points at this BW?    │── yes ──► back to §5.3
     └────────────┬────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more BWs for this band combo?            │── yes ──► back to §5.2.5
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more bands in THIS band-group?           │── yes ──► back to §5.2
     └────────────┬─────────────────────────────┘
                  │ no
                  ▼
     ┌──────────────────────────────────────────┐
     │ more band-groups in your test plan?      │── yes ──► EEPROM rewrite + reboot
     └────────────┬─────────────────────────────┘           then back to §5.2
                  │ no
                  ▼
                STOP
                (verdict = PASS iff every RX antenna
                 at every (band, freq, IM3 point)
                 meets BLER ≤ 5 %)

See How to send commands in the Common Test Parameters Reference (top of document) for what each "Where to run" cell below means.

#### 5.2.5 Per-BW setup (repeat for each BW within the current band combo)

Do these steps once per BW within the current band combo. The BW XML is pushed; no DL fire is needed for this Rx test — the wanted signal comes from SG #1, Tone 1 is CW (SG #2), Tone 2 is a 5 MHz modulated LTE waveform from SG #3.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 0a | SG #2 init (skip if already initialized this session). Reset, lock to the same external 10 MHz reference, LTE personality OFF — Tone 1 is CW. | SG #2 (Tone 1) | *RST*OPC?:ROSC:SOUR EXT:ROSC:EXT:FREQ 10MHZ:RAD:LTE STAT OFF | EXT REF solid; CW mode |
| 0b | Pre-arm SG #2 (Tone 1) to wanted + 2·Δf at the per-test tone power; output OFF. Substitute <TONE1_FREQ_HZ> = wanted_ul_hz + 2 × Δf_hz and <TONE_POWER_DBM> from 3GPP TS 36.141 Table 7.5.4.5-1. §5.4 will turn RF ON. | SG #2 (Tone 1) | :FREQ:CW <TONE1_FREQ_HZ>:POW:AMPL <TONE_POWER_DBM>:OUTP:STAT OFF | SG #2 readouts updated; RF off |
| 0c | SG #3 init (skip if already initialized this session). Reset, lock to the same external 10 MHz reference, LTE uplink personality ON — Tone 2 is a 5 MHz modulated LTE interferer. | SG #3 (Tone 2) | *RST*OPC?:ROSC:SOUR EXT:ROSC:EXT:FREQ 10MHZ:RAD:LTE:LINK UL:RAD:LTE:LINK:DUPL FDD:RAD:LTE STAT ON | EXT REF solid; LTE UL personality active |
| 0d | Pre-arm SG #3 (Tone 2) to wanted + Δf at the per-test tone power; output OFF. Substitute <TONE2_FREQ_HZ> = wanted_ul_hz + Δf_hz and <TONE_POWER_DBM> from 3GPP TS 36.141 Table 7.5.4.5-1. §5.4 will turn RF ON. | SG #3 (Tone 2) | :FREQ:CW <TONE2_FREQ_HZ>:POW:AMPL <TONE_POWER_DBM>:OUTP:STAT OFF | SG #3 readouts updated; RF off |
| 1a | Edit a copy of oru_startup_slave.xml. Set <channel-bandwidth> in Hz (e.g. 10000000 for 10 MHz) and <center-of-channel-bandwidth> to the band's fm UL freq in Hz. | DU shell → editor | Edit fields:<channel-bandwidth>10000000</channel-bandwidth><center-of-channel-bandwidth><FM_UL_HZ></center-of-channel-bandwidth> | File saved |
| 1b | Push the runtime BW XML to the FHGW | netopeer2-cli session (from §5.1 step 12) | > edit-config --target running --config=<BW_RUNTIME_FILE>.xml --rpc 40 | NETCONF OK; FHGW carriers re-initialise. Wait ~5 s. |
| 2 | Load the A3-x FRC waveform on SG #1 (wanted) for the wanted-signal BW. Substitute the FRC name per the table below. | SG #1 (wanted) | :RAD:LTE:UL:WAV "A3-5" | Panel shows the FRC name, PRB allocation, QPSK 1/3 |
| 3 | Load the 5 MHz LTE modulated waveform on SG #3 (Tone 2). Per 3GPP §7.5.4 the modulated interferer is a 5 MHz LTE signal regardless of the wanted BW. <TBD: confirm SG #3 waveform name — A3-4 vs the 3GPP "5 MHz reference channel for interferer"> | SG #3 (Tone 2) | :RAD:LTE:UL:WAV "A3-4" | Panel shows 5 MHz LTE PRB |
| 4 | Burst-gate SG #1 + SG #3 to UL slots. TDD bands only (B41) — skip on FDD. Both modulated SGs need burst gating; SG #2 stays CW. | SG #1 + SG #3 | :RAD:LTE:UL:TDD:CONF 3:RAD:LTE:UL:TDD:SSFR 8:SOUR:RAD:LTE:UL:TRIG:SOUR FRAME:SOUR:RAD:LTE:UL:BURS:STAT ON (on both SGs) | Both SGs transmit only during UL subframes |
| 5 | Start UL reception in test mode. Substitute the BW (MHz) and matching FRC. | testmac console | rctul 10 A3-5 | DU enters UL-RX mode; BLER counter visible (initially 100 % — no SG enabled yet) |

FRC selection (for step 2 and step 5):

| BW | FRC |
| --- | --- |
| 5 MHz | A3-4 |
| 10 MHz | A3-5 |
| 15 MHz | A3-6 |
| 20 MHz | A3-7 |

✓ DU in UL-RX mode at this BW. SG #1 + SG #3 waveforms loaded but RF disabled. → Continue to §5.3 for the first freq point.

#### 5.4 Per-antenna measurement (repeat for each antenna, repeat 3b/3c onward for each (Δf, tone-power) point)

Do these steps once per antenna in the band's active RX set. For each antenna, the inner loop is per (Δf, tone-power) point — the configurations published in 3GPP TS 36.141 Table 7.5.4.5-1 (typically a single nominal point per band/BW, occasionally a small set). The wanted SG stays on at sens + 6 dB; Tone 1 and Tone 2 are reconfigured per point.

| # | Step | Where to run | Command | Expected |
| --- | --- | --- | --- | --- |
| 1 | Route the RF switch for ant_i. Substitute <BANK> and <PORT> from your setup's RX switch mapping. | RF switch (HTTP) | GET http://<SWITCH_IP>/:SP6T:<BANK>:STATE:<PORT> | 1 |
| 2 | Apply cable-loss correction to ALL THREE SGs for this antenna at their respective frequencies. Wanted-path loss is for (antenna_port, wanted_freq); Tone 1 loss is for (antenna_port, wanted_freq + 2·Δf); Tone 2 loss is for (antenna_port, wanted_freq + Δf). Each (positive) loss adds to its SG's output. Do not enable RF yet. | SG #1 + SG #2 + SG #3 | :POW:AMPL <WANTED_PWR + WANTED_LOSS> on SG #1:POW:AMPL <TONE1_PWR + TONE1_LOSS> on SG #2:POW:AMPL <TONE2_PWR + TONE2_LOSS> on SG #3 | All three SG amplitudes update |
| 3a | Enable the wanted SG (SG #1) — modulated LTE UL at sens + 6 dB | SG #1 (wanted) | :OUTP:STAT ON | SG #1 transmits; DU BLER drops to near 0 % within ~2 s (wanted only, no tones yet) |
| 3b | Configure SG #2 (Tone 1, CW) for this IM3 point. Substitute <TONE1_FREQ_HZ> = wanted_freq + 2·Δf (Hz) and <TONE1_POWER_DBM> per 3GPP TS 36.141 Table 7.5.4.5-1 + this antenna's Tone-1-path cable loss. Do not enable RF yet. | SG #2 (Tone 1, CW) | :FREQ:CW <TONE1_FREQ_HZ>:POW:AMPL <TONE1_PWR + TONE1_LOSS> | SG #2 readouts update |
| 3c | Configure SG #3 (Tone 2, modulated LTE) for this IM3 point. Substitute <TONE2_FREQ_HZ> = wanted_freq + Δf (Hz) and <TONE2_POWER_DBM> per 3GPP Table 7.5.4.5-1 + this antenna's Tone-2-path cable loss. Do not enable RF yet. | SG #3 (Tone 2, modulated) | :FREQ:CW <TONE2_FREQ_HZ>:POW:AMPL <TONE2_PWR + TONE2_LOSS> | SG #3 readouts update |
| 4 | Enable both interferer SGs. Wait ~2 s for steady state. Order matters slightly: enable the CW (SG #2) first so the modulated tone (SG #3) coming up second doesn't trigger a transient that fakes a brief BLER spike. | SG #2, then SG #3 | :OUTP:STAT ON on SG #2:OUTP:STAT ON on SG #3 | All three SGs transmit; IM3 product 2·f_tone2 − f_tone1 lands on f_wanted |
| 5 | Read BLER on the DU. Average over test_duration window (default 1000 ms). | testmac console | <TBD: testmac BLER cmd> | Numeric BLER % |
| 6 | Verdict for this IM3 point: BLER ≤ 5 % → PASS this point. Record (antenna, wanted_freq, Δf, tone1_power, tone2_power, BLER, verdict). Example: B3, fm, Tone 1 CW at fm+20 MHz / −49.5 dBm, Tone 2 LTE at fm+10 MHz / −49.5 dBm → BLER 4.1 % → PASS. | spreadsheet / notes | — | Row appended |
| 7 | Disable Tone 2 (SG #3) and Tone 1 (SG #2) before moving to the next IM3 point | SG #3, then SG #2 | :OUTP:STAT OFF on SG #3:OUTP:STAT OFF on SG #2 | Both interferer SGs stop; SG #1 still ON |
| 8 | When all IM3 points for this antenna are done, disable SG #1 (wanted) before moving to the next antenna | SG #1 (wanted) | :OUTP:STAT OFF | SG #1 stops |

#### Loop control (when do I go where?)

After step 7, decide based on what's left in your test plan:

| What's left | Where to go next |
| --- | --- |
| More (Δf, tone-power) points at this (antenna, wanted_freq) | → back to §5.4 step 3b with the next IM3 point |
| All IM3 points done for this antenna, more antennas at this freq | → run §5.4 step 8 then back to §5.4 step 1 with the next ant_i |
| All antennas done for this freq, more freq points (fb/fm/ft) at this BW | → back to §5.3 step 1 for the next freq point |
| All freq points done, more BWs for this band combo | → back to §5.2.5 step 1 for the next BW |
| All BWs done, more bands in this band-group | → back to §5.2 step 1 for the next band |
| All bands done, more band-groups in your test plan | → EEPROM rewrite + reboot (see Update FHGW EEPROM model name appendix), then back to §5.2 step 1 |
| All band-groups done | STOP. Overall verdict = PASS iff every RX antenna at every (band, freq, IM3 point) meets BLER ≤ 5 %. |

### 6. Troubleshooting

| Symptom | Likely cause | What to check |
| --- | --- | --- |
| BLER = 100 % even with only the wanted SG enabled (interferers OFF) | Wanted-path carrier freq or power wrong; SG #1 not actually transmitting | Re-verify §5.3 step 1 (UL freq, not DL), §5.3 step 2 (sens + 6 dB), §5.4 step 3a (:OUTP:STAT ON) |
| BLER unchanged when interferers turn ON | Tone freqs misaligned (IM3 product not landing on wanted carrier); verify 2·f_tone2 − f_tone1 = f_wanted numerically | Recompute Tone 1 (CW) at +2·Δf and Tone 2 (modulated) at +Δf; check signs and Δf value per 3GPP table |
| BLER way above 5 % even at the nominal 3GPP point | One or both tone-path cable losses wrong (over-driving FHGW); OR three SGs not locked to common 10 MHz | Re-verify §5.1 step 10 (cable loss at all three freqs); re-verify §5.1 steps 17–18 (EXT REF solid on all 3 SGs) |
| BLER jittery (sometimes < 5 %, sometimes > 5 %) at a fixed setup | Three SGs are slowly drifting against each other (typical when only one's 10 MHz is locked) | Re-verify ALL THREE SGs are on the same external 10 MHz reference |
| BLER on TDD (B41) high with interferers OFF | SG #1 or SG #3 transmitting continuously (one of them not burst-gated) | Re-verify §5.2.5 step 4 — BURSt:STAT ON set on BOTH SG #1 and SG #3 for TDD |
| Tone 2 modulated signal looks like a CW on the spec analyzer | SG #3's LTE personality is OFF | Re-verify §5.1 step 18 — :RAD:LTE STAT ON on SG #3 |
| IM3 fall-on is wrong sign (e.g. 2·f_tone1 − f_tone2 falls in-band by accident) | Swapped Tone 1 / Tone 2 freq assignments | Verify Tone 1 (CW) is at +2·Δf and Tone 2 (modulated) is at +Δf from wanted — the asymmetry is meaningful |

### 7. References

Clauses cited by this test (archive URLs in the umbrella's 3GPP Specification archives):

- TS 36.141 §7.5 Blocking characteristics, §7.5.4 Receiver intermodulation, §7.5.4.5 Test requirements (Table 7.5.4.5-1 — Δf and tone power per band/BW), Annex A Fixed Reference Channels for receiver tests.

- TS 36.104 §7.7 Receiver intermodulation (the underlying radio requirement).

- PW automation thresholds: <TBD: kpi_thresholds.test_lte_Rx_intermodulation — per-point pass criteria> (per source survey: "Two-tone intermodulation").

# Appendix A — Acronyms

| Acronym | Expansion |
| --- | --- |
| 3GPP | 3rd Generation Partnership Project (standards body owning TS 36.x / TS 38.x) |
| ACLR | Adjacent Channel Leakage Ratio (TS 36.141 §6.6) |
| ARFCN | Absolute Radio Frequency Channel Number — LTE channel identifier |
| BOB | Break-Out Board — passive RF breakout that carries the FHGW's TX/RX ports to SMA |
| BW | Channel Bandwidth (1.4, 3, 5, 10, 15, 20 MHz for LTE; 10, 20, 40, 50, 100 MHz for NR) |
| CI | Continuous Integration — automation gate that runs per-merge |
| DL | Downlink (BS → UE direction; Tx tests) |
| DU | Distributed Unit — the L1/L2 host (testmac, l1.sh, l2.sh) |
| EEPROM | Electrically Erasable Programmable Read-Only Memory — stores the FHGW's model name |
| EXM | Keysight E6640A Wireless Test Set (spectrum analyzer mode here) |
| E-TM | E-UTRA Test Model — LTE downlink reference waveform per TS 36.141 Annex C |
| E-UTRA | Evolved Universal Terrestrial Radio Access (LTE radio access) |
| FDD | Frequency Division Duplex |
| FHGW | Fronthaul Gateway — PW's compute board (Xilinx ZU67 RFSoC) under test |
| FR1 | Frequency Range 1 (sub-7-GHz NR; same band space as LTE) |
| FRC | Fixed Reference Channel (uplink reference waveform per TS 36.141 Annex A.3) |
| fb / fm / ft | Bottom / Mid / Top test frequencies in a band (TS 36.141 §4.9.1) |
| L1 | Layer 1 (PHY) software stack on the DU |
| L2 | Layer 2 (MAC) software stack on the DU; hosts the testmac> prompt |
| LLS-C1 | Lower-Layer Split topology C1 (PTP from RU to DU only) |
| netopeer-cli | Open-source NETCONF client used to push DU/FHGW config XMLs |
| NR | New Radio (5G radio access) |
| NR-TM | NR Test Model — NR downlink reference waveform per TS 38.141-1 Annex E |
| OTA | Over The Air (radiated test — not in scope here) |
| P_rated | Rated output power per antenna (from the framework's KPI thresholds JSON) |
| PRACH | Physical Random Access Channel |
| PTP | Precision Time Protocol (IEEE 1588) — Falcon-RX → DU → FHGW timing |
| RFFE | Radio Front End — not present in this bench (FHGW low-power output only) |
| RFSoC | RF System-on-Chip (Xilinx ZU67, integrates DAC/ADC + FPGA) |
| rctdl / rctul | testmac DL / UL fire commands (per-antenna burst) |
| RU | Radio Unit (O-RAN role) |
| SA | Spectrum Analyzer (the EXM in this bench) |
| SCPI | Standard Commands for Programmable Instruments |
| SCS | Sub-Carrier Spacing (NR — 15, 30, 60 kHz typically) |
| SG | Signal Generator (uplink Rx tests substitute the EXM for an SG) |
| TDD | Time Division Duplex |
| testmac | DU L2 test-shell where rctdl / rctul / band-activation commands run |
| TT | Test Tolerance — measurement-uncertainty allowance per TS 36.141 Annex F |
| TS | Technical Specification (3GPP — e.g. TS 36.141, TS 38.141-1) |
| TX / RX | Transmit / Receive antenna port labels (ant1..ant8) |
| UL | Uplink (UE → BS direction; Rx tests) |